ISO/IEC JTC 1/SC 29

Date:   2011-08-30

ISO/IEC DIS 23009-1

ISO/IEC JTC 1/SC 29/WG 11

Secretariat:

Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats

Élément introductif — Élément central — Partie 1: Titre de la partie

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a Draft International Standard and is copyright-protected by ISO. Except as permitted under the applicable laws of the user's country, neither this ISO draft nor any extract from it may be reproduced, stored in a retrieval system or transmitted in any form or by any means, electronic, photocopying, recording or otherwise, without prior written permission being secured.

Requests for permission to reproduce should be addressed to either ISO at the address below or ISO's member body in the country of the requester.

ISO copyright office

Case postale 56  CH-1211 Geneva 20

Tel.  + 41 22 749 01 11

Fax  + 41 22 749 09 47

E-mail  copyright@iso.org

Web  www.iso.org

Reproduction may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	v

Introduction	vi

1	Scope	1

2	Normative references	1

3	Terms, definitions, symbols and abbreviated terms	2

3.1	Terms and definitions	2

3.2	Symbols and abbreviated terms	5

3.3	Conventions	6

4	Introduction	6

4.1	System description	6

4.2	DASH client model	7

4.3	DASH data model overview	8

4.4	Protocols	11

4.5	Media Stream and Representation properties	11

4.6	Brands	13

4.7	Schemes	13

5	Media Presentation	14

5.1	General	14

5.2	Media Presentation Description	15

5.3	Media Presentation Description updates	16

5.4	MPD assembly	17

5.5	Hierarchical data model	19

5.6	Base URL Processing	54

5.7	Program information	55

5.8	Descriptors	56

5.9	DASH metrics descriptor	62

6	Segment formats	63

6.1	Introduction	63

6.2	Segment types	64

6.3	Segment formats for ISO base media file format	66

6.4	Segment formats for MPEG-2 transport streams	69

7	Combined semantics of MPD and Segment formats	75

7.1	Introduction	75

7.2	General	76

7.3	Media Presentation based on the ISO base media file format	77

7.4	Media Presentation based on MPEG-2 TS	79

8	Profiles	81

8.1	Definition	81

8.2	Full profile	82

8.3	ISO Base media file format On Demand profile	82

8.4	ISO Base media file format live profile	83

8.5	ISO Base media file format main profile	85

8.6	MPEG-2 TS main profile	86

8.7	MPEG-2 TS simple profile	87

Annex A (informative)  Example DASH client behaviour	89

A.1	Introduction	89

A.2	Overview	89

A.3	Segment list generation	90

A.4	Seeking	93

A.5	Support for trick modes	94

A.6	Switching Representations	94

A.7	Reaction to error codes	94

A.8	Encoder clock drift control	95

Annex B (normative)  MPD schema	96

Annex C (normative)  MIME type registration for MPD	102

C.1	Introduction	102

C.2	MIME type and subtype	102

C.3	Parameters	103

Annex D (normative) DASH Metrics	104

D.1	Introduction	104

D.2	DASH-Metrics client reference model	104

D.3	Definition of observation points	104

D.4	Semantics of the DASH metrics	105

Annex E (normative)  Byte range requests with regular HTTP GET methods	112

E.1	Background	112

E.2	Construction rule	112

E.3	Examples	113

Annex F (informative)  Guidelines for extending DASH with other delivery formats	114

F.1	Adding delivery formats to DASH	114

F.2	Media Presentation authoring rules	114

Annex G (informative)  MPD Examples and MPD Usage	115

G.1	Example MPD for ISO Base media file format On Demand profile	115

G.2	Example for ISO Base media file format Live profile	116

G.3	Example for MPEG-2 TS file format Live profile	117

G.4	Example for multiple stereo views	118

G.5	Example for SVC alternative streams	119

G.6	Example for trick play support	120

G.7	Example for content protected by multiple schemes	120

G.8	Example for usage of Role descriptor	122

G.9	Example for MPD security	122

Bibliography	125

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

ISO/IEC 23009-1 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

ISO/IEC 23009 consists of the following parts, under the general title Information technology — Dynamic adaptive streaming over HTTP (DASH):

- Part 1: Media presentation description and segment formats

Introduction

Dynamic Adaptive Streaming over HTTP (DASH) is intended to support a media-streaming model for delivery of media content in which control of the session lies exclusively with the client. Clients may request data using the HTTP protocol from standard web servers that have no DASH-specific capabilities. Consequently, this standard focuses not on client or server procedures but on the data formats used to provide a DASH Media Presentation.

This part of ISO/IEC 23009 specifies formats for adaptive streaming delivery of MPEG media over HTTP. This part of ISO/IEC 23009 is applicable to streaming services over the Internet.

Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats

# Scope

This part of ISO/IEC 23009 specifies formats for adaptive streaming delivery of MPEG media over HTTP. This part ISO/IEC 23009 is applicable to streaming services over the Internet.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ITU-T Rec. H.222.0 | ISO/IEC 13818-1, Information technology – Generic coding of moving pictures and associated audio information: Systems

ITU-T Rec.H.262 | ISO/IEC 13818-2, Information technology – Generic coding of moving pictures and associated audio information: Video

ISO/IEC 14496-10, Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding

ISO/IEC 14496-12, Information technology – Coding of audio-visual objects – Part 12: ISO base media file format (technically identical to ISO/IEC 15444-12)

ISO/IEC 23003-3, Information technology – MPEG audio technologies – Part 3: Unified Speech and Audio Coding

ISO/IEC 23001-7, Information technology – MPEG systems technology – Part 7: Common encryption in ISO base media file format files

IETF RFC 1521, MIME (Multipurpose Internet Mail Extensions) Part One: Mechanisms for Specifying and Describing the Format of Internet Message Bodies, September 1993

IETF RFC 1738, Uniform Resource Locators (URL), December 1994

IETF RFC 2141, URN Syntax, May 1997.

IETF RFC 2616, Hypertext Transfer Protocol – HTTP/1.1, June 1999.

IETF RFC 3023, XML Media Types, January 2001.

IETF RFC 3406, Uniform Resource Names (URN) Namespace Definition Mechanisms, October 2002.

IETF RFC 3986, Uniform Resource Identifier (URI): Generic Syntax, January 2005.

IETF RFC 4122, A Universally Unique IDentifier (UUID) URN Namespace, July 2005.

IETF RFC 4337, MIME Type Registration for MPEG-4, March 2006.

IETF RFC 5646, Tags for Identifying Languages, September 2009.

IETF RFC 6381, The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types, August 2011.

W3C XLINK XML Linking Language (XLink) Version 1.1, W3C Recommendation 06, May 2010

# Terms, definitions, symbols and abbreviated terms

## Terms and definitions

For the purposes of this document, the following terms and definitions apply.

3.1.1

access unit

unit of a media stream with an assigned Media Presentation time

3.1.2

accessibility

the degree to which a media content or certain media content components are available to as many people as possible

3.1.3

Adaptation Set

a set of interchangeable encoded versions of one or several media content components

3.1.4

available Segment

Segment which is accessible at its assigned HTTP-URL and a possibly assigned byte range, i.e. the request with an HTTP GET results in a reply of the Segment and 2xx status code

3.1.5

Bitstream Switching Segment

Segment that if present contains essential data to switch to the Representation it is assigned to

3.1.6

complementary Representation

Representation which complements at least one dependent Representation

3.1.7

continuous media

media with an inherent notion of time, for example, speech, audio, video, timed text or timed metadata

3.1.8

DASH metric

a metric identified by key and defined in this part of the specification

3.1.9

dependent Representation

Representation for which Segments from its complementary Representations are necessary for presentation and/or decoding of the contained media content components

3.1.10

earliest presentation time

the smallest presentation time of any access unit of a Media Segment or Subsegment for a media stream

3.1.11

group

collection of Representations that are expected to not being presented jointly

3.1.12

HTTP-URL

URL with a fixed scheme of “http://” or “https://”

3.1.13

Index Segment

Segment that primarily contains indexing information for Media Segments

3.1.14

Initialisation Segment

Segment containing metadata that is necessary to present the media streams encapsulated in Media Segments

3.1.15
media content 
one media content period or a contiguous sequence of media content periods

3.1.16
media content component 
one continuous component of the media content with an assigned media component type that can be encoded individually into a media stream

3.1.17
media content component type
a single type of media content such as audio, video, or text

3.1.18

media content period

set of media content components that have a common timeline as well as relationships on how they may be presented

3.1.19

Media Presentation

collection of data that establishes a bounded or unbounded presentation of media content

3.1.20

Media Presentation Description
MPD

formalized description for a Media Presentation for the purpose of providing a streaming service

3.1.21

Media Presentation timeline

concatenation of the timeline of all Periods which itself is common to all Representations in the Period

3.1.22

Media Segment

Segment that complies with media format in use and enables playback when combined with zero or more preceding segments, and an Initialisation Segment (if any)

3.1.23

media stream

encoded version of a media content component

3.1.24

Media Subsegment

Subsegment that only contains media data but no Segment Index

3.1.25

Period

interval of the Media Presentation, where a contiguous sequence of all Periods constitutes the Media Presentation

3.1.26

presentation time

a time associated to an access unit that maps it to the Media Presentation timeline

3.1.27

Representation

collection and encapsulation of one or more media streams in a delivery format and associated with descriptive metadata

3.1.28

Segment

smallest addressable unit in an MPD with a defined format

3.1.29
Segment availability start time 
time instant in wall-clock time at which a Segment becomes an available Segment

3.1.30
Segment availability end time 
time instant in wall-clock time at which a Segment ceases to be an available Segment

3.1.31

Segment Index

a compact index of the time range to byte range mapping within a Media Segment separately from the MPD

3.1.32

stream access point
SAP

position in a Representation enabling playback of a media stream to be started using only the information contained in Representation data starting from that position onwards (preceded by initialising data in the Initialisation Segment, if any)

3.1.33

Sub-Representation

part of a Representation described in the MPD that is present in the entire Period

3.1.34

Subsegment

smallest unit within Media Segments that is indexed by a Segment Index

3.1.35
valid Segment URL 
a HTTP-URL that is promised to reference a Segment during its Segment availability period

3.1.36
wall-clock time 
time as stated by UTC

## Symbols and abbreviated terms

For the purposes of this document, the following symbols and abbreviated terms apply.

AVC		advanced video coding

CAT		conditional access table

DASH		dynamic adaptive streaming over HTTP

DM			DASH Metrics

DRM		digital rights management

ECM		entitlement control message

HTTP		hypertext transfer protocol

IDR			instantaneous decoding refresh

ISOBMFF	ISO base media file format

MPD		Media Presentation Description

MVC		multi-view video coding

PAT			program association table

PCR		program clock reference

PES		packetized elementary stream

PID			packet identifier

PMT		program map table

PSI			program specific information

PTS			presentation time stamp

SAP		stream access point

SEI			supplementary enhancement information

SVC		scalable video coding

TCP		transmission control protocol

TLS			transport layer security

TS			transport stream

URI			uniform resource identifier

URL		uniform resource locator

URN		uniform resource name

UUID		universally unique identifier

UTC		coordinated universal time

XML		extensible mark-up language

## Conventions

The following naming conventions apply in this document.

- Elements in an XML document are identified by an upper-case first letter and in bold face as Element. To express that an element Element1 is contained in another element Element2, we may write Element2.Element1. If an element's name consists of two or more combined words, camel-casing is typically used, e.g. ImportantElement.

- Attributes in an XML document are identified by a lower-case first letter as well as they are preceded by a ‘@’-sign, e.g. @attribute. To point to a specific attribute @attribute contained in an element Element, one may write Element@attribute. If an attribute's name consists of two or more combined words, camel-casing is typically used after the first word, e.g. @veryImportantAttribute.

- Namespace qualification of elements and attributes is used as per XML standards, in the form of namespace:Element or @namespace:attribute The fully qualified namespace will be provided in the schema fragment associated with the declaration.

- Variables defined in the context of this document are specifically highlighted with italics, e.g. InternalVariable.

- Structures that are defined as part of the hierarchical data model are identified by an upper-case first letter, e.g. Period, Adaptation Set, Representation, Segment, etc.

- The term "this clause" refers to the entire clause that with a single digit. The term "this subclause" refers to all text contained in the subclause with the lowest hierarchy heading.

# Introduction

## System description

Dynamic Adaptive Streaming over HTTP (DASH) specifies XML and binary formats that enable delivery of media content from standard HTTP servers to HTTP clients and enable caching of content by standard HTTP caches.

This part of ISO/IEC 23009 primarily defines two formats:

- The Media Presentation Description (MPD) describes a Media Presentation, i.e. a bounded or unbounded presentation of media content. In particular, it defines formats to announce resource identifiers for Segments and to provide the context for these identified resources within a Media Presentation. These resource identifiers are HTTP-URLs possibly combined with a byte range.

- The Segment formats specify the formats of the entity body of the HTTP response to an HTTP GET request or a partial HTTP GET with the indicated byte range using HTTP/1.1 as defined in RFC 2616 to a resource identified in the MPD. Segments typically contain efficiently coded media data and metadata according to or aligned with common media formats.

The MPD provides sufficient information for a client to provide a streaming service to the user by accessing the Segments through the protocol specified in the scheme of the defined resources. In the context of this part of ISO/IEC 23009 the assumed protocol is HTTP/1.1. Such a client is referred to as a DASH Client in the remainder of this document. However, this part of ISO/IEC 23009 does not provide a normative specification for such a client.

Figure 1 shows a possible deployment architecture in which the formats defined in this part of ISO/IEC 23009 may be used. Boxes with solid lines indicate devices that are mentioned in this specification as they host or process the formats defined in this specification whereas dashed boxes are conceptual or transparent. This part of ISO/IEC 23009 deals with the definition of formats that are accessible on the interface to the DASH Client, indicated by the solid lines. Any other formats or interfaces are not in scope of this part of ISO/IEC 23009. In the considered deployment scenario, it is assumed that the DASH Client has access to an MPD. The MPD provides sufficient information for the DASH Client to provide a streaming service to the user by requesting Segments from an HTTP server and demultiplexing, decoding and rendering the included media streams.

Figure 1 — Example system for DASH formats

## DASH client model

The design of the formats defined in this part of ISO/IEC 23009 is based on the informative client model as shown in Figure 2. The figure illustrates the logical components of a conceptual DASH client model. In this figure the DASH Access Engine receives the Media Presentation Description (MPD), constructs and issues requests and receives Segments or parts of Segments. In the context of this part of ISO/IEC 23009, the output of the DASH Access Engine consists of media in MPEG container formats (ISO/IEC 14496-12 ISO Base Media File Format or ISO/IEC 13818-1 MPEG-2 Transport Stream), or parts thereof, together with timing information that maps the internal timing of the media to the timeline of the Media Presentation. In Annex F of this part of ISO/IEC 23009, guidance on enabling the use of this part of ISO/IEC 23009 with other container formats is provided.

Figure 2 — DASH Client Model

## DASH data model overview

DASH is intended to support a media-streaming model for delivery of media content in which control of the session lies exclusively with the client. Clients may request data using the HTTP protocol from standard web servers that have no DASH-specific capabilities. Consequently, this standard focuses not on client or server procedures but on the data formats used to provide a DASH Media Presentation.

The collection of encoded and deliverable versions of media content and the appropriate description of these form a Media Presentation. Media content is composed of a single or multiple contiguous media content periods in time. Each media content period is composed of one or multiple media content components, for example audio components in various languages and a video component. Each media content component has an assigned media content component type, for example audio or video.

Each media content component may have several encoded versions, referred to as media streams. Each media stream inherits the properties of the media content, the media content period, the media content component from which it was encoded and in addition it gets assigned the properties of the encoding process such as sub-sampling, codec parameters, encoding bitrate, etc. This describing metadata is relevant for static and dynamic selection of media content components and media streams.

Figure 3 — DASH High-Level Data Model

DASH is based on a hierarchical data model aligned with the presentation in Figure 3. A DASH Media Presentation is described by a Media Presentation Description document. This describes the sequence of Periods in time that make up the Media Presentation. A Period typically represents a media content period during which a consistent set of encoded versions of the media content is available i.e. the set of available bitrates, languages, captions, subtitles etc. does not change during a Period.

Within a Period, material is arranged into Adaptation Sets. An Adaptation Set represents a set of interchangeable encoded versions of one or several media content components. For example there may be one Adaptation Set for the main video component and a separate one for the main audio component. If there is other material available, for example captions or audio descriptions, then these may each have a separate Adaptation Set. Material may also be provided in multiplexed form, in which case interchangeable versions of the multiplex may be described as a single Adaptation Set, for example an Adaptation Set containing both the main audio and main video for a Period. Each of the multiplexed components may be described individually by a media content component description.

An Adaptation Set contains a set of Representations. A Representation describes a deliverable encoded version of one or several media content components. A Representation includes one or more media streams (one for each media content component in the multiplex). Any single Representation within an Adaptation Set is sufficient to render the contained media content components. Typically, clients may switch from Representation to Representation during a Period in order to adapt to network conditions or other factors. Clients may also ignore Representations that rely on codecs or other rendering technologies they do not support or that are otherwise unsuitable.

Within a Representation, the content may be divided in time into Segments. A Segment is the basic unit of data that is advertised in the Media Presentation Description. A URL is provided for each Segment meaning that a Segment is the largest unit of data that can be retrieved with a single HTTP request.

NOTE	This is not strictly true, since the MPD may also include a byte range with the URL, meaning that the Segment is contained in the provided byte range of some larger resource. An intelligent client could in principle construct a single request for multiple Segments, but this would not be the typical case.

DASH defines different timelines. One of the key features in DASH is that encoded versions of different media content components share a common timeline. The presentation time of access unit within the media content is mapped to the global common presentation timeline for synchronization of different media components and to enable seamless switching of different coded versions of the same media components. This timeline is referred as Media Presentation timeline. The Media Segments themselves contain accurate Media Presentation timing information enabling synchronization of components and seamless switching.

A second timeline is used to signal to clients the availability time of segments at the specified HTTP-URLs called Segment availability times. These times are provided in wall-clock time and clients typically compare the wall-clock time to Segment availability times before accessing the segments at the specified HTTP-URLs. For On-Demand services with a static MPD, the availability times of all Segments are identical. For live services when the MPD is updated, the availability times of segments depend on the position of the Segment in the Media Presentation timeline.

Segments have assigned a duration, which is the duration of the media contained in the Segment when presented at normal speed. Typically all Segments in a Representation have the same or roughly similar duration. However Segment duration may differ from Representation to Representation. A DASH presentation can be constructed with relative short segments (for example a few seconds), or longer Segments including a single Segment for the whole Representation.

Short Segments are usually required in the case of live content, where there are restrictions on end-to-end latency. The duration of a Segment is typically a lower bound on the end-to-end latency. DASH does not support the possibility for Segments to be extended over time: a Segment is a complete and discrete unit that must be made available in its entirety.

Segments may be further subdivided into Subsegments each of which contains a whole number of complete access units. There may also be media-format-specific restrictions on Subsegment boundaries, for example in the ISO Base Media File Format a Subsegment must contain a whole number of complete Movie Fragments. If a Segment is divided into Subsegments this division may be described by a compact Segment index, which provides the presentation time range in the Representation and corresponding byte range in the Segment occupied by each Subsegment. Clients may download this index in advance and then issue requests for individual Subsegments.

Clients may switch from Representation to Representation within an Adaptation Set at any point in the media. However, switching at arbitrary positions may be complicated because of coding dependencies within Representations and other factors. It is also desirable to avoid download of 'overlapping' data i.e. media for the same time period from multiple Representations. Usually, switching is simplest at a random access point in the new stream. In order to formalize requirements related to switching DASH defines a codec-independent concept of Stream Access Point and identifies various types of Stream Access Point.

Segmentation and Subsegmentation may be performed in ways that make switching simpler. For example, in the very simplest cases each Segment or Subsegment begins with a random access point and the boundaries of Segments or Subsegments are aligned across the Representations of an Adaptation Set. In this case, switching Representation involves playing to the end of a (Sub)Segment of one Representation and then playing from the beginning of the next (Sub)Segment of the new Representation. The Media Presentation Description and Segment Index provide various indications, which describe properties of the Representations that may make switching simpler. Profiles of this specification may then require these indicators to be set in certain ways, making implementation of clients for those profiles simpler at the cost of requiring the media data to obey the indicated constraints.

For On-Demand services, the Media Presentation Description is a static document describing the various aspects of the Media Presentation. All Segments of the Media Presentation are available on the server once any Segment is available. For live services, however, Segments become available with time as the content is produced. The Media Presentation Description may be updated regularly to reflect changes in the presentation over time, for example Segment URLs for new segments may be added to the MPD and those for old, no longer available Segments may be removed. However, if Segment URLs are described using a template, this updating may not be necessary except for some redundancy/failover cases.

## Protocols

This part of ISO/IEC 23009 may be deployed in a system according to Figure 1 for which

- The DASH Client includes a client as specified in RFC 2616 and,

- The HTTP Server hosting the DASH Segments complies with a server as specified in RFC 2616.

DASH Clients typically use the HTTP GET method or the HTTP partial GET method, as specified in RFC 2616, Clause 9.3, to access Segments or parts thereof.

The use HTTP as a transport protocol inherently provides many advanced features such as caching, redirection or authentication. As another example, transport security in HTTP-based delivery may be achieved by using HTTP over TLS as specified in RFC 2818 [6].

## Media Stream and Representation properties

### Media stream access points

To be able to access a Representation, each of the media streams that are contained in the Representation requires Media Stream Access Points (SAPs). SAPs in the context of this part of ISO/IEC 23009 refer to the SAP definition in ISO/IEC 14496-12 Annex I. ISO/IEC 14496-12 Annex I.3 defines different types of SAPs that provide a relationship between the position where a stream can be accessed, a SAP, relative to the start of a Segment or Subsegment, its presentation time and the presentation times and position of other access unit in the stream. The same SAP type definitions shall apply for this part of ISO/IEC 23009.

A SAP is a position in a Representation that enables playback of a media stream to be started using only the information contained in Representation data starting from that position onwards (preceded by initialising data in the Initialisation Segment, if any).

For each SAP the properties, ISAP, TSAP, ISAU, TDEC, TEPT, and TPTF are identified and defined in ISO/IEC 14496-12 Annex I.2.

In particular, TSAP is defined to be earliest presentation time of any access unit of the media stream such that all access units of the media stream with presentation time greater than or equal to TSAP can be correctly decoded using data in the Representation starting at byte position ISAP and no data before ISAP.

NOTE	The type of SAP is dependent only on which Access Units are correctly decodable and their arrangement in presentation order. The types informally correspond with some common terms:

- Type 1 corresponds to what is known in some coding schemes as a “Closed GoP random access point” (in which all access units, in decoding order, starting from ISAP can be correctly decoded, resulting in a continuous time sequence of correctly decoded access units with no gaps) and in addition the access unit in decoding order is also the first access unit in presentation order.

- Type 2 corresponds to what is known in some coding schemes as a “Closed GoP random access point”, for which the first access unit in decoding order in the media stream starting from ISAU is not the first access unit in presentation order.

- Type 3 corresponds to what is known in some coding schemes as an “Open GoP random access point”, in which there are some access units in decoding order following ISAU that cannot be correctly decoded and have presentation times less than TSAP.

- Type 4 corresponds to what is known in some coding schemes as an "Gradual Decoding Refresh (GDR) random access point”, in which there are some access units in decoding order starting from and following ISAU that cannot be correctly decoded and have presentation times less than TSAP.

- Type 5 corresponds to the case for which there is at least one access unit in decoding order starting from ISAP that cannot be correctly decoded and has presentation time greater than TDEC and where TDEC is the earliest presentation time of any access unit starting from ISAU.

- Type 6 corresponds to the case for which there is at least one access unit in decoding order starting from ISAP that cannot be correctly decoded and has presentation time greater than TDEC and where TDEC is not the earliest presentation time of any access unit starting from ISAU.

SAPs are mostly relevant for two purposes in this part of ISO/IEC 23009:

- For randomly accessing a Media Presentation, for example at the startup of the Media Presentation, after a seeking operation or after an error event especially in live cases.

- To permit switching between two Representations whereby for seamless switching each media stream i in the switch-from Representation is presented up to TSAP(i) and each media stream i in the switch-to Representation is presented from the media Stream Access Point starting from TSAP(i).

There are obvious benefits for the client to be able to identify SAPs and one or several of their properties, in particular ISAP and TSAP for each media stream without requiring to access data at positions following ISAP. DASH provides functionalities to explicitly signal such information by using signals in the MPD or the Segment Index or combinations of the two.

### Non-overlapping Segments and Subsegments

Segments and Subsegments represent units for which the client has an exact map on how to access and download the unit using HTTP GET or HTTP partial GET methods.

Segments (respectively Subsegments) are typically generated by segmenting encoded media streams into appropriate units. If the generation of Segments (respectively Subsegments) adheres to certain rules, then the sequential decoding and presentation of Media Segments (respectively Subsegments) results in a correct presentation of all contained media streams. To define such rules the notion of “non-overlapping” segments (respectively Subsegments) is defined as follows.

- Let

- TE(S,i) be the earliest presentation time of any access unit in stream i of a Segment or Subsegment S,

- TL(S,i) be the latest presentation time of any access unit in stream i of a Segment or Subsegment S.

Then two segments (respectively Subsegments), A and B, which may or may not be of different Representations, are non-overlapping if TL(A,i) < TE(B,i) for all media streams i in A and B or if TL(B,i) < TE(A,i) for all streams i in A and B where i refers to the same media component.

The property of “non-overlapping” segments (respectively Subsegments) is used to define the terms Segment alignment and Subsegment alignment.

### Bitstream concatenation

A sequence of Segments (respectively Subsegments) is a "conforming Segment (respectively Subsegment) sequence" if the concatenation of all Segments (respectively Subsegments) in the sequence of Segments (respectively Subsegments) results in a bitstream that conforms to the media formats in use (including container and codecs).

NOTE	This implies that a player conforming to the media format can play the resulting bitstream.

## Brands

The ISO Base Media File Format, ISO/IEC 14496-12, defines the concept of brands; brand values identify specifications or conformance points. This part of the specification specifies several brands, as listed in Table 1.

Table 1 — Brands defined in this part of ISO/IEC 23009

| Brand Identifier | Clause in this part of ISO/IEC 23009 | Informative description |
| --- | --- | --- |
| msdh | 6.3.4.2 | Media Segment conforming to the general format type for ISO Base Media File Format. |
| msix | 6.3.4.3 | Media Segment conforming to the Indexed Media Segment format type for ISO Base Media File Format. |
| sims | 6.3.4.4 | Media Segment conforming to the Sub-Indexed Media Segment format type for ISO Base Media File Format. |
| dash | 6.3.5.2 | - ISO Base Media File Format file specifically designed for DASH including movie fragments and Segment Index. |
| sisx | 6.4.6.2 | - Single Index Segment used to index MPEG-2 TS based Media Segments. |
| risx | 6.4.6.3 | - Representation Index Segment used to index MPEG-2 TS based Media Segments. |
| ssss | 6.4.6.4 | - Subsegment Index Segment used to index MPEG-2 TS based Media Segments. |
| lmsg | 7.3.1 | - last Media Segment indicator for ISO base media file format |

## Schemes

This part of the specification specifies several schemes as listed in Table 2.

Table 2 — Schemes defined in this part of ISO/IEC 23009

| Scheme Identifier | Clause in this part of ISO/IEC 23009 | Informative description |
| --- | --- | --- |
| urn:mpeg:dash:mpd:2011 | Annex B | The namespace of the XML schema for the MPD |
| urn:mpeg:dash:mp4protection:2011 | 5.8.5.2 | protection schemes identified by a the Scheme Type within the Scheme Type Box of the Protection Scheme Information Box of ISO/IEC14496-12. |
| urn:mpeg:dash:13818:1:CA_descriptor:2011 | 5.8.5.2 | Conditional Access System used for ISO/IEC 13818-1 (MPEG-2 Transport Stream) |
| urn:mpeg:dash:14496:10:frame_packing_arrangement_type:2011 | 5.8.5.3 | - frame-packing arrangement as defined by Table D-8 of ISO/IEC 14496-10 |
| urn:mpeg:dash:13818:1:stereo_video_format_type:2011 | 5.8.5.3 | - frame-packing arrangement as defined by Table L-1 of ISO/IEC 13818-1 |
| urn:mpeg:dash:23002:3:audio_channel_configuration:2011 | 5.8.5.4 | - channel configuration as defined by Table 65 of ISO/IEC 23002-3. |
| urn:mpeg:dash:role:2011 | 5.8.5.5 | - DASH role scheme |
| urn:mpeg:dash:stereoid:2011 | 5.8.5.6 | - scheme for multiple views media content description |
| urn:mpeg:dash:profile:full:2011 | 8.2 | - identifier for Full profile |
| urn:mpeg:dash:profile:isoff-on-demand:2011 | 8.3 | - identifier for ISO Base media file format On Demand profile |
| urn:mpeg:dash:profile:isoff-live:2011 | 8.4 | - identifier for ISO Base media file format live profile |
| urn:mpeg:dash:profile:isoff-main:2011 | 8.5 | - identifier for ISO Base media file format main profile |
| urn:mpeg:dash:profile:mp2t-main:2011 | 8.6 | - identifier for MPEG-2 TS main profile |
| urn:mpeg:dash:profile:mp2t-simple:2011 | 8.7 | - identifier for MPEG-2 TS simple profile |

# Media Presentation

## General

A Media Presentation is a collection of data that is accessible to a DASH Client to provide a streaming service to the user.

A Media Presentation is described by an MPD including possible updates of the MPD. The MPD is defined in 5.2 and the MPD update mechanism is defined in 5.3. Assembly of a fragmented MPD is defined in 5.4. The data model that constitutes a Media Presentation is defined in 5.5. In 5.6, the formats and processing of URLs in the MPD is introduced. Program information is defined in 5.7. Descriptors associated to Representations or collections thereof are provided in 5.8. DASH metric collection description is specified in 5.9.

## Media Presentation Description

### General

The Media Presentation Description (MPD) is a document that contains metadata required by a DASH Client to construct appropriate HTTP-URLs to access Segments and to provide the streaming service to the user.

NOTE	actual playback of the media streams included in the Representations is not controlled by the MPD information. Playback is controlled by the media engine operating on the media streams contained in the Representations in the usual way.

The format of URLs in the MPD and the process to generate HTTP GET and partial GET requests from URLs provided in the MPD is defined in 5.6.

The MPD is an XML document that is formatted according to the XML schema provided in Annex B. Some context on the schema is provided in 5.2.2.

The MPD shall be authored such that, after XML attributes or elements in the DASH namespace but not in the XML schema documented in Annex B are removed, the result is a valid XML document formatted according to that schema and that complies with this part of ISO/IEC 23009.

Following XML rules, the MPD document shall contain exactly one MPD element as specified in 5.5.

The MIME type of the MPD document is defined in Annex C.

The delivery of the MPD is not in scope of this part of ISO/IEC 23009. However, if the MPD is delivered over HTTP, then the MPD document may be transfer encoded for transport, as described in RFC 2616.

NOTE	As an example the GZip algorithm as defined in RFC 1952 [3] may be used for Transfer-encoding.

NOTE	MPD encryption is not a normative part of this standard. However, if operating in an insecure environment and required by the content/service provider, elements and attributes of MPD may be encrypted to protect their confidentiality by using the syntax and processing rules specified in the “XML Encryption Syntax and Processing” by W3C [8].

NOTE	MPD integrity protection is not a normative part of this standard. However, if operating in an insecure environment and required by the content/service provider, the digital signing and verification procedures specified in the “XML Signature Syntax and Processing” by W3C [9] may be used to protect data origin authenticity and integrity of the MPD.

### Schema

The initial part of the XML schema of the MPD is provided below, including namespace and other definitions. Specific types, elements and attributes are introduced in the remainder of this subclause. The complete normative MPD schema is provided in Annex B of this part of ISO/IEC 23009. In case of any inconsistencies the schema in Annex B takes precedence over the XML syntax snippets provided in this clause.

<?xml version="1.0" encoding="UTF-8"?> <xs:schema targetNamespace="urn:mpeg:DASH:schema:MPD:2011" attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns="urn:mpeg:DASH:schema:MPD:2011"> <xs:import namespace="http://www.w3.org/1999/xlink" schemaLocation="http://www.w3.org/1999/xlink.xsd"/> <xs:annotation> <xs:appinfo>Media Presentation Description</xs:appinfo> <xs:documentation xml:lang="en"> This Schema defines the Media Presentation Description for MPEG-DASH. </xs:documentation> </xs:annotation> <!-- MPD: main element --> <xs:element name="MPD" type="MPDtype"/> ... </xs:schema>

## Media Presentation Description updates

If MPD@type is set to 'dynamic', the MPD may be updated during the Media Presentation. Updates typically extend the accessible Segment list for each Representation, introduce a new Period or terminate the Media Presentation.

When the MPD is updated

- the value of MPD@id, if present, shall be the same in the original and the updated MPD;

- the values of any Period@id attributes shall be the same in the original and the updated MPD, unless the containing Period element has been removed.

- the values of any AdaptationSet@id attributes shall be the same in the original and the updated MPD unless the containing Period element has been removed.

- any Representation with the same @id and within the same Period as a Representation appearing in the previous MPD shall provide functionally equivalent attributes and elements, and shall provide functionally identical Segments with the same indices in the corresponding Representation in the new MPD.

If the attribute MPD@minimumUpdatePeriod is not present, no update to the MPD is expected, the attribute MPD@mediaPresentationDuration shall be present and the MPD shall remain valid until the Media Presentation end time.

If the attribute MPD@minimumUpdatePeriod is present updates to the MPD are expected and restricted in a sense that at the location where the MPD is available at a certain time, the MPD is also valid for the duration of the value of the MPD@minimumUpdatePeriod attribute. Specifically the following shall hold:

If the i-th version of the MPD is the last version of MPD till the end of the Media Presentation, let Texp(i) be the Media Presentation end time. Otherwise, let Texp(i) be the sum of the value of MPD@minimumUpdatePeriod and the wall-clock time at which the i-th version of the MPD is updated (and replaced with the (i+1)-th version). The i-th MPD shall remain valid until Texp(i) in the following sense:

- all Segments with availability start time less than Texp(i) shall be available at their availability start times at the location advertised in the i-th MPD.

- all Representations have a Segment with an availability start time, Tavail, which is less than Texp(i) and with duration not less than (Texp(i) - Tavail). Note that the actual duration of this Segment is not known at the client until this Segment or the updated MPD is fetched and this Segment duration may be less than the previous Segment duration if it is the last Segment in the Period or if the SegmentTimeline is in place.

NOTE 1: The clients may not know Texp(i), but they can each calculate a lower bound on Texp(i) by adding MPD@minimumUpdatePeriod to the wall-clock time at which they request the MPD.

NOTE 2: The second condition above ensures that sufficient media is contained in each Representation to present the Media Presentation up to Texp(i) for a client that begins playing each Segment at the earliest possible time (its availability start time).

NOTE 3: The result of the MPD validity requirement is that all items a client expects to be able to retrieve (both segments and MPD elements) are guaranteed to be available for retrieval during the periods that the client can expect them to be accessible.

NOTE 4: An MPD may contain no Period element or only an early available Period may be provided. In this case, updates to the MPD are expected in order to provide the start time of the first Period, which coincides with the start of the actual Media Presentation.

## MPD assembly

### Introduction

A mechanism for referencing a remote element from within a local MPD is defined. A subset of W3C XLINK simple links is defined consisting of

- restricted syntax and semantics as specified in 5.4.2, and

- the processing model as specified in 5.4.3.

- If the MPD is updated, then the rules in 5.3 apply to the MPD after dereferencing all remote elements.

### Syntax and semantics

Table 3 provides the XLINK attributes that are used in this part of ISO/IEC 23009 and shall be supported accordingly.

Table 3 — XLINK attributes used in this part of ISO/IEC 23009

| Attribute | Comments and Usage |
| --- | --- |
| @xlink:type | Identifies the type of W3C XLINK being used. In the context of this part of ISO/IEC 23009, all references shall be W3C XLINK simple links. The attribute @xlink:type is optional with fixed setting @xlink:type="simple". |
| @xlink:href | Identifies the remote element by an URI as defined in IETF RFC 3986. In the context of this part of ISO/IEC 23009, such URIs shall exclusively be HTTP-URLs. |
| @xlink:show | Defines the desired behaviour of a remote element once dereferenced from within a MPD as defined in W3C XLINK. In the context of this part of ISO/IEC 23009, the attribute @xlink:show is optional with fixed setting @xlink:show="embed". NOTE In W3C XLINK, the behaviour of conforming XLink applications when embedding XML-based ending resources, such as a remote element, is not defined. Thus, the actual behaviour for this part of ISO/IEC 23009 is defined in 5.4.3. |
| @xlink:actuate | Defines the desired timing of dereferencing a remote element from within a MPD as defined in W3C XLINK. The following attribute values are allowed in this part of ISO/IEC 23009: - onLoad: an application should dereference the remote element immediately on loading the MPD. - onRequest (default): formally, an application should dereference the remote element only on a post-loading event triggered for the purpose of dereferencing. In the context of this part of ISO/IEC 23009, the application dereferences the link only for those resources it needs (or anticipates it probably will need). Examples include dereferencing a link in a Period element when the play-time is expected to enter that Period, dereferencing an Adaptation Set link when it appears to contain Representations that will be needed, and so on. |

The restricted schema for XLINK in the context of ISO/IEC 23009 is referred to as "xlink.xsd" in any schema in this part of ISO/IEC 23009 and defined as follows:

<?xml version='1.0' encoding='UTF-8'?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.w3.org/1999/xlink" xmlns:xlink="http://www.w3.org/1999/xlink"> <xs:attribute name="type" type="xs:token" fixed="simple"/> <xs:attribute name="href" type="xlink:hrefType"/> <xs:simpleType name="hrefType"> <xs:restriction base="xs:anyURI"/> </xs:simpleType> <xs:attribute name="show" type="xs:token" fixed="embed"/> <xs:attribute name="actuate" type="xlink:actuateType" default="onRequest"/> <xs:simpleType name="actuateType"> <xs:restriction base="xs:token"> <xs:enumeration value="onLoad"/> <xs:enumeration value="onRequest"/> </xs:restriction> </xs:simpleType> </xs:schema>

### Processing

The following rules apply to the processing of URI references within @xlink:href:

  - URI references to remote elements that cannot be resolved shall be treated as invalid references and invalidate the MPD.

  - URI references to remote elements that are inappropriate targets for the given reference shall be treated as invalid references (see below for the appropriate targets) and invalidate the MPD.

  - URI references that directly or indirectly reference themselves are treated as invalid circular references and invalidate the MPD.

  - Any URI reference to a remote element shall be an HTTP-URL.

  - If a URI reference is relative then reference resolution as defined in 5.6.4 shall apply.

The remote elements referenced from within an MPD (referred to as appropriate targets) shall be embedded into the MPD by applying the following rules:

  - Attributes and elements obtained from the remote element shall be added to the element of the MPD that contains @xlink:href and shall be merged with the ones already present in the MPD. If the same attributes are present in both MPD and remote element, the attribute values should be the same. If they are not identical, then the value of the attribute of the MPD takes precedence over the value of the attribute in the remote element.

  - The remote element referenced by the @xlink:href shall conform to the type definition of the element in the MPD that contains @xlink:href.

  - All XLINK attributes shall be removed after dereferencing is completed.

  - Only a single element shall be included in a remote element.

  - All resources in the remote element referenced by @xlink:href shall have an availability end time as specified by MPD@availabilityEndTime.

## Hierarchical data model

### Introduction

#### Overview

A Media Presentation as described in the MPD consists of

- A sequence of one or more Periods as described in 5.5.2.

- Each Period contains one or more Adaptation Sets as described in 5.5.3. In case an Adaptation Set contains multiple media content components, then each media content component is described individually as defined in 5.5.4.

- Each Adaptation Set contains one or more Representations as described in 5.5.5.

- A Representation may contain one or more Sub-Representations as described in 5.5.6.

- Adaptation Sets, Representations and Sub-Representations share common attributes and elements that are described in 5.5.7.

- Each Period may contain one or more Subsets that restrict combination of Adaptation Sets for presentation. Subsets are described in 5.5.8.

- Each Representation consists of one or more Segments described in 6. Segment Information is introduced in 5.5.9. Segments contain media data and/or metadata to access, decode and present the included media content. Representations may also include Sub-Representations as defined in 5.5.6 to describe and extract partial information from a Representation.

- Each Segment consists of one or more Subsegments. Subsegments are described in 6.2.3.2.

The summary of the semantics of the attributes and elements within an MPD element are provided in Table 4 of 5.5.1.2. The XML syntax of the MPD element is provided in 5.5.1.3.

#### Semantics

Table 4 — Semantics of MPD element

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
| MPD |  |  |  | The root element that carries the Media Presentation Description for a Media Presentation. |
|  |  | @id | O | specifies an identifier for the Media Presentation. It is recommended to use an identifier that is unique within the scope in which the Media Presentation is published. |
|  |  | @profiles | O | specifies a the list of Media Presentation profiles as described in 8. The contents of this attribute shall conform to either the pro-simp or pro-fancy productions of RFC6381, Section 4.5, without the enclosing DQUOTE characters. As profile identifier the URI defined for the conforming Media Presentation profiles as described in 8 shall be used. |
|  |  | @type | OD default: static | specifies whether the Media Presentation Description may be updated (@type="dynamic") or not (@type="static"). NOTE Static MPDs are typically used for On-Demand services, whereas dynamic MPDs are used for live services. |
|  |  | @availabilityStartTime | CM Must be present for @type='dynamic' | For @type='dynamic' this attribute shall be present. In this case it specifies the anchor for the computation of the earliest availability time (in UTC) for any Segment in the Media Presentation. For @type=“static” if present, it specifies the Segment availability start time for all Segments referred to in this MPD. If not present, all Segments described in the MPD shall become available at the time the MPD becomes available. |
|  |  | @availabilityEndTime | O | specifies the latest Segment availability end time for any Segment in the Media Presentation. When not present, the value is unknown. |
|  |  | @mediaPresentationDuration | CM Must be present for @type='static' | specifies the duration of the entire Media Presentation. If the attribute is not present, the duration of the Media Presentation is unknown. In this case the attribute MPD@minimumUpdatePeriod shall be present. This attribute shall be present when the attribute MPD@minimumUpdatePeriod is not present. |
|  |  | @minimumUpdatePeriod | O | If this attribute is present, it specifies the smallest period between potential changes to the MPD. This can be useful to control the frequency at which a client checks for updates. If this attribute is not present it indicates that the MPD does not change. If MPD@type is ‘static’, @minimumUpdatePeriod shall not be present. Details on the use of the value of this attribute are specified in 5.3. |
|  |  | @minBufferTime | M | specifies a common duration used in the definition of the Representation data rate (see @bandwidth attribute in 5.5.5.2). |
|  |  | @timeShiftBufferDepth | O | specifies the duration of the time shifting buffer that is guaranteed to be available for a Media Presentation with type 'dynamic'. When not present, the value is infinite. This value of the attribute is undefined if the type attribute is equal to ‘static’. |
|  |  | @suggestedPresentationDelay | O | when @type is 'dynamic', it specifies a fixed delay offset in time from the presentation time of each access unit that is suggested to be used for presentation of each access unit. For more details refer to 7.2.1. when @type is 'static' the value of the attribute is undefined and may be ignored. |
|  |  | @maxSegmentDuration | O | specifies the maximum duration of any Segment in any Representation in the Media Presentation, i.e. documented in this MPD and any future update of the MPD. If not present, then the maximum Segment duration shall be the maximum duration of any Segment documented in this MPD. |
|  |  | @maxSubsegmentDuration | O | specifies the maximum duration of any Media Subsegment in any Representation in the Media Presentation. If not present, the same value as for the maximum Segment duration is implied. |
|  |  | ProgramInformation | 0…N | specifies descriptive information about the program. For more details refer to the description in 5.7. |
|  |  | BaseURL | 0…N | specifies a Base URL that can be used for reference resolution and alternative URL selection. For more details refer to the description in 5.6. |
|  | Location |  | 0…N | specifies a location at which the MPD is available. |
|  |  | Period | 1…N | specifies the information of a Period. For more details refer to the description in 5.5.2. |
|  |  | Metrics | 0 ... N | specifies the DASH Metrics. For more details see 5.9. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>…<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @ |  |  |  |  |

#### XML syntax

<!-- MPD Type --> <xs:complexType name="MPDtype"> <xs:sequence> <xs:element name="ProgramInformation" type="ProgramInformationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Location" type="xs:anyURI" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Period" type="PeriodType" maxOccurs="unbounded"/> <xs:element name="Metrics" type="MetricsType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:string"/> <xs:attribute name="profiles" type="xs:string"/> <xs:attribute name="type" type="PresentationType" default="static"/> <xs:attribute name="availabilityStartTime" type="xs:dateTime"/> <xs:attribute name="availabilityEndTime" type="xs:dateTime"/> <xs:attribute name="mediaPresentationDuration" type="xs:duration"/> <xs:attribute name="minimumUpdatePeriod" type="xs:duration"/> <xs:attribute name="minBufferTime" type="xs:duration" use="required"/> <xs:attribute name="timeShiftBufferDepth" type="xs:duration"/> <xs:attribute name="suggestedPresentationDelay" type="xs:duration"/> <xs:attribute name="maxSegmentDuration" type="xs:duration"/> <xs:attribute name="maxSubsegmentDuration" type="xs:duration"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Presentation Type enumeration --> <xs:simpleType name="PresentationType"> <xs:restriction base="xs:string"> <xs:enumeration value="static"/> <xs:enumeration value="dynamic"/> </xs:restriction> </xs:simpleType>

### Period

#### Overview

A Media Presentation consists of one or more Periods. A Period is defined by a Period element in the MPD element.

The type of the Period, either a regular Period or an Early Available Period, as well as the PeriodStart time of a regular Period is determined as follows:

- If the attribute @start is present in the Period, then the Period is a regular Period and the PeriodStart is equal to the value of this attribute.

- If the @start attribute is absent, but the previous Period element contains a @duration attribute then then this new Period is also a regular Period. The start time of the new Period PeriodStart is the sum of the start time of the previous Period PeriodStart and the value of the attribute @duration of the previous Period.

- If (i) @start attribute is absent, and (ii) and the Period element is the first in the MPD, and (iii) the MPD@type is 'static', then the PeriodStart time shall be set to zero.

- If (i) @start attribute is absent, and (ii) the previous Period element does not contains a @duration attribute or the Period element is the first in the MPD, and (iii) the MPD@type is 'dynamic', then this Period is an Early Available Period (see below for details).

For any regular Period the following holds: PeriodStart reflects the actual time that should elapse after playing the media of all prior Periods in this Media Presentation relative to the PeriodStart time of the first Period in the Media Presentation. The Period extends until the PeriodStart of the next Period, or until the end of the Media Presentation in the case of the last Period. More specifically, the difference between the PeriodStart time of a Period and either the PeriodStart time of the following Period, if this is not the last Period, or the value of the MPD@mediaPresentationDuration if this is the last one, is the presentation duration in Media Presentation timeline of the media content represented by the Representations in this Period.

Early Available Periods may be used to advertise initialisation of other non-media data before the media data itself is available. Period elements documenting early available Periods shall not occur before any Period element documenting a regular Period. For Early Available Periods, any resources that are announced in such a Period element shall be available. Such a Period element shall not contain URLs to Media Segments. The data contained in such a Period element does not represent a Period in the Media Presentation. Only when the PeriodStart time becomes known through an update of the MPD, such a Period element represents a regular Period. However, an update of the MPD may even remove a Period element representing an Early Available Period in later updates of the MPD as long as no PeriodStart time is associated with the Period.

To avoid dereferencing of a remote element containing a Period element solely to determine the Period timeline, e.g. in case of seeking, Period@start or previous Period's Period@duration should be present in the MPD.

The semantics of the attributes and elements within a Period element are provided in Table 5 of 5.5.2.2. The XML syntax of the Period element is provided in 5.5.2.3.

#### Semantics

Table 5 — Semantics of Period element

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | Period |  |  | specifies the information of a Period. |
|  |  | @xlink:href | O | specifies a reference to an external Period element |
|  |  | @xlink:actuate | OD default: onRequest | specifies the processing instructions, which can be either "onLoad" or "onRequest". This attribute shall not be present if the @xlink:href attribute is not present. |
|  |  | @id | O | specifies an identifier for this Period. The idenfifier shall be unique within the scope of the Media Presentation. |
|  |  | @start | O | if present, specifies the PeriodStart time of the Period. The PeriodStart time is used as an anchor to determine the MPD start time of each Media Segment as well as to determine the presentation time of each each access unit in the Media Presentation timeline. |
|  |  | @duration | O | if present specifies the duration of the Period to determine the PeriodStart time of the next Period. |
|  |  | @bitstreamSwitching | OD Default: false | When set to ‘true’, this is equivalent as if the AdaptationSet@bitstreamSwitching for each Adaptation Set contained in this Period is set to 'true'. In this case, the AdaptationSet@bitstreamSwitching attribute shall not be set to 'false' for any Adaptation Set in this Period. |
|  |  | BaseURL | 0…N | specifies a base URL that can be used for reference resolution and alternative URL selection. For more details refer to the description in 5.6. |
|  |  | SegmentBase | 0...1 | specifies default Segment Base information. Information in this element may be overridden by information in AdapationSet.SegmentBase and Representation.SegmentBase. For more details see 5.5.9. |
|  |  | SegmentList | 0...1 | specifies default Segment List information. Information in this element may be overridden by information in AdapationSet.SegmentList and Representation.SegmentList. For more details see 5.5.9. |
|  |  | SegmentTemplate | 0...1 | specifies default Segment Template information. Information in this element may be overridden by information in AdapationSet.SegmentTemplate and Representation.SegmentTemplate. For more details see 5.5.9. |
|  |  | AdaptationSet | 0...N | specifies an Adaptation Set. At least one Adaptation Set shall be present in each Period. However, the actual element may be present only in a remote element if xlink is in use, For more details see 5.5.3. |
|  |  | Subset | 0...N | specifies a Subset. For more details see 5.5.8. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Note that the conditions only holds without using xlink:href. If linking is used, then all attributes are "optional" and <minOccurs=0> Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |

#### XML syntax

<!-- Period --> <xs:complexType name="PeriodType"> <xs:sequence> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="AdaptationSet" type="AdaptationSetType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Subset" type="SubsetType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="id" type="xs:string" /> <xs:attribute name="start" type="xs:duration"/> <xs:attribute name="duration" type="xs:duration"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean" default="false"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

### Adaptation Sets

#### Overview

Each Period consists of one or more Adaptation Sets. An Adaptation Set is described by an AdaptationSet element. AdaptationSet elements are contained in a Period element.

An Adaptation Set contains alternate encodings of one or more media content components. Each alternate encoding is contained in a Representation as described in 5.5.5. All Representations contained in one Adaptation Set represent the same media content components and therefore contain media streams that are considered to be perceptually equivalent.

Representations are arranged into Adaptation Sets according to the media content component properties of the media content components present in the Representations, namely

- the language as described by the @lang attribute,

- the media component type described by the @contentType attribute,

- the picture aspect ratio as described by the @par attribute,

- the role property as described by the Role elements,

- the accessibility property as described by the Accessibility elements,

- the viewpoint property as described by the ViewPoint elements,

- the rating property as described by the Rating elements.

- Representations shall appear in the same Adaptation Set if and only if they have identical values for all of these media content component properties for each media content component.

- The values for the elements Role, Accessibility, ViewPoint and Rating are generally not provided within the scope of this part of ISO/IEC 23009. However, a number of simple schemes are defined in 5.8.5.

If there exist multiple media content components then the properties of each media content component shall be described by a separate ContentComponent element as defined in 5.5.4. The ContentComponent element shares common elements and attributes with the AdaptationSet element. Default values or values applicable to all media content components may be provided directly in the AdaptationSet element. Attributes present in the AdaptationSet shall not be repeated in the ContentComponent element.

The AdaptationSet element may contain default values for elements and attributes associated to the contained Representations. The list of possible present elements and attributes that are common to AdaptationSet and Representation (and also SubRepresentation) are collected in 5.5.7. Any of the common attributes shall only be present either in the AdaptationSet element or in the Representation element, but not in both.

The AdaptationSet element also supports the description of ranges for the @bandwidth, @width, @height and @frameRate attributes associated to the contained Representations, which provide a summary of all values for all the Representations within this Adaptation Set. The Representations contained with an Adaptation Set shall not contain values outside the ranges documented for that Adaptation Set.

Adaptation Sets may be further arranged into groups using the @group attribute. The semantics of this grouping is that the media content within one Period is represented by:

  - either one Representation from group 0, if present,

  - or the combination of at most one Representation from each non-zero group.

- If the AdaptationSet@group attribute is not present then all Representations in this Adaptation Set are assigned to a non-zero group specific to this Adaptation Set.

- The semantics of the attributes and elements within an AdaptationSet element are provided in Table 6 of 5.5.3.2. The XML syntax of the AdaptationSet element is provided in 5.5.3.3.

#### Semantics

Table 6 — Semantics of AdaptationSet element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | AdaptationSet |  |  | Adaptation Set description |
|  |  |  | @xlink:href | O | specifies a reference to external AdaptationSet element |
|  |  |  | @xlink:actuate | OD default: 'onRequest' | specifies the processing instructions, which can be either "onLoad" or "onRequest". |
|  |  |  | @id | O | specifies an unique identifier for this Adaptation Set in the scope of the Period. The attribute shall be unique in the scope of the containing Period. The attribute shall not be present in a remote element. |
|  |  |  | @group | O | specifies an identifier for the group that is unique in the scope of the containing Period. |
|  |  |  | CommonAttributesElements | - | specifies the common attributes and elements (see 5.5.7) |
|  |  |  | @lang | O | Declares the language code(s) for this Adaptation Set. The syntax and semantics according to IETF RFC 5646 shall be used. |
|  |  |  | @contentType | O | specifies the media content component type for this Adaptation Set. A value of the top-level Content-type 'type' value as defined in RFC1521, section 4 shall be taken. |
|  |  |  | @par | O | specifies the picture aspect ratio of the video media component type, in the form of a string consisting of two integers separated by ‘:’, e.g.,”16:9”. When this attribute is present, and the attributes @width and @height for the set of Representations are also present, the picture aspect ratio as specified by this attribute shall be the same as indicated by the values of @width, @height, and @sar, i.e. it shall express the same ratio as (@width * sarx): (@height * sary), with sarx the first number in @sar and sary the second number. |
|  |  |  | @minBandwidth | O | specifies the minimum @bandwidth value in all Representations in this Adaptation Set. This value has the same units as the @bandwidth attribute. |
|  |  |  | @maxBandwidth | O | specifies the maximum @bandwidth value in all Representations in this Adaptation Set. This value has the same units as the @bandwidth attribute. |
|  |  |  | @minWidth | O | specifies the minimum @width value in all Representations in this Adaptation Set. This value has the same units as the @width attribute. |
|  |  |  | @maxWidth | O | specifies the maximum @width value in all Representations in this Adaptation Set. This value has the same units as the @width attribute. |
|  |  |  | @minHeight | O | specifies the minimum @height value in all Representations in this Adaptation Set. This value has the same units as the @height attribute. |
|  |  |  | @maxHeight | O | specifies the maximum @height value in all Representations in this Adaptation Set. This value has the same units as the @height attribute. |
|  |  |  | @minFrameRate | O | specifies the minimum @framerate value in all Representations in this Adaptation Set. This value is encoded in the same format as the @frameRate attribute. |
|  |  |  | @maxFrameRate | O | specifies the maximum @framerate value in all Representations in this Adaptation Set. This value is encoded in the same format as the @frameRate attribute. |
|  |  |  | @segmentAlignment | OD default: false | - when not set to ‘false’, this specifies that for any two Representations, X and Y, within the same Adaptation Set, the m-th Segment of X and the n-th Segment of Y are non-overlapping (as defined in 4.5.2) whenever m is not equal to n. - For Adaptation Sets containing Representations with multiple media content components, this attribute value shall be either 'true' or 'false'. - For Adaptation Sets containing Representations with a single media content component, when two AdaptationSet elements within a Period share the same integer value for this attribute, then for any two Representations, X and Y, within the union of the two Adaptation Sets, the m-th Segment of X and the n-th Segment of Y are non-overlapping (as defined in 4.5.2) whenever m is not equal to n. |
|  |  |  | @bitstreamSwitching | O | When this flag is set to ‘true’, the following applies: - All Representations in the Adaptation Set shall have the same number M of Media Segments; - Let R1, R2, ..., RN be all the Representations within the Adaptation Set. - Let - Si,j, for j > 0, be the jth Media Segment in the ith Representation (i.e., Ri) - if present, let Si,0 be the Initialisation Segment in the ith Representation, and - if present, let Bi be the Bitstream Switching Segment in the ith Representation. - The sequence of - any Initialisation Segment, if present, in the Adaptation Set, with, - if Bitstream Switching Segments are present, Bi(1), Si(1),1, Bi(2), Si(2),2, ..., Bi(k), Si(k),k, ..., Bi(M), Si(M),M - else Si(1),1, Si(2),2, ..., Si(k),k, ..., Si(M),M, wherein any i(k) for all k values in the range of 1 to M, respectively, is an integer value in the range of 1 to N, results in a "conforming Segment sequence" as defined in 4.5.3 with the media format as specified in the @mimeType attribute. - More detailed rules may be defined for specific media formats. |
|  |  |  | @subsegmentAlignment | OD default: false | If the @subsegmentAlignment for an Adaptation Set is set to other than ‘false’, all following conditions shall be satisfied: - Each Media Segment shall be indexed (i.e. either it contains a Segment index or there is an Index Segment providing an index for the Media Segment) - For any two Representations, X and Y, within the same Adaptation Set, the m-th Subsegment of X and the n-th Subsegment of Y are non-overlapping (as defined in 4.5.2) whenever m is not equal to n. - For Adaptation Sets containing Representations with a single media content component, when two AdaptationSet elements within a Period share the same integer value for this attribute, then for any two Representations, X and Y, within the union of the two Adaptation Sets, the m-th Subsegment of X and the n-th Subsegment of Y are non-overlapping (as defined in 4.5.2) whenever m is not equal to n. |
|  |  |  | @subsegmentStartsWithSAP | OD default: 0 | - when greater than 0, specifies that each Subsegment with SAP_type greater than 0 starts with a SAP of type less than or equal to the value of @subsegmentStartsWithSAP. A Subsegment starts with SAP when the Subsegment contains a SAP, and for the first SAP, ISAU is the index of the first access unit that follows ISAP, and ISAP is contained in the Subsegment. - The semantics of @subsegmentStartsWithSAP equal to 0 are unspecified. |
|  |  |  | Accessibility | 0 … N | - specifies information about accessibility scheme - For more details refer to 5.8.1 and 5.8.4.3. |
|  |  |  | Role | 0 … N | specifies information on role annotation scheme - For more details refer to 5.8.1 and 5.8.4.2. |
|  |  |  | Rating | 0 … N | - specifies information on rating scheme. - For more details refer to 5.8.1 and 5.8.4.4. |
|  |  |  | Viewpoint | 0 … N | - specifies information on viewpoint annotation scheme. - For more details refer to 5.8.1 and 5.8.4.5. |
|  |  |  | ContentComponent | 0…N | specifies the properties of one media content component contained in this Adaptation Set. For more details refer to 5.5.4. |
|  |  |  | BaseURL | 0…N | specifies a base URL that can be used for reference resolution and alternative URL selection. For more details refer to the description in 5.6. |
|  |  |  | SegmentBase | 0...1 | specifies default Segment Base information. Information in this element may be overridden by information in the Representation.SegmentBase. For more details see 5.5.9. |
|  |  |  | SegmentList | 0...1 | specifies default Segment List information. Information in this element may be overridden by information in the Representation.SegmentList. For more details see 5.5.9. |
|  |  |  | SegmentTemplate | 0...1 | specifies default Segment Template information. Information in this element may be overridden by information in the Representation.SegmentTemplate. For more details see 5.5.9. |
|  |  |  | Representation | 0 … N | specifies a Representation. At least one Representation element shall be present in each Adaptation Set. The actual element may however be part of a remote element. For more details refer to 5.5.5. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory, F=Fixed. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Note that the conditions only holds without using xlink:href. If linking is used, then all attributes are "optional" and <minOccurs=0> Elements are bold; attributes are non-bold and preceded with an @, List of elements and attributes is in italics bold |  |  |  |  |  |

#### XML syntax

<!-- Adaptation Set --> <xs:complexType name="AdaptationSetType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentComponent" type="ContentComponentType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="Representation" type="RepresentationType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="group" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="xs:string"/> <xs:attribute name="par" type="xs:string"/> <xs:attribute name="minBandwidth" type="xs:unsignedInt"/> <xs:attribute name="maxBandwidth" type="xs:unsignedInt"/> <xs:attribute name="minWidth" type="xs:unsignedInt"/> <xs:attribute name="maxWidth" type="xs:unsignedInt"/> <xs:attribute name="minHeight" type="xs:unsignedInt"/> <xs:attribute name="maxHeight" type="xs:unsignedInt"/> <xs:attribute name="minFrameRate" type="xs:string"/> <xs:attribute name="maxFrameRate" type="xs:string"/> <xs:attribute name="segmentAlignment" type="ConditionalUintType" default="false"/> <xs:attribute name="subsegmentAlignment" type="ConditionalUintType" default="false"/> <xs:attribute name="subsegmentStartsWithSAP" type="SAPType" default="0"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Conditional Unsigned Integer (unsignedInt or boolean) --> <xs:simpleType name="ConditionalUintType"> <xs:union memberTypes="xs:unsignedInt xs:boolean"/> </xs:simpleType>

### Media Content Component

#### Overview

Each Adaptation Set contains one or more media content components. The properties of each media content component are described by a ContentComponent element or may be described directly on the AdaptationSet element if only one media content component is present in the Adaptation Set. ContentComponent elements are contained in an AdaptationSet element.

- The semantics of the attributes and elements within a ContentComponent element are provided in Table 6 of 5.5.3.2. The XML syntax of the ContentComponent element is provided in 5.5.3.3.

#### Semantics

Table 7 — Semantics of ContentComponent element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | ContentComponent |  |  | description of a content component |
|  |  |  | @id | O | specifies an identifier for this media component. The attribute shall be unique in the scope of the containing Adaptation Set. |
|  |  |  | @lang | O | same semantics as in Table 6 for @lang attribute |
|  |  |  | @contentType | O | same semantics as in Table 6 for @contentType attribute |
|  |  |  | @par | O | same semantics as in Table 6 for @par attribute. |
|  |  |  | Accessibility | 0 … N | same semantics as in Table 6 for Accessibility element |
|  |  |  | Role | 0 … N | same semantics as in Table 6 for Role element |
|  |  |  | Rating | 0 … N | same semantics as in Table 6 for Rating element |
|  |  |  | Viewpoint | 0 … N | same semantics as in Table 6 for ViewPoint element |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory, F=Fixed. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @, List of elements and attributes is in italics bold |  |  |  |  |  |

#### XML syntax

<!-- Content Component --> <xs:complexType name="ContentComponentType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="xs:string"/> <xs:attribute name="par" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

### Representation

#### Overview

Representations are described by the Representation element. Representation elements are contained in an AdaptationSet element.

A Representation is one of the alternative choices of the complete set or subset of media content components comprising the media content during the defined Period.

A Representation starts at the start of the Period PeriodStart and continues to the end of the Period, i.e. the start of the next Period or the end of the Media Presentation.

Each Representation includes one or more media streams, where each media stream is an encoded version of one media content component.

A Representation consists of one or more Segments.

Each Representation either shall contain an Initialisation Segment or each Media Segment in the Representation shall be self-initialising, i.e. the Media Segment itself conforms to the media type as specified in the @mimeType attribute for this Representation.

When a Representation is not a dependent Representation, i.e. the @dependencyId attribute is absent, then concatenation of the Initialisation Segment, if present, and all consecutive Media Segments in one Representation shall represent a conforming Segment sequence as defined in 4.5.3 conforming to the media type as specified in the @mimeType attribute for this Representation.

Dependent Representations are described by a Representation element that contains a @dependencyId attribute. Dependent Representations are regular Representations except that they depend on a set of complementary Representations for decoding and/or presentation. The @dependencyId contains the values of the @id attribute of all the complementary Representations, i.e. Representations that are necessary to present and/or decode the media content components contained in this dependent Representation.

For any dependent Representation, X that depends on complementary Representation Y, the m-th Subsegment of X and the n-th Subsegment of Y shall be non-overlapping (as defined in 4.5.3) whenever m is not equal to n. For dependent Representations the concatenation of the Initialisation Segment with the sequence of Subsegments of the dependent Representations, each being preceded by the corresponding Subsegment of each of the complementary Representations in order as provided in the @dependencyId attribute shall represent a conforming Subsegment sequence as defined in 4.5.3 conforming to the media format as specified in the @mimeType attribute for this dependent Representation.

NOTE	When decoding of a dependent Representation is started from a SAP in the (Sub)Segment with number i, the decoding process does not need to access data from the complementary Representation(s) from any earlier (sub)segments than (sub)Segment with number i of the complementary Representation(s).

The semantics of the attributes and elements within a Representation are provided in Table 8 of 5.5.5.2. The XML syntax of the Representation type is provided in 5.5.5.3.

#### Semantics

Table 8 —Semantics of Representation element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Representation |  |  | This element contains a description of a Representation. |
|  |  |  | @id | M | specifies a identifier for this Representation. The identifier shall be unique within a Period unless the Representation is functionally identically to another Representation in the same Period. The identifier shall not contain whitespace characters. If used in the template-based URL construction as defined in 5.5.9.4.4, the string shall only contain characters that are permitted within an HTTP-URL according to RFC 1738. |
|  |  |  | @bandwidth | M | specifies a bound on the data rate and data rate variation of the Representation as follows: Consider a hypothetical constant bitrate channel of bandwidth with the value of this attribute in bits per second (bps). Then, if the Representation is continuously delivered over this channel, starting at any SAP that is indicated either by @startWithSAP or by any Segment Index box, a client can be assured of having enough data for continuous playout providing playout begins after @minBufferTime * @bandwidth bits have been received (i.e. at time @minBufferTime after the first bit is received). For dependent Representations this value shall specify the minimum bandwidth as defined above of this Representation and all complementary Representations. |
|  |  |  | @qualityRanking | O | specifies a quality ranking of the Representation relative to other Representations in the same Adaptation Set. Lower values represent higher quality content. If not present then no ranking is defined. |
|  |  |  | @dependencyId | O | specifies all complementary Representations the Representation depends on in the decoding and/or presentation process as a whitespace-separated list of values of @id attributes. This attribute shall not be present where there are no dependencies. |
|  |  |  | @mediaStreamStructureId | O | The attribute may be present for Representations containing video and its semantics are unspecified for any other type of Representations. If present, the attribute @mediaStreamStructureId specifies a whitespace-separated list of media stream structure identifier values. If media streams share the same media stream structure identifier value, the media streams shall have the following characteristics: - The media streams have the same number of Stream Access Points of type 1 to 3. - The values of TSAP, TDEC, TEPT, and TPTF of the i-th SAP of type 1 to 3 in one media stream are identical to the values of TSAP, TDEC, TEPT, and TPTF, respectively, of the i-th SAP of type 1 to 3 in the other media streams for any value of i from 1 to the number of SAPs of type 1 to 3 in any of the media streams. - A media stream formed by concatenating the media stream of a first Representation until ISAU (exclusive) of the i-th SAP of type 1 to 3 and the media stream of a second Representation (having the same media stream structure identifier value as for the first Representation) starting from the ISAU (inclusive) of the i-th SAP of type 1 to 3 conforms to the specification in which the media stream format is specified for any value of i from 1 to the number of SAPs of type 1 to 3 in either media stream. Furthermore, the decoded pictures have an acceptable quality regardless of type of the Stream Access Point access unit used. All media stream structure identifier values for one Adaptation Set shall differ from those of another Adaptation Set. NOTE Indicating multiple media stream structure identifier values for a Representation can be useful in cases where switching between Representations A and B as well as between Representations B and C is allowed at non-IDR intra pictures, but switching between Representations A and C would cause too severe a degradation in the quality of the leading pictures and is hence not allowed. To indicate these permissions and restrictions, Representation A would contain @mediaStreamStructureId equal to “1”, Representation B would contain @mediaStreamStructureId equal to “1 2”, and Representation C would contain @mediaStreamStructureId equal to “2” |
|  |  |  | CommonAttributesElements | - | Common Attributes and Elements, for more details see 5.5.7 |
|  |  |  | BaseURL | 0…N | specifies a Base URL that can be used for reference resolution and alternative URL selection. For more details refer to the description in 5.6. |
|  |  |  | SubRepresentation | 0 … N | specifies information about a Sub-Representation that is embedded in the containing Representation. For more details see 5.5.6. |
|  |  |  | SegmentBase | 0...1 | specifies default Segment Base information. For more details see 5.5.9. |
|  |  |  | SegmentList | 0 ... 1 | specifies the Segment List information. For more details see 5.5.9. |
|  |  |  | SegmentTemplate | 0 ... 1 | specifies the Segment Template information. For more details see 5.5.9. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @, List of elements and attributes is in italics bold |  |  |  |  |  |

#### XML syntax

<!-- Representation --> <xs:complexType name="RepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SubRepresentation" type="SubRepresentationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:attribute name="dependencyId" type="StringVectorType"/> <xs:attribute name="mediaStreamStructureId" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Whitespace-separated list of strings --> <xs:simpleType name="StringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType>

### Sub-Representation

#### Overview

Sub-Representations are embedded in regular Representations and are described by the SubRepresentation element. SubRepresentation elements are contained in a Representation element.

The SubRepresentation element describes properties of one or several media content components that are embedded in the Representation. It may for example describe the exact properties of an embedded audio component (e.g., codec, sampling rate, etc.), an embedded sub-title (e.g., codec) or it may describe some embedded lower quality video layer (e.g. some lower frame rate, etc.).

Sub-Representations and Representation share some common attributes and elements.

In case the @level attribute is present in the SubRepresentation element,

- Sub-Representations provide the ability for accessing a lower quality version of the Representation in which they are contained. In this case, Sub-Representations for example allow extracting the audio track in a multiplexed Representation or may allow for efficient fast-forward or rewind operations if provided with lower frame rate;

- the Initialisation Segment and/or the Media Segments and/or the Index Segments shall provide sufficient information such that the data can be easily accessed through HTTP partial GET requests. The details on providing such information shall be defined by the media format in use. For media formats defined in this part of ISO/IEC 23009, the Subsegment Index as defined in 6.3.2.4 shall be used.

If the @level attribute is absent, then the SubRepresentation element is solely used to provide a more detailed description for media streams that are embedded in the Representation.

The semantics of the attributes and elements within a Representation are provided in Table 9 of 5.5.6.2. The XML syntax of the Representation type is provided in 5.5.6.3.

#### Semantics

Table 9 —Semantics of SubRepresentation element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | SubRepresentation |  |  | specifies a Sub-Representation. |
|  |  |  | @level | O | specifies the Sub-Representation level. If @level attribute is present and for media formats used in this part of ISO/IEC 23009, a Subsegment Index as defined in 6.3.2.4 shall be available for each Media Segment in the containing Representation. |
|  |  |  | @dependencyLevel | O | specifies the set of Sub-Representations within this Representation that this Sub-Representation depends on in the decoding and/or presentation process as a whitespace-separated list of @level values. |
|  |  |  | @bandwidth | CM shall be present if @level is present | Identical to the @bandwidth definition in Representation, but applied to this Sub-Representation. This attribute shall be present if the @level attribute is present. |
|  |  |  | @contentComponent | O | if present, specifies the set of all media content components that are contained in this Sub-Representation as a whitespace-separated list of values of ContentComponent@id values. |
|  |  |  | CommonAttributesElements | - | Common Attributes and Elements. For details see 5.5.7. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold, List of elements and attributes is in italics bold |  |  |  |  |  |

#### XML syntax

<!-- SubRepresentation --> <xs:complexType name="SubRepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:attribute name="level" type="xs:unsignedInt"/> <xs:attribute name="dependencyLevel" type="UIntVectorType"/> <xs:attribute name="bandwidth" type="xs:unsignedInt"/> <xs:attribute name="contentComponent" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Whitespace-separated list of unsigned integers --> <xs:simpleType name="UIntVectorType"> <xs:list itemType="xs:unsignedInt"/> </xs:simpleType>

### Common attributes and elements

#### Overview

The elements AdaptationSet, Representation and SubRepresentation have assigned common attributes and elements. The attributes and elements listed in Table 10 of 5.5.6.2 may be present in all three elements.

The semantics of the common attributes and elements are provided in Table 10 in 5.5.7.2, the syntax is provided in 5.5.7.3.

The 'Use' column in Table 10 shall be interpreted that an attribute marked with 'M' shall be available for a Representation, i.e. it shall either be present in the Representation element, or if not, it shall be in the containing AdaptationSet element. An attribute marked with 'O' may be absent in both.

#### Semantics

Table 10 — Common Adaptation Set, Representation and Sub-Representation attributes and elements

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Common attributes and elements |  |  |  |
|  |  |  | @profiles | O | specifies the profiles which the associated Representation(s) conform to the list of Media Presentation profiles as described in 8. The value shall be a subset of the respective value in any higher level of the document hierarchy (Representation, Adaptation Set, MPD). If not present, the value is inferred to be the same as in the next higher level of the document hierarchy. For example, if the value is not present for a Representation, then @profiles at the Adaptation Set level is valid for the Representation. The same syntax as defined in 5.5.1.2 shall be used. |
|  |  |  | @width | O | specifies the horizontal visual presentation size of the video media type on a grid determined by the @sar attribute. In the absence of @sar width and height are specified as if the value of @sar were "1:1" NOTE The visual presentation size of the video is equal to the number of horizontal and vertical samples used for presentation after encoded samples are cropped in response to encoded cropping parameters, “overscan” signaling, or “pan/scan” display parameters, e.g. SEI messages. |
|  |  |  | @height | O | specifies the vertical visual presentation size of the video media type, on a grid determined by the @sar attribute. |
|  |  |  | @sar | O | specifies the sample aspect ratio of the video media component type, in the form of a string consisting of two integers separated by ‘:’, e.g.,”10:11”. The first number specifies the horizontal size of the encoded video pixels (samples) in arbitrary units. The second number specifies the vertical size of the encoded video pixels (samples) in same units as the horizontal size. |
|  |  |  | @frameRate | O | specifies the output frame rate (or in the case of interlaced, half the output field rate) of the video media type in the Representation. If the frame or field rate is varying, the value is the average frame or half the average field rate field rate over the entire duration of the Representation. The value is coded as a string, either containing two integers separated by a "/", ("F/D"), or a single integer "F". The frame rate is the division F/D, or F, respectively, per second (i.e. the default value of D is “1”). |
|  |  |  | @audioSamplingRate | O | Either a single decimal integer value specifying the sampling rate or a whitespace separated pair of decimal integer values specifying the minimum and maximum sampling rate of the audio media component type. The values are in samples per second. |
|  |  |  | @mimeType | M | specifies the MIME type of the concatenation of the Initialisation Segment, if present, and all consecutive Media Segments in the Representation. |
|  |  |  | @segmentProfiles | O | specifies the profiles of Segments that are essential to process the Representation. The detailed semantics depend on the value of the @mimeType attribute. The contents of this attribute shall conform to either the pro-simp or pro-fancy productions of RFC6381, Section 4.5, without the enclosing DQUOTE characters. As profile identifier the brand identifier for the Segment as defined in 6 shall be used. |
|  |  |  | @codecs | M | specifies the codecs present within the Representation. The codec parameters shall also include the profile and level information where applicable. The contents of this attribute shall conform to either the simp-list or fancy-list productions of RFC6381, Section 3.2, without the enclosing DQUOTE characters. The codec identifier for the Representation's media format, mapped into the name space for codecs as specified in RFC6381, Section 3.3, shall be used. |
|  |  |  | @maximumSAPPeriod | O | when present, specifies the maximum SAP interval in seconds of all contained media streams, where the SAP interval is the maximum time interval between the TSAP of any two successive SAPs of types 1 to 3 inclusive of one media stream in the associated Representations. |
|  |  |  | @startWithSAP | O | when present and greater than 0, specifies that in the associated Representations, each Media Segment starts with a SAP of type less than or equal to the value of this attribute value in each media stream. A Media Segment starts with a SAP in a media stream if the stream contains a SAP in that Media Segment, ISAU is the index of the first access unit that follows ISAP and ISAP is contained in the Media Segment. |
|  |  |  | @maxPlayoutRate | O | specifies the maximum playout rate as a multiple of the regular playout rate, which is supported with the same decoder profile and level requirements as the normal playout rate. |
|  |  |  | @codingDependency | O | When present and ‘true’, for all contained media streams, specifies that there is at least one access unit that depends on one or more other access units for decoding. When present and ‘false’, for any contained media stream, there is no access unit that depends on any other access unit for decoding (e.g. for video all the pictures are intra coded). If not specified on any level, there may or may not be coding dependency between access units. |
|  |  |  | @scanType | O | specifies the scan type of the source material of the video media component type. The value may be equal to one of “progressive”, “interlaced” and “unknown”. If not specified on any level, the scan type is "progressive". |
|  |  |  | FramePacking | 0 … N | specifies frame-packing arrangement information of the video media component type. When no FramePacking element is provided for a video component, frame-packing shall not used for the video media component. For details see 5.8.1 and 5.8.4.6 |
|  |  |  | AudioChannelConfiguration | 0 … N | - specifies the audio channel configuration of the audio media component type. For details see 5.8.1 and 5.8.4.7. |
|  |  |  | ContentProtection | 0 … N | specifies information about content protection schemes used for the associated Representations. For details see 5.8.1 and 5.8.4.1. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>..<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |

#### XML syntax

<!-- Representation base (common attributes and elements) --> <xs:complexType name="RepresentationBaseType"> <xs:sequence> <xs:element name="FramePacking" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="AudioChannelConfiguration" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentProtection" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="profiles" type="xs:string"/> <xs:attribute name="width" type="xs:unsignedInt"/> <xs:attribute name="height" type="xs:unsignedInt"/> <xs:attribute name="sar" type="xs:string"/> <xs:attribute name="frameRate" type="xs:string"/> <xs:attribute name="audioSamplingRate" type="xs:string"/> <xs:attribute name="mimeType" type="xs:string"/> <xs:attribute name="segmentProfiles" type="xs:string"/> <xs:attribute name="codecs" type="xs:string"/> <xs:attribute name="maximumSAPPeriod" type="xs:double"/> <xs:attribute name="startWithSAP" type="SAPType"/> <xs:attribute name="maxPlayoutRate" type="xs:double"/> <xs:attribute name="codingDependency" type="xs:boolean"/> <xs:attribute name="scanType" type="VideoScanType"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Stream Access Point type enumeration --> <xs:simpleType name="SAPType"> <xs:restriction base="xs:unsignedInt"> <xs:minInclusive value="0"/> <xs:maxInclusive value="6"/> </xs:restriction> </xs:simpleType> <!-- Video Scan type enumeration --> <xs:simpleType name="VideoScanType"> <xs:restriction base="xs:string"> <xs:enumeration value="progressive"/> <xs:enumeration value="interlaced"/> <xs:enumeration value="unknown"/> </xs:restriction> </xs:simpleType>

### Subsets

#### Overview

Subsets are described by the Subset element contained in the Period element.

Subsets provide a mechanism to restrict the combination of active Adaptation Sets where an active Adaptation Set is one for which the DASH client is presenting at least one of the contained Representations.

A Subset defines a set of one or more Adaptation Sets. The presence of a Subset element within a Period element expresses the intention of the creator of the Media Presentation that a client should act as follows: At any time, the set of active Adaptation Sets shall be a subset of the Adaptation Sets of one of the specified Subsets. Any Adaptation Set not explicitly contained in any Subset element is implicitly contained in all specified Subsets.

This implies that

- Empty Subsets are not allowed.

- No Subset should contain all the Adaptation Sets.

Each Adaptation Set for which the value of the @id is provided in the @contains attribute is contained in this Subset.

The semantics of the attributes and elements within a Subset are provided in Table 11 of 5.5.8.2. The XML syntax of the Subset type is provided in 5.5.8.3.

#### Semantics

Table 11 — Subset element semantics

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | Subset |  |  | specifies a Subset |
|  |  | - @contains | M | specifies the Adaptation Sets contained in a Subset by providing a white-space separated list of the @id values of the contained Adaptation Sets. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |

#### XML syntax

<!-- Subset --> <xs:complexType name="SubsetType"> <xs:attribute name="contains" type="UIntVectorType" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

### Segments and Segment information

#### General

A Segment is the smallest addressable unit described by an MPD and has a defined format. Segment formats are defined in 6. This clause defines the MPD information for Segments.

Specifically, a Segment shall referenced by an HTTP-URL included in the MPD, where an HTTP-URL is defined as an <absolute-URI> according to RFC 3986, Clause 4.3, with a fixed scheme of “http://” or "https://", possibly restricted by a byte range if a range attribute is provided together with the URL. The byte range shall be expressed as a byte-range-spec as defined in RFC 2616, Clause 14.35.1. It is restricted to a single expression identifying a contiguous range of bytes.

Each Segment referenced through an HTTP-URL in the MPD is associated with a Segment availability interval, i.e. a time window in wall-clock time at which the Segments can be accessed via the HTTP-URL. The Segment availability interval window is described by a Segment availability start time and a Segment availability end time.

- Representations are assigned Segment Information through the presence of the elements BaseURL, SegmentBase, SegmentTemplate and/or SegmentList. The Segment Information provides information on the location, availability and properties of all Segments contained in one Representation. Specifically, information on the presence and location of Initialisation, Media, Index and Bitstream Switching Segments is provided.

The elements SegmentBase, SegmentTemplate and SegmentList may be present in the Representation element itself. In addition, to express default values, they may be present in the Period and AdaptationSet element. At each level at most one of the three, SegmentBase, SegmentTemplate and SegmentList shall be present. Further, if SegmentTemplate or SegmentList on one level of the hierarchy, then the other one shall not be present on any lower hierarchy level.

- SegmentBase, SegmentTemplate and SegmentList shall inherit attributes and elements from the same element on a higher level. If the same attribute or element is present on both levels, the one on the lower level shall take precedence over the one on the higher level.

Several mechanisms are available to specify the Segment Information. Specifically, each Representation shall have assigned exactly one of the following choices to determine the Segment Information, either by direct presence in the Representation element or by inheritance from the higher levels:

- one or more SegmentList elements - for syntax and semantics refer to 5.5.9.3.

- one SegmentTemplate element - for syntax and semantics refer to 5.5.9.4.

- one or more BaseURL elements, at most one SegmentBase element, and no SegmentTemplate or SegmentList element. The SegmentBase element is defined in 5.5.9.2

- All three elements SegmentBase, SegmentTemplate and SegmentList share common elements based on the SegmentBase element. Furthermore, SegmentTemplate and SegmentList share common attributes and elements. The common information is defined in 5.5.9.2.

The derivation and details of Initialisation, Media, Index and Bitstream Switching Segment Information based on the above information is provided in 5.5.9.5.

#### Segment base information

##### Overview

- The SegmentBase element contains information that is sufficient is only a single Media Segment is provided per Representation and the Media Segment URL is included in the BaseURL element.

- In case multiple Media Segments are present, either a SegmentList or a SegmentTemplate is used that share the multiple Segment base information as provided in in 5.5.9.2.3, Table 13.

- If the Representation contains more than one Media Segment, then either the attribute @duration or the element SegmentTimeline shall be present. The attribute @duration and the element SegmentTimeline shall not be present at the same time. Segments described by the Segment base information are referenced by an HTTP-URL conforming to the type URLType as defined in Table 14.

The semantics of the attributes and elements for the SegmentBase element and the Segment base information are provided in 5.5.9.2.2, Table 12 and the multiple Segment base information in Table 13 in 5.5.9.2.2. The XML syntax of the Segment Base Information is provided in 5.5.9.2.3.

##### Semantics

Table 12 — Semantics of SegmentBase element and Segment Base Information type

| Element or Attribute Name |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - SegmentBase - Segment Base Information |  |  | specifies Segment base element as well as the type for the Segment base information. |
|  |  |  |  | @timescale | O | specifies the timescale in units per seconds to be used for the derivation of different real-time duration values in the Segment Information. If not present on any level, it shall be set to 1. NOTE This may be any frequency but typically is the media clock frequency of one of the media streams (or a positive integer multiple thereof). |
|  |  |  |  | @presentationTimeOffset | O | specifies the presentation time offset of the Representation relative to the start of the Period. The value of the presentation time offset in seconds is the division of the value of this attribute and the value of the @timescale attribute. |
|  |  |  |  | @indexRange | O | specifies the byte range that contains the Segment Index in all Media Segments of the Representation. The byte range shall be expressed and formatted as a byte-range-spec as defined in RFC 2616, Clause 14.35.1. It is restricted to a single expression identifying a contiguous range of bytes. |
|  |  |  |  | @indexRangeExact | O | when set to 'true' specifies that for all Segments in the Representation, the data outside the prefix defined by @indexRange contains the data needed to access all access units of all media streams syntactically and semantically. This attribute shall not be present if @indexRange is absent. |
|  |  |  |  | Initialisation | 0 ... 1 | specifies the URL including a possible byte range for the Initialisation Segment. For the type definition refer toTable 14. |
|  |  |  |  | RepresentationIndex | 0 ... 1 | specifies the URL including a possible byte range for the Representation Index Segment. For the type definition refer toTable 14. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |

Table 13 — Semantics of MultipleSegmentBaseInformation type

| Element or Attribute Name |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - MultipleSegmentBaseInformation |  |  | specifies multiple Segment base information. |
|  |  |  |  | @duration | O | If present, specifies the constant approximate Segment duration. All Segments within this Representation element have the same duration unless it is the last Segment within the Period, which could be significantly shorter. The value of the duration in seconds is the division of the value of this attribute and the value of the @timescale attribute associated to the containing Representation. |
|  |  |  |  | @startNumber | O | specifies the number of the first Media Segment in this Representation in the Period. |
|  |  |  |  | Segment Base Information |  | specifies Segment base information. |
|  |  |  |  | SegmentTimeline | 0...1 | specifies the timeline of arbitrary Segment durations For more details see 5.5.9.6. |
|  |  |  |  | BitstreamSwitching | 0 ... 1 | specifies the URL including a possible byte range for the Bitstream Switching Segment. For the type definition refer to Table 14. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |

Table 14 — Semantics of elements of type URLType

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | Element of type URLType |  |  | defines an HTTP-URL |
|  |  | @sourceURL | O | specifies the source URL part and shall be formated either as an <absolute-URI> according to RFC 3986, Clause 4.3, with a fixed scheme of “http://” or "https://" or as a <relative-ref> according to RFC 3986, Clause 4.2. If not present, then any BaseURL element is mapped to the sourceURL attribute and the range attribute shall be present. |
|  |  | @range | O | specifies the byte range restricting the above HTTP-URL. The byte range shall be expressed and formatted as a byte-range-spec as defined in RFC 2616, Clause 14.35.1. It is restricted to a single expression identifying a contiguous range of bytes. If not present, the element refers to the entire resource referenced in the @sourceURL attribute. |

##### XML-Syntax

<!-- Segment information base --> <xs:complexType name="SegmentBaseType"> <xs:sequence> <xs:element name="Initialisation" type="URLType" minOccurs="0"/> <xs:element name="RepresentationIndex" type="URLType" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="timescale" type="xs:unsignedInt"/> <xs:attribute name="presentationTimeOffset" type="xs:unsignedInt"/> <xs:attribute name="indexRange" type="xs:string"/> <xs:attribute name="indexRangeExact" type="xs:boolean"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Multiple Segment information base --> <xs:complexType name="MultipleSegmentBaseType"> <xs:complexContent> <xs:extension base="SegmentBaseType"> <xs:sequence> <xs:element name="SegmentTimeline" type="SegmentTimelineType" minOccurs="0"/> <xs:element name="BitstreamSwitching" type="URLType" minOccurs="0"/> </xs:sequence> <xs:attribute name="duration" type="xs:unsignedInt"/> <xs:attribute name="startNumber" type="xs:unsignedInt"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Segment Info item URL/range --> <xs:complexType name="URLType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="sourceURL" type="xs:anyURI"/> <xs:attribute name="range" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

#### Segment list

##### Overview

- The Segment list is defined by one or more SegmentList elements. Each SegmentList element itself contains a list of SegmentURL elements for a consecutive list of Segment URLs. Each Segment URL may contain the Media Segment URL and possibly a byte range. The Segment URL element may also contain an Index Segment.

The semantics of the attributes and elements for the Segment list are provided in 5.5.9.3.2, Table 15. The XML syntax of the Segment Information is provided in 5.5.9.3.3.

##### Semantics

Table 15 — Semantics of SegmentList element

| Element or Attribute Name |  |  |  |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SegmentList |  |  |  |  |  | specifies Segment information. |
|  |  |  |  |  |  | @xlink:href |  | O | specifies a reference to external SegmentList element |
|  |  |  |  |  |  | @xlink:actuate |  | OD default: "onRequest" | specifies the processing set, can be either "onLoad" or "onRequest" |
|  |  |  |  |  | - MultipleSegmentBaseInformation |  |  |  | Multiple Segment base information as defined in 5.5.9.2. |
|  |  |  |  |  |  | SegmentURL |  | 0 … N | specifies a Media Segment URL and a possibly present Index Segment URL |
|  |  |  |  |  |  |  | @media | O | in combination with the @mediaRange attribute specifies the HTTP-URL for the Media Segment. It shall be formated as an <absolute-URI> according to RFC 3986, Clause 4.3, with a fixed scheme of “http://” or "https://" or as a <relative-ref> according to RFC 3986, Clause 4.2. If not present, then any BaseURL element is mapped to the @media attribute and the range attribute shall be present. |
|  |  |  |  |  |  |  | @mediaRange | O | specifies the byte range within the resource identified by the @media corresponding to the Media Segment. The byte range shall be expressed and formatted as a byte-range-spec as defined in RFC 2616, Clause 14.35.1. It is restricted to a single expression identifying a contiguous range of bytes. If not present, the Media Segment is the entire resource referenced by the @media attribute. |
|  |  |  |  |  |  |  | @index | O | in combination with the @indexRange attribute specifies the HTTP-URL for the Index Segment. It shall be formated as an <absolute-URI> according to RFC 3986, Clause 4.3, with a fixed scheme of “http://” or "https://" or as a <relative-ref> according to RFC 3986, Clause 4.2. If not present and the @indexRange not present either, then no Index Segment information is provided for this Media Segment. If not present and the @indexRange present, then the @media attribute is mapped to the @index. If the @media attribute is not present either, then any BaseURL element is mapped to the @index attribute and the @indexRange attribute shall be present. |
|  |  |  |  |  |  |  | @indexRange | O | specifies the byte range within the resource identified by the @index corresponding to the Index Segment. If @index is not present, it specifies the byte range of the Segment Index in Media Segment. The byte range shall be expressed and formatted as a byte-range-spec as defined in RFC 2616, Clause 14.35.1. It is restricted to a single expression identifying a contiguous range of bytes. If not present, the Index Segment is the entire resource referenced by the @index attribute. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Note that the conditions only holds without using @xlink:href. If linking is used, then all attributes are "optional" and <minOccurs=0> Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |  |  |  |

##### XML-Syntax

<!-- Segment List --> <xs:complexType name="SegmentListType"> <xs:complexContent> <xs:extension base="MultipleSegmentBaseType"> <xs:sequence> <xs:element name="SegmentURL" type="SegmentURLType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Segment URL --> <xs:complexType name="SegmentURLType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="media" type="xs:anyURI"/> <xs:attribute name="mediaRange" type="xs:string"/> <xs:attribute name="index" type="xs:anyURI"/> <xs:attribute name="indexRange" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

#### Segment template

##### Overview

- The Segment template is defined by the SegmentTemplate element. In this case, specific identifiers that are substituted by dynamic values assigned to Segments, to create a list of Segments. The substitution rules are provided in 5.5.9.4.4.

The semantics of the attributes and elements for the Segment list are provided in 5.5.9.4.2, Table 16. The XML syntax of the Segment Information is provided in 5.5.9.4.3.

##### Semantics

Table 16 — Semantics of SegmentTemplate element

| Element or Attribute Name |  |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SegmentTemplate |  |  |  | specifies Segment template information. |
|  |  |  |  |  | MultipleSegmentBaseInformation |  | Provides the Multiple Segment base information as defined in 5.5.9.2. |
|  |  |  |  |  | @media | O | specifies the template to create the Media Segment List. |
|  |  |  |  |  | @index | O | specifies the template to create the Index Segment List. If neither the $Number$ nor the $Time$ identifier is included, this provides the URL to a Representation Index. |
|  |  |  |  |  | @initialisation | O | specifies the template to create the Initialisation Segment. Neither $Number$ nor the $Time$ identifier shall be included. |
|  |  |  |  |  | @bitstreamSwitching | O | specifies the template to create the Bitstream Switching Segment. Neither $Number$ nor the $Time$ identifier shall be included. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |  |

##### XML syntax

<!-- Segment Template --> <xs:complexType name="SegmentTemplateType"> <xs:complexContent> <xs:extension base="MultipleSegmentBaseType"> <xs:attribute name="media" type="xs:string"/> <xs:attribute name="index" type="xs:string"/> <xs:attribute name="initialisation" type="xs:string"/> <xs:attribute name="bitstreamSwitching" type="xs:string"/> </xs:extension> </xs:complexContent> </xs:complexType>

##### Template-based Segment URL construction

The SegmentTemplate@media attribute, the SegmentTemplate@index, the The SegmentTemplate@initialisation attributeand the SegmentTemplate@bitstreamSwitching attribute each contain a string that may contain one or more of the identifiers as listed in Table 17.

In each URL, the identifiers from Table 17 shall be replaced by the substitution parameter defined in Table 17. Identifier matching is case-sensitive. If the URL contains unescaped $ symbols which do not enclose a valid identifier then the result of URL formation is undefined. In this case it is expected that the DASH Client ignores the entire containing Representation element and the processing of the MPD continues as if this Representation element was not present. The format of the identifier is also specified in Table 17.

Each identfier may be suffixed, within the enclosing ‘$’ characters, with an additional format tag aligned with the printf format tag as defined in IEEE 1003.1-2008 [10] following this prototype:

%0[width]d

The width parameter is an unsigned integer that provides the minimum number of characters to be printed. If the value to be printed is shorter than this number, the result shall be padded with zeros. The value is not truncated even if the result is larger.

The Media Presentation shall be authored such that the application of the substitution process results in valid Segment URLs.

Strings outside identifiers shall only contain characters are permitted within URLs according to RFC 1738.

Table 17 — Identifiers for URL templates

| $<Identifier>$ | Substitution parameter | Format |
| --- | --- | --- |
| $$ | Is an escape sequence, i.e. "$$" is replaced with a single "$" | not applicable |
| $RepresentationID$ | This identifier is substituted with the value of the attribute Representation@id of the containing Representation. | The format tag shall not be present. |
| $Number$ | This identifier is substituted with the number of the corresponding Segment. For an example DASH Client using this identifier to construct the list Media Segment URLs, refer to B.3. | The format tag may be present. If no format tag is present, a default format tag with width=1 shall be used. |
| $Bandwidth$ | This identifier is substituted with the value of Representation@bandwidth attribute value. | The format tag may be present. If no format tag is present, a default format tag with width=1 shall be used. |
| $Time$ | This identifier is substituted with the value of the SegmentTimeline@t attribute for the Segment being accessed. Either $Number$ or $Time$ may be used but not both at the same time. | The format tag may be present. If no format tag is present, a default format tag with width=1 shall be used. |

#### Segment information

##### Overview

The Segment Information provides the following information:

- the presence or absence of Initialisation, Index and Bitstream Switching Segment information

- the HTTP-URL and possibly a byte range for each accessible Segment in each Representation,

- all valid Segment URLs declared by the containing MPD,

- for services with MPD@type='dynamic', the Segment availability start time and Segment availability end time of each Segment,

- an approximate Media Presentation start time of each Media Segment in the Media Presentation timeline within the Period.

The derivation of Initialisation, Media, Index amd Bitstream Switching Segment Information from the elements SegmentBase, SegmentList and SegmentTemplate is provided in what follows. Reference resolution as defined in 5.6.4 and base URL selection as defined in 5.6.5 using BaseURL elements as defined in 5.6 shall be applied to any URLs.

##### Initialisation Segment information

Each Representation has assigned at most one Initialisation Segment.

The presence of an Initialisation Segment is indicated by the presence of SegmentBase.Initialisation, SegmentList.Initialisation, the SegmentTemplate.Initialisation element or the SegmentTemplate@initialisation attribute that may contain URL and byte range information or URL construction rules for the Initialisation Segment.

If neither Initialisation element nor SegmentTemplate@initialisation attribute are present for a Representation then each Media Segment within the Representation shall be self-initialising.

For services with MPD@type='dynamic', the Segment availability start time of the Initialisation Segment is the sum of the value of the MPD@availabilityStartTime and the PeriodStart time as defined in 5.5.2.1 of the containing Period and the Segment availability end time of the Initialisation Segment is the largest Segment availability end time of any Media Segment in this Representation. For Segment availability for Media Segments refer to 5.5.9.5.3.

The data structures retrieved from the Initialisation URL are defined in 6.2.2.

##### Media Segment information

Each Representation has assigned a list of consecutive Media Segments. Each entry in the list of a Media Segment has assigned the following parameters:

- A valid Media Segment URL and possibly a byte range,

- the number of the Media Segment in the Representation,

- the MPD start time of the Media Segment in the Representation providing an approximate presentation start time of the Segment,

- MPD duration of the Media Segment providing an approximate presentation duration of the Segment.

- These parameters are specified by the SegmentTemplate or SegmentList elements. To obtain at least one entry in the list of Media Segments, one of the following shall apply:

- if SegmentTemplate element is present the Template-based Segment URL construction in 5.5.9.4.4 shall be applied with the number of the Media Segment in the Media Segment list. The first number in the list is determined by the value of the SegmentTemplate@startNumber attribute, if present, or is 1 in case this attribute is not present.

- if one or more SegmentList elements are present they contain itself a list of SegmentURL elements for a consecutive list of Media Segment URLs. The first number in the list is determined by the value of the SegmentList@startNumber attribute, if present, or is 1 in case this attribute is not present. The sequence of multiple SegmentList elements within a Representation shall result in Media Segment List with consecutive numbers.

- none of the above: In this case only a single Media Segment shall be present with the URL provided by a BaseURL element and the SegmentBase element may be present.

The MPD start time is relative to the start of the Representation provided by the MPD. The MPD start time and the MPD duration are approximate and do not reflect the exact Media Presentation time. For more details on the relation of MPD start times and Media Presentation time refer to 7.2.1.

For the derivation of the MPD start time and duration of each Media Segment in the list of Media Segments, the Number of the Media Segment and the following information is used.

- If neither @duration attribute nor SegmentTimeline element is present, then the Representation shall contain exactly one Media Segment. The MPD start time is 0 and the MPD duration is obtained as for the last Media Segment in the Representation (see below for more details).

- If @duration attribute is present, then the MPD start time of the Media Segment is determined as (Number-NumberStart-1) times the value of the attribute @duration with NumberStart the value of the @startNumber attribute. The MPD duration of the Media Segment is the value of the attribute @duration unless the Media Segment is the last one the Representation (see below for more details).

- If @duration attribute is not present and the SegmentTimeline element is present then rules in 5.5.9.6 apply to determine the start time and duration of each Media Segment in the Media Segment list.

- To determine the duration of the only or the last Media Segment of any Representation in a Period, the MPD shall include sufficient information to determine the duration of the containing Period. For example, the MPD@mediaPresentationDuration, or Period@duration, or next Period@start may be present.

For services with MPD@type='dynamic', the Segment availability start time of a Media Segment is the sum of the value of the MPD@availabilityStartTime, the PeriodStart time of the containing Period as defined in 5.5.2.1 and the MPD start time of the Media Segment. The Segment availability end time of a Media Segment is the sum of the Segment availability start time, the MPD duration of the Media Segment and the value of the attribute MPD@timeShiftBufferDepth.

The MPD shall include URL information for all Segments with an availability start time less than both (i) the end of the Media Presentation and (ii) the sum of the latest time at which this version of the MPD is available on the server and the value of the MPD@minimumUpdatePeriod.

The data structures retrieved from the URL referring to a Media Segment are defined in 6.2.3.

##### Index Segment information

Each Segment typically has assigned Segment Index information that may be provided in an explicitly declared Index Segment.

The presence of explicit Index Segment information is indicated

  - by the presence of one RepresentationIndex element providing the Segment Index for the entire Representation, or

  - by the presence of at least one of the two attributes @index and @indexRange in the SegmentList.SegmentURL element, or

  - by the presence of SegmentTemplate@index attribute. If either $Number$ or $Time$ are present the Template-based Segment URL construction in 5.5.9.4.4 shall be applied with number set to the number of the corresponding Media Segment. If not present, the SegmentTemplate@index attribute constitutes a reference to Representation Index.

The @indexRange attribute may also be used to provide the byte range for an index within a Media Segment, where this is allowed by the Media Segment format. In this case the @index attribute shall not be present and the range specified shall lie completely within any byte range specified for the Media Segment.

The availability of Index Segments is identical to the availability to the Media Segments they correspond to.

The data structures retrieved from the URL referring to an Index Segment are defined in 6.2.4.

##### Bitstream Switching Segment information

Each Representation has assigned at most one Bitstream Switching Segment. The Bitstream Switching Segment is only relevant in case the @bitstreamSwitching flag is set to 'true' and may enable the creation of a conforming Segment sequence for segments from different Representations.

The presence of a Bitstream Switching Segment is indicated by the presence of the BitstreamSwitching element or the SegmentTemplate@bitstreamSwitching attribute that that may contain URL and byte range information or construction rules for the URL.

If neither BitstreamSwitching element nor SegmentTemplate@bitstreamSwitching attribute are present for a Representation and the @bitstreamSwitching flag is set to 'true', there are no Bitstream Switching Segments.

The Segment availability time of the Bitstream Switching Segment is identical to the one specified for the Initialisation Segment in 5.5.9.5.2.

The data structures retrieved from the URL referring to a Bitstream Switching Segment are defined in 6.2.5.

#### Segment timeline

##### General

The SegmentTimeline element expresses the earliest presentation time and presentation duration (in units based on the @timescale attribute) for each Segment in the Representation. The use is an alternative to providing the @duration attribute and provides three additional features:

- the specification of arbitrary Segment durations,

- the specification of accurate Segment durations for one media stream where the duration expresses presentation duration of the Segment, and

- the signalling of discontinuities of the Media Presentation timeline for which no Segment data is present in a specific Representation.

For compactness the syntax of this element includes run-length compression to express a sequence of Segments having constant duration.

The SegmentTimeline element shall contain a list of S elements each of which describes a sequence of contiguous segments of identical MPD duration. The S element contains a mandatory @d attribute specifying the MPD duration, an optional @r repeat count attribute specifying the number of contiguous Segments with identical MPD duration minus one and an an optional @t time attribute specifying the MPD start time of the first Segment in the series.

The @r attribute has a default value of zero (i.e., a single Segment in the series) when not present. For example, a repeat count of three means there are four contiguous Segments, each with the same MPD duration.

Any @d value shall not exceed the value of MPD@maxSegmentDuration.

The textual order of the S elements within the SegmentTimeline element shall match the numbering (and thus time) order of the corresponding Media Segments.

When the SegmentTemplate is in use and the $Time$ identifier is present in the SegmentTemplate@media then

- at least one Segment Index (‘sidx’) box shall be present

- the values of the SegmentTimeline shall describe accurate timing of each Media Segment. Specifically, these values shall reflect the information provided in the Segment index (‘sidx’) box, i.e.

  - the value of @timescale shall be identical to the value of the timescale field in the first ‘sidx’ box,

  - the value of S@t shall be identical to the value of the earliest_presentation_time in the first ‘sidx’ box of the Media Segment described in S.

  - the value of S@d shall be identical to sum of the the values of all Subsegment_duration fields in the first ‘sidx’ box of the Media Segment described in S.

- The Segment URL for a Media Segment is obtained by replacing the $Time$ identifier by the earliest presentation time obtained from the SegmentTimeline.

NOTE	As the earliest presentation time of the next Media Segment in the same Representation may be derived from the actual Media Segment, e.g. by the use of the Segment Index, the Segment URL may be generated without reading of the updated MPD that contains the update to the Segment Timeline.

The semantics of the attributes and elements for Segment Timeline are provided in 5.5.9.6.2, Table 18. The XML syntax of the Segment Timeline is provided in 5.5.9.6.3.

##### Semantics

Table 18 — Semantics of SegmentTimeline element

| Element or Attribute Name |  |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SegmentTimeline |  |  |  | specifies the Segment timeline information |
|  |  |  |  | S |  | 1 .. N | specifies Segment start time and duration for a contigous sequence of segments of identical durations. The textual order of the S elements must match the indexed (and thus time) order of the corresponding Media Segments. |
|  |  |  |  |  | @t | O | specifies the MPD start time, in @timescale units, the first Segment in the series starts relative to the beginning of the Period. The value of this attribute must be equal to or greater than the sum of the previous S element earliest presentation time and the sum of the contiguous Segment durations. If the value of the attribute is greater than what is expressed by the previous S element, it expresses discontinuities in the timeline. If not present then the value shall be assumed to be zero for the first S element and for the subsequent S elements, the value shall be assumed to be the sum of the previous S element's earliest presentation time and contiguous duration (i.e. previous S@t + @d * (@r + 1)). |
|  |  |  |  |  | @d | M | specifies the Segment duration, in units of the value of the @timescale. |
|  |  |  |  |  | @r | OD default: 0 | specifies the repeat count of the number of following contiguous Segments with the same duration expressed by the value of @d. This value is zero-based (e.g. a value of three means four Segments in the contiguous series). |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |  |

##### XML syntax

<!-- Segment Timeline --> <xs:complexType name="SegmentTimelineType"> <xs:sequence> <xs:element name="S" minOccurs="1" maxOccurs="unbounded" > <xs:complexType> <xs:attribute name="t" type="xs:unsignedInt"/> <xs:attribute name="d" type="xs:unsignedInt" use="required"/> <xs:attribute name="r" type="xs:unsignedInt" use="optional" default="0"/> </xs:complexType> </xs:element> </xs:sequence> </xs:complexType>

## Base URL Processing

### Overview

The BaseURL element may be used to specify one or more common locations for Segments and other resources. Reference resolution as defined in 5.6.4 shall be applied to each URL in the MPD. Handling of multiple alternative base URLs is addressed in 5.6.5.

The semantics of the attributes and elements for the Base URL are provided in 5.6.2, Table 19. The XML syntax of the Base URL is provided in 5.6.3.

### Semantics

Table 19 — Semantics of BaseURL element

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | BaseURL |  |  | A URL that can be used as Base URL. The content of this element is a URI string as described in 5.6.4. |
|  |  | @serviceLocation | O | This attribute specifies a relationship between BaseURLs such that BaseURL elements with the same @serviceLocation value are likely to have their URLs resolve to services at a common network location, for example a common Content Delivery Network. |
|  |  | @byteRange | O | if present specifies HTTP partial GET requests may alternatively be issued by adding the byte range into a regular HTTP-URL based on the value of this attribute and the construction rules in Annex E.2. NOTE Such alternative requests are expected to not be used unless the DASH application requires this. For more details refer to Annex E. |

### XML syntax

<!-- Base URL --> <xs:complexType name="BaseURLType"> <xs:simpleContent> <xs:extension base="xs:anyURI"> <xs:attribute name="serviceLocation" type="xs:string"/> <xs:attribute name="byteRange" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:extension> </xs:simpleContent> </xs:complexType>

### Reference resolution

URLs at each level of the MPD are resolved according to RFC3986 with respect to the BaseURL element specified at that level of the document or the level above in the case of resolving base URLs themselves (the document “base URI” as defined in RFC 3986 Section 5.1 is considered to be the level above the MPD level). If only relative URLs are specified and the document base URI cannot be established according to RFC3986 then the MPD should not be interpreted. URL resolution applies to all URLs found in MPD documents.

In addition to the document level (the level above the MPD level), base URL information may be present on the following levels:

- On MPD level in MPD.BaseURL element. For details refer to 5.5.1.2.

- On Period level in Period.BaseURL element. For details refer to 5.5.2.2.

- On Adaptation Set level in AdaptationSet.BaseURL element. For details refer to 5.5.3.2.

- On Representation level in Representation.BaseURL. For details refer to 5.5.5.2.

### Alternative base URLs

If alternative base URLs are provided through the BaseURL element at any level, identical Segments shall be accessible at multiple locations. In the absence of other criteria, the DASH Client may use the first BaseURL element as “base URI". The DASH Client may use base URLs provided in the BaseURL element as “base URI” and may implement any suitable algorithm to determine which URLs it uses for requests.

## Program information

### Overview

Descriptive information on the program may be provided for a Media Presentation within the ProgramInformation element.

When multiple ProgramInformation elements are present, the @lang attribute shall be present and each element shall describe the Media Presentation sufficiently in the language defined by the value of the @lang attribute.

For each language, the program information may specify title, source of the program, copyright information, and a URL to more information.

The semantics of the attributes within the ProgramInformation element are provided in Table 20 of 5.7.2. The XML syntax of ProgramInformation element is provided in 5.7.3.

### Semantics

Table 20 — Program information semantics

| Element or Attribute Name |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | ProgramInformation |  |  | specifies descriptive information about the program |
|  |  | @lang | O | Declares the language code(s) for this Program Information. The syntax and semantics according to IETF RFC 5646 shall be applied. |
|  |  | @moreInformationURL | O | If provided, this attribute specifies an absolute URL which provides more information about the Media Presentation. |
|  |  | Title | 0 ... 1 | specifies the title for the Media Presentation |
|  |  | Source | 0 ... 1 | specifies information about the original source (for example content provider) of the Media Presentation. |
|  |  | Copyright | 0 ... 1 | specifies a copyright statement for the Media Presentation, usually starting with the copyright symbol, unicode U+00A9 |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |

### XML syntax

<!-- Program Information --> <xs:complexType name="ProgramInformationType"> <xs:sequence> <xs:element name="Title" type="xs:string" minOccurs="0"/> <xs:element name="Source" type="xs:string" minOccurs="0"/> <xs:element name="Copyright" type="xs:string" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="moreInformationURL" type="xs:anyURI"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

## Descriptors

### General

The MPD may contain descriptors that make use of a common syntax as defined in this subclause. The elements of type DescriptorType provide a flexible mechanism for DASH content authors to annotate and extend the MPD, Period, AdaptationSet and Representation elements.

The descriptor elements are all structured in the same way, namely they contain a @schemeIdUri attribute that provides a URI to identify the scheme and an optional attribute @value attribute. The semantics of the element are specific to the scheme employed. The URI identifying the scheme may be a URN or a URL.

In this part of ISO/IEC 23009, specific elements for descriptors are defined in 5.8.4.

The MPD does not provide any specific information on how to use these elements. It is up to the application that employs DASH formats to instantiate the description elements with appropriate scheme information. However, this part of ISO/IEC 23009 defines some specific schemes in 5.8.5.

DASH applications that use one of these elements must first define a Scheme Identifier in the form of a URI and must then define the value space for the element when that Scheme Identifier is used. The Scheme Identifier appears in the @schemeIdUri attribute.

In the case that a simple set of enumerated values are required, a text string may be defined for each value and this string must be included in the @value attribute. If structured data is required then any element/attribute extensibility in a separate namespace.

Two elements of type DescriptorType are equivalent, if the element name, the value of the @schemeIdUri and the value of the @value attribute are equivalent. If the @schemeIdUri is a URN, then equivalence shall refer to lexical equivalence as defined in clause 5 of RFC 2141. If the @schemeIdUri is a URL, then equivalence shall refer to equality on a character-for-character basis as defined in clause 6.2.1 of RFC3986. If the @value attribute is not present, equivalence is determined by the equivalence for @schemeIdUri only. Attributes and element in extension namespaces are not used for determining equivalence.

The semantics of the attributes within an element of the type DescriptorType are provided in Table 21 of 5.8.2. The XML schema definition of DescriptorType is provided in 5.8.3. The specific descriptors follow these syntax and semantics.

### Semantics of generic descriptor

Table 21 — Semantics of elements of type DescriptorType

| Element or Attribute Name |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Element of type DescriptorType |  |  | specifies a descriptor. |
|  |  |  |  | @schemeIdUri | M | specifies a URI to identify the scheme. The semantics of this element are specific to the scheme specified by this attribute. The @schemeIdUri may be a URN or URL. When a URL is used, it should also contain a month-date in the form mmyyyy; the assignment of the URL must have been authorized by the owner of the domain name in that URL on or very close to that date, to avoid problems when domain names change ownership. |
|  |  |  |  | @value | O | specifies the value for the descriptor element. The value space and semantics must be defined by the owners of the scheme identified in the @schemeIdUri attribute. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |

### XML syntax of generic descriptor

<!-- Descriptor --> <xs:complexType name="DescriptorType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="schemeIdUri" type="xs:anyURI" use="required"/> <xs:attribute name="value" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

### Specific descriptors

#### Content protection

For the element ContentProtection the @schemeIdUri attribute is used to identify a content protection scheme. This attribute should provide sufficient information, possibly in conjunction with the @value and/or extension attributes and elements, such as the DRM system(s), encryption algorithm(s), and key distribution scheme(s) employed, to enable a client to determine whether it can possibly play the protected content. The ContentProtection element can be extended in a separate namespace to provide information specific to the content protection scheme (e.g., particular key management systems or encryption methods).

When the ContentProtection element is not present the content shall not be content-protected.

When multiple ContentProtection elements are present, each element shall describe a content protection scheme that is sufficient to access and present the Representation.

#### Role

For the element Role the @schemeIdUri attribute is used to identify the role scheme employed to identify the role of the media content component. Roles define and describe characteristics and/or structural functions of media content components.

One Adaptation Set or one media content component may have assigned multiple roles even within the same scheme.

This part of ISO/IEC 23009 defines a simple role scheme in 5.8.5.5.

In addition, this part of ISO/IEC 23009 defines other roles schemes to support signalling for multiple view signals in 5.8.5.6.

#### Accessibility

For the element Accessibility the @schemeIdUri attribute is used to identify the accessibility scheme employed. Accessibility is a general term used to describe the degree to which the DASH Media Presentation is available to as many people as possible.

NOTE	Accessibility elements fulfil a very similar purpose with respect to media content components as for Role elements, but are specifically intended for accessibility.

- One Adaptation Set or one media content component may have assigned multiple accessibility purposes even within the same scheme.

This part of ISO/IEC 23009 does not define a specific accessibility scheme, but the simple role scheme in 5.8.5.5 may be used to express a minimum amount of accessibility information.

#### Rating

- For the element Rating the @schemeIdUri attribute is used to identify the rating scheme employed.

- Ratings specify that content is suitable for presentation to audiences for which that rating is known to be appropriate, or for unrestricted audiences.

NOTE	if an audience has a rating restriction it is intended that content that has associated ratings should not be presented to that audience, unless at least one scheme is recognized and the rating value indicates that the content is appropriate to that audience.

- This part of ISO/IEC 23009 does not define a rating scheme.

#### Viewpoint

- For the element Viewpoint the @schemeIdUri attribute is used to identify the viewpoint scheme employed.

- Adaptation Sets containing non-equivalent Viewpoint element values contain different media content components. The Viewpoint elements may equally be applied to media content types that are not video.

Adaptation Sets with equivalent Viewpoint element values are intended to be presented together. This handling should be applied equally for recognised and unrecognised @schemeIdUri values.

This part of ISO/IEC 23009 does not define a viewpoint scheme.

#### Frame-packing

- For the element FramePacking the @schemeIdUri attribute is used to identify the frame-packing configuration scheme employed.

Multiple FramePacking elements may be present. If so, each element shall contain sufficient information to select or reject the described Representations.

NOTE	if the scheme or the value for all FramePacking elements are not recognized the DASH client is expected to ignore the described Representations.

This part of ISO/IEC 23009 defines frame-packingframe-packing schemes in 5.8.5.6.

#### Audio channel configuration

- For the element AudioChannelConfiguration the @schemeIdUri attribute is used to identify the audio channel configuration scheme employed.

Multiple AudioChannelConfiguration elements may be present indicating that the Representation supports multiple audio channel configurations. For example, it may describe a Representation that includes MPEG Surround audio supporting stereo and multichannel.

NOTE	if the scheme or the value for this descriptor is not recognized the DASH client is expected to ignore the descriptor.

A scheme for audio channel configuration is defined in 5.8.5.4 of this part of ISO/IEC 23009.

### Specific scheme definitions

#### General

The definition of specific schemes (both syntax and semantics) to be used in any of the descriptor elements requires the definition of the URI by the authors to link the content description to the Media Presentation. This clause defines some schemes and scheme identifiers to enable usage of existing code points in combination with this part of ISO/IEC 23009 as well as to provide simple means to support different functionalities.

#### Content protection

The following defines a set of URIs that identify specific content protection schemes, i.e. schemes contained in the ContentProtection element:

- For Representations based on ISO/IEC14496-12, the following URI is defined to indicate protection schemes identified by a the Scheme Type within the Scheme Type Box of the Protection Scheme Information Box of the file:

urn:mpeg:dash:mp4protection:2011

In this scheme, the value of the @value attribute shall be the 4CC contained in the Scheme Type Box, suitably escaped according to RFC 2141.

- For Representations based on ISO/IEC 13818-1 (MPEG-2 Transport Stream), the following URI are is defined to indicate the Conditional Access System used:

urn:mpeg:dash:13818:1:CA_descriptor:2011

In this scheme, the value of the @value attribute shall be the 4-digit lower-case hexadecimal Representation of the 16-bit CA_system_ID from the CA_descriptor as defined in ISO/IEC 13818-1.

- For Representations based on ISO/IEC14496-12 a content protection scheme using the Protection System Specific Header Box defined in ISO/IEC 23001-7 may be identified in the ContentProtection element. In this case a UUID URN as defined in RFC 4122 indicating the UUID specified in the SystemId field of the Protection System Specific Header Box shall be used. This does not imply that such schemes cannot define alternative URNs, or that all UUID URNs refer to schemes of this type.

#### Frame-packing

The following defines a set of URIs that identify specific frame-packing arrangments, i.e. schemes contained in the FramePacking element:

- For Adaptation Sets or Representations that contain a video component that conforms to ISO/IEC 14496-10, the URI urn:mpeg:dash:14496:10:frame_packing_arrangement_type:2011 is defined to indicate the frame-packing arrangement as defined by Table D-8 of ISO/IEC 14496-10 (‘Definition of frame_packing_arrangement_type’). The @value shall be the ‘Value’ column as specified in Table D-8 and shall be interpreted according to the ‘Interpretation’ column in the same table.

- For Adaptation Sets or Representations that contain a video component that conforms to ISO/IEC 13818-1, the URI urn:mpeg:dash:13818:1:stereo_video_format_type:2011 is defined to indicate the frame-packing arrangement as defined by Table L-1 of ISO/IEC 13818-1 (‘Definition of stereo_video_format_type’). The @value shall be the ‘stereo_video_format_type’ column as specified in Table L-1 and shall be interpreted according to the ‘Meaning’ column in the same table.

#### Audio channel configuration scheme

The following defines a URI that identifies channel configuration signalling for Representations that contain an audio component the URI "urn:mpeg:dash:23002:3:audio_channel_configuration:2011" is defined to indicate the channel configuration as defined by Table 65 (Channel Configurations, meaning of channelConfigurationIndex, mapping of channel elements to loudspeaker positions') of ISO/IEC 23002-3. The @value shall be the ‘value’ column as specified in Table 65 and shall be interpreted according to the remaining columns in the same table.

#### DASH role scheme

The URN "urn:mpeg:dash:role:2011" is defined to identify the role scheme defined in Table 22. Note that Role@value shall be assigned to Adaptation Sets that contain a media component type to which this role is associated.

- Table 22 — Role@value attríbute for scheme with a value "urn:mpeg:dash:role:2011"

| - Role@value | - Description |
| --- | --- |
| - caption | - captions (see note 3 below) |
| - subtitle | - subtitles (see note 3 below) |
| - main | - main media component(s) which is/are intended for presentation if no other information is provided |
| - alternate | - media content component(s) that is/are an alternative to (a) main media content component(s) of the same media component type (see note 2 below) |
| - supplementary | - media content component that is supplementary to a media content component of a different media component type (see Note 1 below) |
| - commentary | - media content component with commentary (e.g. director’s commentary) (typically audio) |
| dub | - media content component which is presented in a different language from the original (e.g. dubbed audio, translated captions) |
| NOTES 1) A normal audio/video program labels both the primary audio and video as "main". However, when the two media component types are not equally important, for example (a) video providing a pleasant visual experience to accompany a music track that is the primary content or (b) ambient audio accompanying a video showing a live scene such as a sports event, that is the primary content, the accompanying media may be assigned a "supplementary" role. 2) alternate media content components should carry other descriptors to indicate in what way it differs from the main media content components (e.g. a Viewpoint descriptor or a Role descriptor), especially when multiple alternate media content components including multiple supplementary media content components are available. 3) open (“burned in”) captions or subtitles would be marked as media type component "video" only, but having a descriptor saying “caption” or “subtitle”; |  |

#### DASH Multiple views scheme

A scheme is defined for multiple views media content description.

This scheme may be used with the Role descriptor applied to a ContentComponent element of type video or to an AdaptationSet element. If this scheme is applied to an AdaptationSet element containing multiple views, each media content component of type video shall have a ContentComponent element that contains the Role descriptor using this scheme. A Role element of this scheme is used to indicate which views amongst the views comprising multiple presentable stereo pairs the contained media content component or components represent. If N views are available that can be combined into M valid stereo pairs, the Role with @schemeIdURI equal to "urn:mpeg:dash:stereoid:2011" signals which views form a stereo pair and which one is the left view and which one is the right view of each stereo pair. The @value of the Role element shall contain a space-delimited list of view indicators 'li' or ‘rj’ where i, j are non-negative decimal integers. A stereo pair i (0 <= i < M) is formed by using a view whose Role element includes the view indicator 'li' as the left view and a view whose Role element contains and the view indicator ‘ri’ as the right view. Within the @value attribute view indicators shall be ordered with all left view indictors preceding all right view indicators and within each group in ascending order of view index.

## DASH metrics descriptor

### Overview

This part of ISO/IEC 23009 does not define mechanisms for reporting metrics, however it does define a set of metrics and a mechanism that may be used by the service provider to trigger metric collection and reporting at the clients, should a reporting mechanism be available. The trigger mechanism is based on the Metrics element in the MPD. The element contains the list of DASH Metrics for which the measurements are desired, the time interval and the granularity for the measurements, as well as the scheme according to which the metric reporting is desired.

The semantics of the attributes within the Metrics element are provided in Table 23 of 5.9.2. The XML syntax of Metrics element is provided in 5.9.3.

The semantics of the Reporting element are provided in 5.9.4.

### Semantics

Table 23 — Semantics of Metrics element

| Element or Attribute Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Metrics |  |  | DASH metric element |
|  |  |  | @metrics | M | specifies all DASH Metrics (as a list of DASH Metric keys as defined in Annex D, separated by a comma) that the client is desired to report. |
|  |  |  | Range | 0 ... N | specifies the time period during which DASH Metrics collection is requested. When not present, DASH Metrics reporting is requested for the whole duration of the content. |
|  |  |  | @starttime | O | specifies the start time of the DASH Metrics collection operation. When not present, DASH Metrics collection is requested from the beginning of content consumption. For services with MPD@type='dynamic'sessions, the start time is indicated in wallclock time by adding the value of this attribute to the value of the MPD@availabilityStartTime attribute. For services with MPD@type='static', the start time is indicated in Media Presentation time and is relative to the PeriodStart time of the first Period in this MPD. |
|  |  |  | @duration | O | specifies the duration of the DASH metrics collection interval. The value of the attribute expresses in Media Presentation time. |
|  |  |  | Reporting | 1 ... N | specifies information about the requested reporting method and formats. |

### XML syntax

<!-- Metrics --> <xs:complexType name="MetricsType"> <xs:sequence> <xs:element name="Reporting" type="DescriptorType" maxOccurs="unbounded"/> <xs:element name="Range" type="RangeType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="metrics" type="xs:string" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Metrics Range --> <xs:complexType name="RangeType"> <xs:attribute name="starttime" type="xs:duration"/> <xs:attribute name="duration" type="xs:duration"/> </xs:complexType>

### Metric reporting

DASH clients should collect metrics based on the Metric element and report the collected metrics using one of the reporting schemes in the Reporting descriptor in the Metrics element.

It is expected that elements containing unrecognised reporting schemes are ignored by the DASH client.

If multiple Reporting elements are present, it is expected that the client processes one of the recognised reporting schemes.

No reporting scheme is specified in this part of ISO/IEC 23009. It is expected that external specifications may define formats and delivery for the reporting data. External specifications defining a reporting scheme should take specific care to respect privacy issues.

# Segment formats

## Introduction

The Segment formats specify the formats of the entity body of the HTTP response when issuing a HTTP GET request or a HTTP partial GET with the indicated byte range through HTTP/1.1 as defined in RFC 2616 to a resource identified in the MPD. Segments typically contain efficiently coded media and metadata according to or aligned with common media formats.

Different Segment types are defined in 6.2.

- This part of ISO/IEC 23009 focuses on Segment formats based on MPEG container formats. Specifically,

- in 6.3, Segment formats are described for use with Media Segments based on the ISO Base Media File Format as defined in ISO/IEC 14496-12;

- In 6.4, Segment formats are described for use with Media Segments based on the MPEG-2 Transport Stream as defined in the ISO/IEC 13818-2;

- In both cases the Segment formats are defined such that the Media Segment formats comply with the respective container formats.

- Guidelines for adding other Segment formats are provided in Annex F.

## Segment types

### Introduction

Four different Segment types are defined:

- Initialisation Segments containing initialisation information for accessing the Representation in 6.2.2,

- Media Segments containing encoded media content components in 6.2.3,

- Index Segments primarily containing indexing information for Media Segments in 6.2.4,

- Bitstream Switching Segments containing essential data to switch to the Representation to which it is assigned in 6.2.5.

### Initialisation Segment

The Initialisation Segment contains initialisation information for accessing the Representation. The Initialisation Segment shall not contain any media data with an assigned presentation time.

NOTE	The Initialisation Segment is conceptually processed by the media engine in Figure 2 to initialise the media engines for enabling play-out of Media Segments of the containing Representation.

The Initialisation Segment is media format specific and more details shall be defined for each media format that permits the presence of an Initialisation Segment.

### Media Segment

#### General

A Media Segment contains and encapsulates media streams that are either described within this Media Segment or described by the Initialisation Segment of this Representation or both.

In addition, a Media Segment

  - shall contain a number of complete access units.

  - should contain at least one Stream access point (SAP) for each contained media stream.

  - should provide information how to access the Media Presentation within this Segment, e.g. exact presentation time and an index. There is no requirement that a Media Segment starts with a SAP, but it is possible to signal in the MPD that all media streams in a Segments within a Representation start with a SAP.

  - if it is the first Media Segment in the Representation, it shall contain only media streams that start with a SAP of type 1 or 2.

  - shall contain sufficient information to time-accurately present each contained media component in the Representation without accessing any previous Media Segment in this Representation provided that the Media Segment contains a SAP for each media stream. The time-accuracy enables a client to seamlessly switch Representations and jointly present multiple Representations.

  - may be divided into Subsegments by a Segment Index as defined in 6.2.3.2. In some media formats the Segment Index may be contained in the Media Segment. In other formats the Segment Index may be included in a dedicated Index Segment. For more details on Index Segments refer to 6.2.4.

  - shall specify all Media Presentation times relative to the start of the Period and compensated with the value of the @presentationTimeOffset. The presentation time in Media Segments shall be accurate to ensure accurate alignment of all Representations in one Period. For more details refer to 7.2.1.

- The Media Segment is media format specific and more details are typically specified for individual media formats.

#### Subsegments and Segment Index

Media Segments may contain multiple Subsegments. Each Subsegment shall contain a number of complete access units. There may also be media-format-specific restrictions on Subsegment boundaries. If a Segment is divided into multiple Subsegments this division is described by a compact Segment index, which provides the presentation time range in the Representation and corresponding byte range in the Segment occupied by each Subsegment for one or more media streams. Clients may download this index in advance and then issue requests for individual Subsegments.

NOTE	Segment Index information is conceptually processed by the DASH access client in Figure 2 in order to access Subegments by the use of HTTP partial GET requests.

- In addition, the Segment Index provides timing and stream access information. This includes the earliest presentation time of access units in each Subsegment of an indexed media stream and the presentation time of the first SAP, if present.

- If a Segment Index is present for at least one media stream, then for any media stream for which no Segment Index is present, referred to as non-indexed stream, the following applies:

- every access unit of the non-indexed streams shall be a SAP of type 1.

- for each Subsegment, every non-indexed stream must contain exactly one access unit within the Subsegment with presentation time less than or equal to the earliest presentation time of the Subsegment

- When multiple media streams are indexed in a single index file, the corresponding Segment Index for different media streams should index the same number of Subsegments.

If no Segment Index is provided for a Media Segment, then the Media Segment constitutes one Subsegment.

The Segment Index may be included in the Media Segment, typically in the beginning of the file. Segment Index information may also be provided in separate Index Segments as defined in 6.2.4. A Subsegment may itself be further subdivided using further Segment Index boxes. If a Subsegment only contains media data but no Segment Index, it is referred to as Media Subsegment.

- The Segment Index may contain additional Subsegment indexing information for accessing different levels of Subsegments in a Media Subsegment. For more details refer to 6.2.3.3.

A generic mechanism for indexing of Media Segments is provided by the Segment Index (‘sidx’) box in ISO/IEC 14496-12. This indexing applies to all media formats defined in this part of ISO/IEC 23009. In this case,

- the earliest presentation time of a Subsegment is documented in the earliest_presentation_time field.

- the byte range is document by the first_offset field and the reference_size field. If two Segment Index boxes document the same byte range, then the value of their first_offset field and their reference_size field shall be identical.

#### Subsegment Index

Media Subsegments may be indexed further to enable accessing different levels of Subsegments in a Media Subsegment. This Subsegment Index may also be provided in separate Index Segments together with the Segment Index.

A generic syntax and semantic for Subsegment indexing is provided by the Subsegment Index (‘ssix’) in ISO/IEC 14496-12.

### Index Segment

Index Segments contain information that is related to Media Segments and primarily contain indexing information for Media Segments. An Index Segment may provide information for one or more Media Segments.

The Index Segment may be media format specific and more details shall be defined for each media format that permits Index Segments.

### Bitstream Switching Segment

A Bitstream Switching Segment contains data essential for switching to the Representation it is assigned to.

The Bitstream Switching Segment is media format specific and more details shall be defined for each media format that permits Bitstream Switching Segments.

## Segment formats for ISO base media file format

### Introduction

This Clause defines Segment formats for use with Media Segments that are based on the ISO Base Media File Format as specified in ISO/IEC 14496-12. All Segment formats defined in 6.3 shall contain one or more boxes in accordance with the box structure of the ISO base media file format ISO/IEC 14496-12.

Refinements on generic concepts are introduced in 6.3.2. Segment formats are defined for Initialisation Segments (6.3.3), Media Segments (6.3.4), and Self-Initialising Media Segments (6.3.5). Bitstream Switching Segments and Index Segments are not defined for this media format.

### Preliminaries: Refinements of generic concepts

#### Subsegments

Media Subsegments for Media Segments based on the ISO base media file format are defined as a self-contained set of one or more consecutive movie fragments; such a set contains one or more movie fragment boxes with the corresponding media data (‘mdat’) box(es). A media data box containing data referenced by a movie fragment (‘moof’) box shall follow that movie fragment box and preceed the next movie fragment box containing information about the same track.

For a Media Subsegment, the value of the reference_type field in the describing Segment Index (‘sidx’) box shall be set to 0.

#### Media stream access points

Different types of media stream access points for the ISO base media file format are defined in ISO/IEC 14496-12, Annex I.

#### Segment Index

If the Segment Index is provided the Segment Index (‘sidx’) box in ISO/IEC 14496-12 shall be used. Exact definitions for the use of the Segment Index (‘sidx’) box with media formats based on the ISO base media file format are specified in ISO/IEC 14496-12.

#### Subsegment Index

If the Subsegment Index is provided the Subsegment Index (‘ssix’) box in ISO/IEC 14496-12 shall be used for. Exact definitions for the use of the Subsegment Index (‘ssix’) box for the use with media formats based on the ISO base media file format are specified in ISO/IEC 14496-12.

### Initialisation Segment format

The Initialisation Segment is conformant with the ISO base media file format.

The Initialisation Segment shall contain an “ftyp” box, and a “moov” box. It shall not contain any “moof” boxes. It may contain other boxes, such as the “pdin” box. The tracks in the “moov” box shall contain no samples (i.e. the entry_count in the “stts”, “stsc”, and “stco” boxes shall be set to 0), and the “moov” box is thus small.

NOTE	This may reduce the start-up time significantly as the Initialisation Segment needs to be downloaded before any Media Segment can be processed.

The “mvex” box shall be contained in the “moov” box to indicate that the client has to expect movie fragments. The “mvex” box also sets default values for the tracks and samples of the following movie fragments.

The Initialisation Segment provides the client with the metadata that describes the encoding of the media content, specifically of the Representation. The media engine in the client uses the information in the “moov” box to identify the available media content components and their characteristics.

NOTE	It is expected that the media engine in the DASH clients does not require any information in the MPD for successful decoding and presentation of the contained media streams.

### Media Segment types

#### General

Media Segments can be of different types: simple Media Segments, Indexed Media Segments and Sub-Indexed Media Segments.

All Media Segments shall conform to the general definitions in 6.3.4.2. Additional type-specific constraints are provided further below in 6.3.4.

Further rules on Media Segments in combination with certain MPD attributes are provided in .

Note that Media Segments may conform to multiple types. Conformance can be expressed by adding the brand(s) to the ‘styp’ box as a compatible brand and, if applicable, as the major brand.

Unless specified otherwise, the boxes referred in 6.3.4 are specified in ISO/IEC 14496-12.

#### General format type

A Media Segment conforming to the Media Segment Format for DASH is defined as follows:

- Each Media Segment may contain a ‘styp’ box and if present shall carry ‘msdh’ as a compatible brand. The conformance requirement of this brand is defined in this subclause.

- Each Media Segment shall contain one or more whole self-contained movie fragments. A whole, self-contained movie fragment is a movie fragment (‘moof’) box and a media data (‘mdat’) box that contains all the media samples that do not use external data references referenced by the track runs in the movie fragment box.

- Each ‘moof’ box shall contain at least one track fragment.

- The ‘moof’ boxes shall use movie-fragment relative addressing for media data that does not use external data references and the flag ‘default-base-is-moof’ shall also be set; absolute byte-offsets shall not be used for this media data. In a movie fragment, the duration by which each track extends should be as close to equal as practical. In particular, as movie fragments are accumulated, the track durations should remain close to each other and there should be no 'drift'.

- Each ‘traf’ box shall contain a ‘tfdt’ box.

NOTE	'The track fragment adjustment box ‘tfad’ as defined in 3GPP TS26.244 may also be present. DASH clients should not apply both the alignment established by the ‘tfdt’ and the time-shifting implied by the ‘tfad’, which would result in a double correction.

- Each Media Segment may contain one or more ‘sidx’ boxes. If present, the first ‘sidx’ box shall be placed before any ‘moof’ box and the first Segment Index box shall document the entire Segment.

#### Indexed Media Segment

A Media Segment conforming to the Indexed Media Segment Format is defined as follows:

- Each Media Segment shall comply with the general type as defined in 6.3.4.2 and in addition in each self-contained movie fragment, the movie fragment (‘moof’) box is immediately followed by its corresponding media data (‘mdat’).

- Each Media Segment shall contain one or more ‘sidx’ boxes. The first ‘sidx’ box shall be placed before any ‘moof’ box and shall document Subsegments that span the composition time of the entire Segment.

- Each Media Segment shall carry ‘msix’ as a compatible brand. The conformance requirements of this brand are defined in this subclause.

#### Sub-Indexed Media Segment

A Media Segment conforming to the Sub-Indexed Media Segment Format is defined as follows:

- It shall conform to the indexed Media Segment format as specified in 6.3.4.2.

- The Subsegment Index box (‘ssix’) shall be present and shall follow immediately after the ‘sidx’ box that documents the same Subsegment. This immediately preceding ‘sidx’ shall only index Media Subsegments.

- It shall carry ‘sims’ in the Segment Type box (‘styp’) as a compatible brand. The conformance requirements of this brand are defined in this subclause.

### Self-Initialising Media Segment formats

#### General format type

A Self-Initialising Media Segment conforms to the concatenation of an Initialisation Segment and a Media Segment without the 'styp' box preceeding the Media Segment.

The Self-Initialising Media Segment is conformant with the ISO base media file format.

NOTE	Since one Representation only contains one self-initialising Media Segment, switching is expected to happen within the Segment, e.g., at a Subsegment that contains a SAP.

#### Indexed self-initialising Media Segment

The Indexed Self-Initialising Media Segment conforms to the concatenation of an Initialisation Segment and a single Indexed Media Segment without the 'styp' box preceeding the Media Segment and shall carry ‘dash’ as a compatible brand.

The format of the Indexed self-initialising Media Segment is a conforming ISO base media file format file and defines the ‘dash’ brand.

## Segment formats for MPEG-2 transport streams

### Introduction

This clause introduces Segment formats that are suitable to be used if Media Segments are valid MPEG-2 TS, conforming to ISO/IEC 13818-1.

NOTE	It is possible to encapsulate MPEG-2 TS formatted media within an ISO base media file format. This mode of operation is not discussed in this Subclause. If MPEG-2 TS formatted media is encapsulated in an ISO base media file format, then the rules as defined in 6.3 apply.

Refinements on generic concepts are introduced in 6.4.2. Segment formats are defined for Initialisation Segments (see 6.4.3), Media Segments (see 6.4.4), Bitstream Switching Segments (see 6.4.5) and Index Segments (see 6.4.6). MPEG-2 TS specific box structures are defined in 6.4.7.

### Preliminaries: Refinements of generic concepts

#### Subsegment

In the context of MPEG-2 TS based delivery formats, a Subsegment is defined as an indexed set of access units consecutive in decode order. A subsegment shall contain complete access units for the indexed media stream (i.e., stream for which reference_ID equals PID), however it may contain incomplete PES packets from other media streams.

These access units are encapsulated in one or more PES packets. Each PES packet is encapsulated into one or more TS packets with the same PID value.

#### Media stream access points

For the case of MPEG-2 TS, a media stream is equivalent to an Elementary Stream as defined in ISO/IEC 13818-1.

Different types of media stream access points are defined in ISO/IEC 14496-12, Annex K. The same type definitions shall apply for the MPEG-2 TS. More specifically, in the case of MPEG-2 TS a SAP corresponds to an Elementary Stream Random Access Point, as defined in ISO/IEC 13818-1. Consequently, ISAU is the position of the first (sync) byte of a TS packet with PID assigned to this Elementary Stream. This TS packet contains the first byte of a PES packet, which, in turn, contains the Elementary Stream Access Point. PES packet starting at ISAU shall contain only an integral number of access units and shall contain a PTS.

NOTE 1	ISAU generally corresponds to the start of a TS packet with PID value for one Elementary Stream, the payload_unit_start_indicator field set to `1`, adaptation_field_control set to `11`, and the random_access_indicator field in the Adaptation Field is set to `1`. For SAP types 1-3, the random_access_indicator field in the Adaptation Field is commonly set to `1` (this is the case unless no PES payload bytes are found within the packet payload).

NOTE 2	Following the definitions in this Subclause, the first packet of the PCR PID is present at or prior to the TS packet at smallest ISAP. If PCR’s are carried on a media PID, the first packet of this PID is the first packet following the initialisation data, and carries a PCR. In order to avoid changing the underlying content, the implementer may choose to add a packet carrying only adaptation field with a PCR, but no payload. This packet is placed prior to the smallest ISAU of any stream in this Representation.

NOTE 3	If Index Segment is provided, and the `pcrb` box is present, PCR can be inferred from this box.

#### Segment Index

If the Segment Index is provided the Segment Index (‘sidx’) box in ISO/IEC 14496-12 shall be used for Segment Indexing. In addition to these definitions, the following conditions shall be met for a Segment Index used to describe MPEG-2 TS based Media Segment:

- reference_ID field of `sidx` box shall be the PID value of the indexed stream.

- All media offsets within `sidx` boxes shall be to the first (sync) byte of a TS packet

NOTE	Times within `sidx` boxes are expressed in units of the timescale field, rather than in 90KHz clock ticks.

#### Subsegment Index

If the Subsegment Index is provided the Subsegment Index (‘ssix’) box in ISO/IEC 14496-12 shall be used for indexing byte ranges within a subsegment. In addition to these definitions, the following conditions shall be met for a Segment Index used to describe MPEG-2 TS based Media Segment.

- All media offsets within `ssix` boxes shall be to the first (sync) byte of a TS packet

### Initialisation Segment types and formats

#### Initialisation information

Initialisation information is any information necessary to enable the media engine to start decoding the payload of any TS packet belonging to any media stream within a (Sub)Segment.

Untimed initialisation information includes PAT, CAT, PMT, EMM, and any other PSI information possibly included by the Media Presentation author. Any additional information that does not alter the Media Presentation timeline is allowed.

Time-varying initialisation information is information that is required for the successfull start of playout, but is different for at least two Subsegments or Segments within a Representation.

Mandatory initialisation information summarizes information that shall be present prior to any media data to enable decoding and presentation. As a consequence, mandatory initialisation information includes at least the following information, in this order:

- PAT (untimed, unless changes within the Representation);

- PMT (untimed, unless changes within the Representation);

- PCR (time-varying)

If MPEG-2 Conditional Access is used, ECM is considered mandatory untimed initialisation information if it does not change for the whole duration of the Period; otherwise it is considered mandatory time-varying initialisation information.

#### Initialisation Segment

An Initialisation Segment shall be a valid MPEG-2 TS, conforming to ISO/IEC 13818-1.

The concatenation of an Initialisation Segment with any Media Segment shall have the same presentation duration as the original Media Segment.

The Initialisation Segment shall contain mandatory untimed initialisation information as defined in 6.4.3.1. Time-varying initialisation information shall not be present in the Initialisation Segment, i.e.

- PCR-bearing packets shall not be present in the Initialisation Segment;

- ECM may be present as long as it does not change within the entire Representation;

- Any PSI table may be present as long as it does not change within the entire Representation;

Initialisation Segment may or may not be present. If it is not present for a given Representation, all Media Segments belonging to this Representation shall be self-initialising. Also, if an Initialisation Segment is used, not all initialisation information needs to reside in the Initialisation Segment, only presence of complete initialisation information in the concatenation of Initialisation Segment and Media Segment is required.

### Media Segment types and formats

#### General

All Media Segments shall conform to the basic Media Segment in 6.4.4.2.

Further rules on Media Segments in combination with certain MPD attributes are provided in 7.4.

#### Basic Media Segment

A Media Segment shall be a valid MPEG-2 TS, conforming to ISO/IEC 13818-1.

As a consequence of the requirement in 5.5.5.1, the concatenation of consecutive Media Segments of the same Representation shall also yield a valid MPEG-2 TS conforming to ISO/IEC 13818-1.

In addition, the following conditions shall be met:

- Media Segment boundaries shall be placed at MPEG-2 TS Packet boundaries

- Media Segments shall contain exactly one program.

- All time-varying initialisation information shall be present between ISAP and ISAU and/or in the Index Segment, if present.

- No Media Segment shall depend on initialisation information appearing in any preceding Media Segment.

Media Segments should contain only complete PES packets. Each PES packet should be comprised of one or more complete access units in each packet. Media Segments should contain only complete access units.

#### Content Protection

All information necessary for decrypting, or locating information required to decrypt, the encrypted TS packets in a (Sub)Segment shall be present before the encrypted packet(s) to which they apply, either in the same (Sub)Segment, and/or in the Initialisation Segment (if used). As an example, this requires the presence of the ECM necessary for decrypting the first encrypted packet of the (Sub)Segment is within the (Sub)Segment before such a packet. A Subsegment may not have an ECM preceding the first encrypted packet if the location of this ECM can be determined using an Index Segment.

NOTE	Sub-Represenations may be arranged such that information such as ECM is included in all Sub-Represenations that need them, for example by assigning the ECM an individual level and add dependency on all relevant Sub-Representations on this level.

#### Self-initialising Media Segment

A Self-initialising Media Segment conforms to the basic Media Segment as defined in 6.4.4.2 and in addition shall contain at the least all mandatory untimed and timed initialisation information as defined in 6.4.3.1.

All required initialisation information as defined in 6.4.3.1 should be present prior to any media data.

### Bitstream Switching Segment

A Bitstream Switching Segment shall be a valid MPEG-2 TS, conforming to ISO/IEC 13818-1.

A Bitstream Switching Segment shall not alter the Media Presentation timeline.

If initialisation information is carried within a Bitstream Switching Segment, it shall be identical to the one in the Initialisation Segment, if present, of the Representation.

NOTE	Authors should use Bitstream Switching Segments when there is a reasonable expectation of non-conforming behaviour (such as continuity counter errors, etc.) at the concatenation point of two consecutive Media Segments from different Representation, lack of correct initialisation information (two Representations with different initialisation information).

### Index Segment

#### General

Index Segments consist of a sequence of ISOBMFF-box-structures.

Index Segments may either be associated to a single Media Segment as specified in 6.4.6.2 or may be associated to all Media Segments in one Representation as specified in 6.4.6.3. An Index Segment may also contain a Subsegment Index as specified in 6.4.6.4 and any other boxes defined in 6.4.7.

It is recommended that Index Segments are at the least provided for one media stream.

NOTE 1	Despite the Media Segments are MPEG-2 TS based, Index Segments are reusing ISOBMFF-box-structures. This allows that the DASH access client in the model of Figure 2 to be universal and independent of the Media Format.

NOTE 2	Index Segments are not valid ISOBMFF files, and complete implementation of ISOBMFF is not necessary to utilize indexes in a MPEG-2 TS based client. A partial implementation would suffice, since only few ISOBMFF boxes, such as 'styp', 'sidx', and 'ssix' are required in order to parse an MPEG-2 TS Index Segment. Other box types may be present in an MPEG-2 TS Index Segment, but if present they shall not contain information required to interpret the `styp`, `sidx` or `ssix` boxes.

#### Single Index Segment

- An Single Index Segment indexes exactly one Media Segment and is defined as follows:

- Each Single Index Segment shall begin with an ‘styp’ box, and the brand ‘sisx’ shall be present in the ‘styp’ box. The conformance requirement of the brand ‘sisx’ is defined in this subclause.

- Each Single Index Segment shall contain one or more Segment Index boxes which index one Media Segment.

- A Single Index Segment may contain one or multiple ‘ssix’ boxes. If present, the ‘ssix’ shall follow the ‘sidx’ box that documents the same Subsegment without any other ‘sidx’ preceding the ‘ssix'.

- A Single Index Segment may contain one or multiple ‘pcrb’ boxes as defined in 6.4.7.2. If present, ‘pcrb’ shall follow the ‘sidx’ box that documents the same Subsegments, i.e. a ‘pcrb’ box provides PCR information for every subsegment indexed in the last ‘sidx’ box.

#### Representation Index Segment

- A Representation Index Segment indexes all Media Segments of one Representation and is defined as follows:

- Each Representation Index Segment shall begin with an ‘styp’ box, and the brand ‘risx’ shall be present in the ‘styp’ box. The conformance requirement of the brand ‘risx’ is defined in this subclause.

- Each Media Segment is indexed by one or more Segment Index box(es); the boxes for a given Media Segment are contiguous;

- Each Segment Index box may be followed by an 'ssix' and/or 'pcrb' box;

- The Segment Index for each Media Segments is concatenated in order, preceded by a single Segment Index box that indexes the Index Segment. This initial Segment Index box shall have one entry in its loop for each Media Segment, and each entry refers to the Segment Index information for a single Media Segment.

The structure of a Representation Index Segment is shown in Figure 3. This figure illustrates a case where a Representation Index Segment is provided and the Subsegment Index is used in order to enable efficient trick mode operation. The figure shows four consecutive Subsegments, S0, S1, S2, and S3, each indexed by an 'sidx' box, and two temporal layers within a video stream, I frames (L0) and P frames (L1), indexed by an 'ssix' box.

Figure 3 — Structure of Representation Segment Index

#### Subsegment Index Segment

A Subsegment Index Segment shall conform to an Index Segment and also includes a Subsegment Index. A Subsegment Index Segment is defined as follows:

- It shall be either a Single Index Segment or a Represention Index Segment.

- The Subsegment Index box (‘ssix’) shall be present and shall follow immediately after the ‘sidx’ box that documents the same Subsegment. The value of the reference_type field shall be equal to 0 for this Subsegment in this immediately preceding Segment Index (‘sidx’) box. If the 'pcrb' box is present, it shall follow 'ssix'.

- It shall carry ‘ssss’ in the Segment Type box (‘styp’) as a compatible brand. The conformance requirement of this brand is defined in this subclause.

### Boxes used with MPEG-2 TS Index Segments

#### Introduction

Index Segments may contain additional auxiliary information contained in boxes conforming to the ISO base media file format boxes. Boxes exclusively relevant for the MPEG-2 TS Media Segments are documented in 6.4.7.

#### MPEG-2 TS PCR information box

##### Definition

Box Type:	‘pcrb’
Container:	File
Mandatory:	No
Quantity:		Zero or one

Signals the PCR information for MPEG-2 TS.

##### Syntax

aligned(8) class MPEG2TSPCRInfoBox extends Box(‘pcrb’, 0) {

unsigned int(32)	subsegment_count;

for( i=1; i <= subsegment_count; i++){

unsigned int(42)	pcr;

unsigned int(6)	pad = 0;

}

}

##### Semantics

- subsegment_count is a positive integer specifying the number of Subsegments for which partial Subsegment information is specified in this box. subsegment_count shall be equal to reference_count in the last Segment Index box.

- pcr for each iteration of the loop, indicates the MPEG-2 TS PCR corresponding to the first (sync) byte of the first MPEG-2 TS packet in the media Subsegment corresponding to the current iteration. Note, that if this TS packet carries a PCR, its value will be different from the one specified in this field, since ISO/IEC 13818-1 defines PCR as relative to the byte containing the last bit of the program_clock_reference_base field.

# Combined semantics of MPD and Segment formats

## Introduction

An MPD and the referenced Segments comprise a Media Presentation. The formats for these two key components of a DASH-compatible Media Presentation are defined in 5 and 6 of this part of ISO/IEC 23009. In this clause, Media Presentation authoring rules are provided on how the MPD and different Segment formats may be combined to establish a complete Media Presentation.

Specifically aspects are addressed that deal with the Segment, that have special alignment with the Segments of other Representations to enable and simplify seamless switching and joint presentation.

General Media Presentation authoring rules are provided in 7.2 and specific ones for each media format are provided in the remainder of 7. Specifically rules when using the ISO base media file format are provided in 7.3 and the rules when using the MPEG-2 TS are provided in 7.4. Guidelines for other formats are provided in Annex F.

NOTE	Representation metadata present in the MPD may also be repeated in the media streams, e.g. in an Initialisation Segment or a Media Segment. The Media Presentation shall be provided such that no mismatch between these two values occurs. If it does, the value in the media stream itself takes precedence over values expressed in the MPD, especially when used in the media decoding process.

## General

### Media Presentation timeline

One of the key features in DASH is that encoded versions of different media components share a common timeline. The presentation time of access unit within the media content is mapped to the global common presentation timeline for synchronization of different media components and to enable seamless switching of different coded versions of the same media components.

The presentation times within each Period are relative to the PeriodStart time of the Period minus the value of the @presentationTimeOffset, TO, of the containing Representation. This means for an access unit with a presentation time TP signalled in the media stream, the Media Presentation time relative to the PeriodStart is TM=TP - TO.

Media Segments should not contain any presentation time TP that is smaller than the value of the @presentationTimeOffset, TO. However, if this is the case, then presentation of the Media Segment is expected to only take place for presentation times greater than or equal to TO.

The MPD start times as defined in 5.5.9.5.3 shall provide an approximation of the Media Presentation time TM within the Period. Specifically, the MPD start time shall be drift-free relative to the presentation time TP signalled in the media stream, i.e. the accuracy of the offset of the MPD start time relative to the presentation time does not depend on the position of the Segment in the Representation.

NOTE	At the start of a new Period, the playout procedure of the media content components may need to be adjusted at the end of the preceding Period to match the PeriodStart time of the new Period as there may be small overlaps or gaps with a Representation at the end of the preceding Period. Overlaps (respectively gaps) may result from Media Segments with actual presentation duration of the media stream longer (respectively shorter) than indicated by the Period duration. Also in the beginning of a Period if the earliest presentation time TP of any access unit of a Representation is not equal to TO then the playout procedures need to be adjusted accordingly.

For the case when MPD@type is "dynamic" and the attribute MPD@suggestedPresentationDelay is present, then the sum of value of the the MPD@availibilityStartTime, the PeriodStart value, the presentation time within the Period of an access unit, TM, and the value of the the attribute MPD@suggestedPresentationDelay provides a mapping of the presentation time of each access unit to the wall-clock time, for example to express synchronization with a content internal time or for other reasons to enable synchronization of presentation to the wall-clock.

Further media format specific definitions of presentation time may be defined.

### Segment Index

- If a Segment Index is present in a Media Segment of one Representation within an Adaptation Set, then the following shall hold:

  - the order of Segment Index boxes for multiple media streams induces an ordering on the media content components equal to the order in which a Segment Index box for a media stream for each component first appears. This ordering shall be the same for all Segments of all Representations of an Adaptation Set. As a consequence, if there is a Segment Index for a media content component in one Segment there shall be a Segment Index for that media component in all Segments in this Adaptation Set.

  - non-indexed media streams in all Representations of an Adaptation Set shall have the same access unit duration.

### Segment alignment

No additional requirements beyond those stated in 5.5.3.2 are defined.

### Subsegment alignment

No additional requirements beyond those stated in 5.5.3.2 are defined.

## Media Presentation based on the ISO base media file format

### General

The Media Presentation as introduced in 5 and 6 is instantiated in this subclause using the ISO base media file format as defined in ISO/IEC 14496-12 as Segment formats.

An ISOBMFF-based DASH Media Presentation is described by an MPD as specified in 5.1. The MIME type of the MPD shall be as defined in Annex C.

The general rules defined in 7.2 shall apply.

The @mimeType attribute of each Representation shall be provided according to RFC4337. Additional parameters may be added according to RFC6381.

The @segmentProfiles shall provide a comma-separated list of the individual Segment profile identifiers.

The following Segment types and formats may be used

- Initialisation Segments complying with formats as defined in 6.3.3.

- Media Segments complying with formats as defined in 6.3.4.2.

- Self-Initialising Media Segments complying with formats as defined in 6.3.5.

For ISO BMFF-based Media Presentation the following applies:

  - In all cases for which a Representation contains more than one Media Segment, the following applies:

    - The Initialisation Segment as defined in 6.3.3 shall be present.

    - Media Segments shall not be self-initialising. The Media Segment format is defined in 6.3.4.

    - If the Media Segment is the last Media Segment in the Representation, this Media Segment may carry the 'lmsg' compatibility brand. If the Media Segment is not the last Media Segment in the Representation, the 'lmsg' compatibility brand shall not be present. The 'lmsg' type is defined in this subclause.

  - In case a Representation contains only a single Media Segment, then one of the following two options are valid.

    - One Initialisation Segment as defined in 6.3.3 and one Media Segment as defined in 6.3.4 that is not self-initialising.

    - One Self-Initialising Media Segment as defined in 6.3.5.2. In this case the @segmentProfiles attribute may be absent.

Index Segments shall not be present. However, a RepresentationIndex element or a @indexRange attribute may be present to signal the byte range for Segment Index within a Media Segment.

The content authoring rules for the Media Segments when setting certain attributes are used for ISO BMFF-based DASH are provided in 7.3.2.

In case Sub-Representations are used, the rules in 7.3.4 shall apply.

### Media presentation timeline

The presentation time TP internal in the media that maps the media to the Media Presentation timeline shall be relative to the movie timeline, i.e. they are composition times after the application of any edit list for the track, as defined in ISO/IEC 14496-12, 8.16.3.

It is recommended that the @timescale attribute in the MPD matches the timescale field in the Media Header Box of a present track. If the Segment Index ('sidx') box is present, then it is further recommended that the track for which the Segment Index ('sidx') box that appears first in the Media Segment is the track defining the value of the @timescale attribute.

### Authoring Rules for specific MPD attributes

#### Segments starting with media stream access points

No additional requirements beyond those stated in 5.5.3.2 and 6.3.2.2 are defined.

#### Bitstream switching

If the @bitstreamSwitching is set to ‘true’ for a set of Representations within an Adaptation Set, the conditions stated in 5.5.3.2 shall be satisfied and the Bitstream Switching Segment shall not be present.

As a consequence of @bitstreamSwitching being set to ‘true’, the following conditions are satisfied:

- The track IDs for the same media content component are identical for each Representation in each Adaptation Set.

- The conditions required for setting the @segmentAlignment attribute to a value other than 'false' for the Adaptation Set are fulfilled.

- The conditions required for setting (i) the @startWithSAP attribute to 2 for the Adaptation Set, or (ii) the conditions required for all Representations within the Adaptation Set to share the same value of @mediaStreamStructureId and setting the @startWithSAP attribute to 3 for the Adaptation Set, are fulfilled.

### Sub-Representations

If a SubRepresentation element is present in a Representation in the MPD and the attribute SubRepresentation@level is present, then the Media Segments in this Representation shall conform to a Sub-Indexed Media Segment as defined in 6.3.4.4. The Initialisation Segment shall contain the Level Assignment (‘leva’) box.

The attribute @level specifies the level to which the described Sub-Representation is associated to in the Subsegment Index. The information in Representation, Sub-Representation and in the Level Assignment (‘leva’) box contains information on the assignment of media data to levels.

Media data should be ordered such that each level provides an enhancement compared to the lower levels.

## Media Presentation based on MPEG-2 TS

### Introduction

In this subclause, a Media Presentation is instantiated based on Media Segment Formats using the MPEG-2 TS as defined in ISO/IEC 13818-1. A MPEG-2 TS-based DASH Media Presentation is described by an MPD as specified in 5.2. The MIME type of the MPD shall be as defined in Annex C.

The general rules defined in 7.2 shall apply.

The @mimeType attribute of each Representation shall be "video/mp2t".

The following Segment types and formats may be used

- Initialisation Segments complying with formats as defined in 6.4.3.

- Media Segments complying with formats as defined in 6.4.4,

- Bitstream Switching Segments complying with formats as defined in 6.4.5,

- Index Segments complying with formats as defined in 6.4.6.

The @segmentProfiles attribute may be absent. If present, it is expected to be ignored.

### Media presentation timeline

The presentation time TP internal in the media that maps the media to the Media Presentation timeline shall be the one defined by the PTS in the MPEG-2 TS.

More specifically, for one Representation, let PTS(i) be the PTS of the the ith access unit in the media stream. Furthermore, let PTSA(i) be PTS(i) adjusted for 33-bit rollovers, i.e. calculated as if PTS had an infinite amount of bits.

TP calculation is based on differences between PTS(i) and PTS(0), and therefore TP(i) = (PTSA(i) – PTS0)*S/90000 with PTS0 typically PTS(0). With appropriate scaling, PTS0 be derived from the value of @presentationTimeOffset attribute.

NOTE	If Index Segment is used, S is provided by in the timescale field of the `sidx` box.

If a media stream contains a discontinuity, the PTSA(i) calculation assumes relative timing is maintained. Therefore, PTSA(i) will be adjusted by the difference between the value of PCR of the first PCR-bearing packet after the discontinuity and its interpolated PCR value (calculated using the pre-discontinuity PCR rate) .

In case of discontinuities it is recommended to add a new Period to reset the value of @presentationTimeOffset.

It is recommended that the @timescale attribute in the MPD matches the clock frequency S of the elementary streams. If the Segment Index ('sidx') box is present, then it is further recommended that the media stream for which the Segment Index ('sidx') box that appears first in the Index Segment is the elementary stream defining the value of the @timescale attribute.

### Authoring rules for specific MPD attributes

#### Segments starting with Media stream access points

No additional requirements beyond those stated in 5.5.3.2 and 6.4.2.2 are defined.

#### Segment alignment

If the @segmentAlignment flag is not set to ‘false’, the requirements stated in 5.5.2 and 5.5.3.2 shall be met. In addition, the Media Segment shall contain only complete PES packets and only complete access units, and the first PES packet shall contain a PTS timestamp.

#### Subsegment alignment

If the @subsegmentAlignment flag is not set to 'false', the semantics as defined in 5.5.3.2 shall apply. In particular, for an MPEG-2 TS-based Media Presentation, Subsegment shall contain only complete PES packets, and the first PES packet from each elementary stream shall contain a PTS.

#### Bitstream switching

If @bitstreamSwitching flag is set to 'true' for a set of Representations within an Adaptation Set, then the conditions stated in 5.5.3.2 shall be satisfied. In addition, the conditions in 5.5.3.2 shall not only hold for the entire sequence from i=1,...,M, but for any consecutive sequence of segments with any start index iS=1,...,M and any end index iE=iS,...,M.

If @bitstreamSwitching flag is set to 'true' the Bitstream Switching Segment may be present, indicated by BitstreamSwitching in the Segment Information. In this case, for any two Representations, X and Y, within the same Adaptation Set, concatenation of Media Segment i of X, Concatenation Segment of Representation Y, and Media Segment i+1 of Representation Y shall be a MPEG-2 TS conforming to ISO/IEC 13818-1.

As a consequence of the conformance rule as stated in 5.5.3.2, at least the following conditons are satisfied if @bitstreamSwitching flag is set to 'true':

- The conditions required for setting the @startWithSAP attribute to 2 for the Adaptatation Set or required for all Representations within the Adaptation Set share the same value of @mediaStreamStructureId and setting the @startWithSAP attribute of the Adapaton Set 3, are fulfilled.

- The conditions required for setting the @segmentAlignment attribute not set to 'false' for the Adaptation Set are fulfilled.

- PCR shall be present in the Segment prior to the first byte of a TS packet payload containing media data, and not inferred from the `pcrb` box

### Sub-Representations

If a SubRepresentation element is present in a Representation in the MPD and the SubRepresentation@level is present, then an Index Segment shall be present and shall conform to the format defined in 6.4.6.4.

The Subsegment Index box shall contain at least one entry for the value of SubRepresentation@level and for each value provided in the SubRepresentation@dependencyLevel. The remaining attributes of the SubRepresentation element should provide sufficient information such that the data contained in the Sub-Representation can be differentiated from the containing Representation as for the MPEG-2 TS no inband assignment of levels is provided.

If Subsegment Index is used for extraction of temporal subsequences, PCR should precede the first bytes of media within the range indicated in the Subsegment index. Also, encryption keys (if used) should not change within the duration of a Subsegment.

# Profiles

## Definition

Profiles of DASH are defined so as to enable interoperability and the signaling of the use of features.

A profile imposes a set of specific restrictions. Those restrictions are typically on features of the Media Presentation Description (MPD) document and on Segment formats, but may also be on content delivered within Segments, such as on media content types, media format(s), codec(s), and protection formats, or on quantitative measures such as bit-rates, Segment durations and sizes, as well as horizontal and vertical visual presentation size. Profiles defined in this part of ISO/IEC 23009 define restrictions on features of this part of ISO/IEC 23009 and on Segment formats only (e.g. not codec types). Externally defined profiles may additionally impose restrictions on other aspects.

NOTE	A profile can also be understood as permission for DASH clients that only implement the features required by the profile to process the Media Presentation (MPD document and Segments). However, as DASH client operation is not specified normatively in this part of ISO/IEC 23009, it is also unspecified how a DASH client conforms to a particular profile. Hence, profiles merely specify restrictions on MPD and Segments rather than DASH client behaviour.

A profile has an identifier, which is a URI. The profiles with which an MPD complies are indicated in the MPD@profiles attribute as a comma-separated list of profile identifiers. Profile identifiers defined in this part of ISO/IEC 23009 are URNs and shall conform to RFC3406. Externally defined profiles may use profile identifiers that are URNs or URLs. When a URL is used, it should also contain a month-date in the form mmyyyy; the assignment of the URL must have been authorized by the owner of the domain name in that URL on or very close to that date, to avoid problems when domain names change ownership.

An MPD is conforming when it satisfies the following:

- The MPD is valid in terms the schema defined in Annex B.

- The MPD conforms to the normative requirements defined in this part of ISO/IEC 23009.

- The MPD conforms to each of the profiles indicated in the MPD@profiles attribute as specified below.

When ProfA is included in the MPD@profiles attribute, the MPD is modified into a profile-specific MPD for profile conformance checking using the following ordered steps:

- The MPD@profiles attribute of the profile-specific MPD contains only ProfA.

- An AdaptationSet element for which @profiles does not or is not inferred to include ProfA is removed from the profile-specific MPD.

- A Representation element for which @profiles does not or is not inferred to include ProfA is removed from the profile-specific MPD.

- All elements or attributes that are either (i) in this part of ISO/IEC 23009 and explicitly excluded by ProfA, or (ii) in an extension namespace and not explicitly included by ProfA, are removed from the profile-specific MPD.

- All elements and attributes that “may be ignored” according to the specification of ProfA are removed from the profile-specific MPD,

An MPD is conforming to profile ProfA when it satisfies the following:

- ProfA is included in the MPD@profiles attribute.

- The profile-specific MPD for ProfA is valid in terms the schema defined in Annex B.

- The profile-specific MPD for ProfA conforms to the normative semantics defined in this part of ISO/IEC 23009.

- The profile-specific MPD for ProfA conforms to the restrictions specified for ProfA.

A Media Presentation is conforming to profile profA when it satisfies the following:

- The MPD of the Media Presentation is conforming to profile ProfA as specified above.

- There is at least one Representation in each Period in the profile-specific MPD for ProfA.

- The Segments of the Representations of the profile-specific MPD for ProfA conform to the restrictions specified for ProfA.

NOTE	In other words, each MPD contains at least one Representation in each Period, which fulfils the requirements of a profile listed in MPD@profiles. There may be stricter rules on the occurrence of Representations in the specified profiles. For example, it can be required that there is at least one Representation for each media type that contains or is inferred to have the profile identifier of a specific profile.

This part of ISO/IEC 23009 defines six profiles.

Three profiles are defined reyling on the ISO base media FF as Segment formats. Both, the ISO Base media file format On Demand profile defined in 8.3 and the ISO Base media file format live profile defined in 8.4 are a subset of the ISO Base media file format main profile defined in 8.5. Two profiles are defined for MPEG-2 TS based Media Segment formats: The MPEG-2 TS simple profile defined in 8.7 is a subset of the MPEG-2 TS main profile defined in 8.6. All profiles are a subset of the full profile is defined in 8.2.

## Full profile

### General

The full profile includes all features and Segment Types defined in this part of ISO/IEC 23009.

The full profile is identified by the URN “urn:mpeg:dash:profile:full:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- The rules for the MPD as defined in 7.3 or 7.4 shall apply.

### Segment format constraints

Representations and Segment formats shall conform to the following constraints:

- Representations shall comply either with the formats defined in 7.3, referring to the Segment formats in 6.3, or to the formats defined in 7.4, referring to the Segment formats in 6.4

## ISO Base media file format On Demand profile

### General

This profile is intended to provide basic support for On-Demand content. The primary constraints imposed by this profile are the requirement that each Representation is provided as a single Segment, that Subsegments are aligned across Representations within an Adaptation Set and that Subsegments must begin with Stream Access Points. This permits scalable and efficient use of HTTP servers and simplifies seamless switching.

The On-Demand profile is identified by the URN “urn:mpeg:dash:profile:isoff-on-demand:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- The rules for the MPD and the segments as defined in 7.3 shall apply.

- Representations not inferred to have @profiles equal to the profile identifier as defined in 8.3.1 may be ignored.

NOTE	A necessary condition to comply with the restrictions defined in 7.3 is that the @mimeType equals video/mp4, audio/mp4, or application/mp4.

- MPD@type shall be “static”.

- The Subset element may be ignored.

- neither the Period.SegmentList element nor the Period.SegmentTemplate element shall be present

- if either the AdaptationSet.SegmentList or the AdaptationSet.SegmentTemplate element is present in an AdaptationSet element then this AdaptationSet element may be ignored.

- if either the Representation.SegmentList or the Representation.SegmentTemplate element is present in a Representation element then this Representation element may be ignored.

- if the Representation element does not contain a BaseURL element then this Representation element may be ignored.

- AdaptationSet elements with AdaptationSet@subsegmentAlignment not present, or set to 'false' may be ignored.

- Representation elements with a @subsegmentStartWithSAP value (either supplied directly or inherited from the containing AdaptationSet) absent, zero or greater than 3 may be ignored.

- Representation elements with @ subsegmentStartWithSAP value (either supplied directly or inherited from the containing AdaptationSet) equal to 3 may be ignored if both the following conditions hold:

  - the containing Adaptation Set contains more than one Representation, and

  - no other Representation has the same value for @mediaStreamStructureId.

- Elements using the @xlink attribute may be ignored from the MPD. The Representations conforming to this profile are those not accessed through an Adaptation Set that uses an @xlink.

### Segment format constraints

Representations and Segments referred to by the Representations in the profile-specific MPD for this profile, the following constraints shall be met:

- Representations shall comply with the formats defined in 7.3, referring to the Segment formats in 6.3.

- Each Representation shall have one Segment that complies with the Self-Initialising Media Segment as defined in 6.3.5.2.

- All Segment Index ('sidx') and Subsegment Index ('ssix') boxes shall be placed before any Movie Fragment ('moof') boxes.

## ISO Base media file format live profile

### General

This profile is optimized for live encoding and low latency delivery of Segments consisting of a single movie fragment of ISO file format with relatively short duration. Each movie fragment may be requested when available using a template generated URL, so it is not normally necessary to request an MPD update prior to each Segment request. Segments are constrained so that they may be concatenated on Segment boundaries, and decrypted without gaps or overlaps in the media data, regardless of adaptive switching of Representations in an Adaptation Set. Note that despite the profile is optimized for live services, the MPD@Type attribute may be set to 'static' to distribute non-live content, for example in case a live Media Presentation is terminated, but kept available as On-Demand service.

The ISO Live profile is identified by the URN “urn:mpeg:dash:profile:isoff-live:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- The rules for the MPD and segments as defined in 7.3 shall apply.

- Representations not inferred to have @profiles equal to the profile identifier as defined in 8.4.1 may be ignored.

- In addition, Representation elements contained in an AdaptationSet element complying to this profile shall have the following constraints:

  - Representation elements with @startWithSAP value (either supplied directly or inherited from the containing AdaptationSet) equal to 3 may be ignored if both the following conditions hold:

    - the containing Adaptation Set contains more than one Representation, and

    - no other Representation has the same value for @mediaStreamStructureId.

  - The SegmentTemplate element shall be present on at least one of the three levels, the Period level containing the Representation, the Adaptation Set containing the Representation, or on Representation level itself.

- Elements using the @xlink attribute may be ignored from the MPD. The Representations conforming to this profile are those not accessed through an Adaptation Set that uses an @xlink.

- Representation elements with a @startWithSAP value (either supplied directly or inherited from the containin) absent, zero or greater than 3 may be ignored.

- Representation elements with a @segmentAlignment not equal 'true' (either supplied directly or inherited from the containing AdaptationSet) may be ignored.

- Representation elements with @startWithSAP value (either supplied directly or inherited from the containing Adaptation Set) equal to 3 may be ignored if both of the following conditions hold:

    - the containing Adaptation Set contains more than one Representation, and

    - no other Representation has the same value for @mediaStreamStructureId.

- Subset elements may be ignored.

### Segment format constraints

Representations and Segments referred to by the Representations in the profile-specific MPD for this profile, the following constraints shall be met:

- Representations shall comply with the formats defined in 7.3, referring to the Segment formats in 6.3.

- Each Representation shall have one Initilialisation Segment and at least one Media Segment.

- Media Segments containing multiple Media Components shall comply with the formats defined in 6.3.4.3, i.e. the brand 'msix'.

- In Media Segments, all Segment Index ('sidx') and Subsegment Index ('ssix') boxes shall be placed before any Movie Fragment ('moof') boxes.

## ISO Base media file format main profile

### General

This profile is identified by the URN “urn:mpeg:dash:profile:isoff-main:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- The rules for the MPD and segments as defined in 7.3 shall apply.

- Representations not inferred to have @profiles equal to the profile identifier as defined in 8.5.1 may be ignored.

- The Subset element may be ignored.

- Elements using the @xlink attribute may be ignored from the MPD. The Representations conforming to this profile are those not accessed through an Adaptation Set that uses an @xlink.

- Representation elements with a @startWithSAP value greater than 3 or @subsegmentStartsWithSAP value greater than 3 may be ignored.

- If MPD@type is ‘dynamic’,

  - AdaptationSet elements with a @segmentAlignment value (either supplied directly or inherited from the containing MPD) ‘false’ or absent may be ignored;

  - Representation elements with a @startWithSAP value (either supplied directly or inherited from the containing AdaptationSet) absent or zero may be ignored;

  - Representation elements with @startWithSAP value (either supplied directly or inherited from the containing AdaptationSet) equal to 3 may be ignored if both the following conditions hold:

    - the containing Adaptation Set contains more than one Representation, and

    - no other Representation has the same value for @mediaStreamStructureId.

### Segment format constraints

Representations and Segments referred to by the Representations in the profile-specific MPD for this profile, the following constraints shall be met:

- Representations shall comply with the formats defined in 7.3, referring to the Segment formats in 6.3.

- At least one SAP of type 1 to 3, inclusive, shall be present for each track in each Subsegment.

- In Media Segments, all Segment Index ('sidx') and Subsegment Index ('ssix') boxes shall be placed before any Movie Fragment ('moof') boxes.

- Each Media Segment of the Representations not having @startWithSAP present shall contain at least one Segment Index box.

## MPEG-2 TS main profile

### General

This profile imposes little constraint on the Media Segment format for MPEG-2 Transport Stream content.

This profile is identified by the URN “urn:mpeg:dash:profile:mp2t-main:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- The rules for the MPD as defined in 7.4 shall apply.

- Representations not complying with the restrictions defined in 7.4 or not inferred to have @profiles equal to the profile identifier as defined in 8.6.1 may be ignored.

- Representations not in group 0 may be ignored;

- Subset may be ignored;

- Representations containing the SegmentTimeline element may be ignored;

- It shall be possible to present a presentation conforming to this profile without resolving @xlink in AdaptationSet or SegmentList elements. Any initial Period elements using @xlink may be ignored, and the first non-excluded Period must have an explicit @start attribute. After the first non-excluded Period, there shall be no Period using @xlink.

### Segment format constraints

Representations and Segments referred to by the Representations in the profile-specific MPD for this profile, the following constraints shall be met:

- Representations shall comply with the formats defined in 7.4, referring to the Segment formats in 6.4.

### Comments and recommendations

The following may be used, if desired:

- Representations not complying with the restrictions defined in 7.4 may still be present, but the presentation must be presentable if they are ignored;

- Both, SegmentTemplate or SegmentList elements may be used; the normal case would be the use of SegmentList elements, however clients should be capable of handling SegmentTemplate elements.

For Representations conforming to this profile, it is recommended that:

- Index Segments are supplied.

- AdaptationSet elements containing Representations conforming to this profile should not set the value of the @segmentAlignment attribute (either supplied directly or inherited from the containing MPD) to ‘false’.

- Representations conforming to this profile should set the value of the @startWithSAP to 1 or 2. @startWithSAP may be set to 3 if @mediaStreamStructureId  is identical across Representations.

## MPEG-2 TS simple profile

### General

This profile is a subset of MPEG-2 TS main profile as defined in 8.6. It poses more restrictions on content encoding and multiplexing in order to allow simple implementation of seamless switching. This is achieved by guaranteeing that a media engine conforming to ISO/IEC 13818-1 can play any bitstream generated by concatenation of consecutive segments from any Representation within the same Adaptation Set.

This profile is identified by the URN “urn:mpeg:dash:profile:mp2t-simple:2011”.

### Media Presentation Description constraints

The Media Presentation Description shall conform to the following constraints:

- All MPD constraints of MPEG-2 TS Main Profile as defined in 8.6.2 shall be obeyed;

- Representations not complying with the restrictions defined in 7.4 or not inferred to have @profiles equal to the profile identifier as defined in 8.7.1 may be ignored.

- If an Index Segment is provided, any Adaptation Set with @SubsegmentAlignment set to ‘false’ may be ignored;

- Any Adaptation Set, which contains more than one Representation and has @bitstreamSwitching not set to ‘true’, may be ignored;

### Segment format constraints

Representations and Segments referred to by the Representations in the profile-specific MPD for this profile, the following constraints shall be met:

- Representations shall comply with the formats defined in 7.4, referring to the Segment formats in 6.4.

- All Media Segment constraints of MPEG-2 TS main profile as defined in 8.6.3 shall be obeyed.

- PSI information, including versions, shall be identical within all Representations contained in an AdaptationSet;

- If MPEG-2 Conditional Access framework is used, same ECM shall be valid for the whole Subsegment, or for the whole Segment if Index Segment is not present.

- For an Index Segment, any single Segment Index ('sidx') box may either reference media, or other 'sidx', but the same 'sidx' box may not reference both.

### Recommendations

For Representations conforming to this profile, it is recommended that:

- Index Segments are supplied

- SegmentTemplate elements are used.

- (informative)

Example DASH client behaviour

  - Introduction

The information on client behaviour is purely informative and does not imply any normative procedures on client implementations. However, this information may serve as a guideline to better understand certain features of the formats in the normative parts of this part of ISO/IEC 23009.

  - Overview

A DASH client is guided by the information provided in the MPD. This example assumes that the MPD@type is 'dynamic'. The behaviour in case MPD@type being 'static' is basically a subset of the description here.

The description in this Annex assumes that the client has access to the MPD at time FetchTime, at its initial location if no MPD.Location element is present, or at a location specified in any present MPD.Location element. FetchTime is defined as the time at which the server processes the request for the MPD from the client. The client typically should not use the time at which it actually successfully received the MPD, but should take into account delay due to MPD delivery and processing. The fetch is considered successful fetching either if the client either obtains an updated MPD or the client verifies that the MPD has not been updated since the previous fetching.

The following example client behaviour is expected to provide a continuous streaming experience to the user.:

  - The client parses the MPD, selects a set of Adaptation Sets suitable for its environment based in information provided in each of the AdaptationSet elements. The selection of Adaptation Sets may also take into account information provided by the AdaptationSet@group attribute and any constraints of a possibly present Subset element.

  - Within each Adaptation Set it selects one specific Representation, typically based on the value of the @bandwidth attribute, but also taking into account client decoding and rendering capabilities. Then it creates a list of accessible Segments for each Representation for the actual client-local time NOW measured in wall-clock time taking into account the procedures introduced in A.3.

  - The client accesses the content by requesting entire Segments or byte ranges of Segments. The client requests Media Segments of the selected Representation by using the generated Segment list.

  - The client buffers media of for at least value of @minBufferTime attribute duration before starting the presentation. Then, once it has identified a Stream Access Point (SAP) for each of the media streams in the different Representations, it starts rendering (in wall-clock-time) of this SAP not before PeriodStart +TSAP and not after PeriodStart +TSAP + @timeShiftBufferDepth provided the observed throughput remains at or above the sum of the @bandwidth attributes of the selected Representations (if not, longer buffering may be needed). For services with MPD@type='dynamic', rendering the SAP at the sum of PeriodStart +TSAP and the value of MPD@suggestedPresentationDelay is recommended, especially of synchronized play-out with other devices adhering to the same rule is desired.

  - Once the presentation has started, the client continues consuming the media content by continuously requesting Media Segments or parts of Media Segments. The client may switch Representations taking into account updated MPD information and/or updated information from its environment, e.g. change of observed throughput. With any request for a Media Segment containing a random access point, the client may switch to a different Representation. Seamless switching can be achieved as the different Representations are time-aligned. Advantageous switching points are announced in the MPD and/or in the Segment Index, if provided.

  - With the wall-clock time NOW advancing, the client consumes the available Segments. As NOW advances the client possibly expands the list of available Segments for each Representation according to the procedures specified in A.3 If the following clauses are both true, an updated MPD should be fetched:

    - The @mediaPresentationDuration attribute is not declared, or if any media described in the MPD does not reach to the end of the Media Presentation and

    - the current playback time gets within a threshold (typically described by at least the sum of the value of the @minBufferTime attribute and the value of the @duration attribute (or the equivalent value in case the SegmentTimeline is used) of the media described in the MPD for any consuming or to be consumed Representation

  - If the clauses in 29) are true, the client should fetch a new MPD, and update FetchTime. Once received the client takes into account the possibly updated MPD and the new FetchTime in the regeneration of the accessible Segment list for each Representation.

In the following clauses a brief overview on Segment list generation, seeking, support for trick modes and switching Representations are provided.

  - Segment list generation

    - General

Assume that the DASH client has access to an MPD. This clause describes how a client may generate a Segment list for one Representation as shown in Table  from an MPD obtained at FetchTime at a specific client-local time NOW. In this description, the term NOW is used to refer to “the current value of the clock at the reference client when performing the construction of an MPD Instance from an MPD”. A client that is not synchronised with a DASH server, which is in turn is expected to be synchronised to UTC, may experience issues in accessing Segments as the Segment availability times provided by the server and the local time NOW may not be synchronized. Therefore, DASH clients are expected to synchronize their clocks to a globally accurate time standard.

Table A.1 — Segment list in example client

| Parameter Name |  |  | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Segments |  |  | 1 | Provides the Segment URL list. |
|  | InitialisationSegment |  | 0, 1 | Describes the Initialisation Segment. If not present each Media Segment is self-initialising. |
|  |  | URL | 1 | The URL where to access the Initialisation Segment (the client would restrict the URL with a byte range if one is provided in the MPD). |
|  | MediaSegment |  | 1 … N | Describes the accessible Media Segments. |
|  |  | startTime | 1 | The MPD start time of the Media Segment in the Period relative to the start time of Period. |
|  |  | duration | 1 | The MPD duration for the Segment |
|  |  | URL | 1 | The URL where to access the Media Segment, possibly combined with a byte range. |
|  | IndexSegment |  | 1 … N | Describes the accessible Index Segments, if present. |
|  |  | URL | 1 | The URL where to access the Index Segment, possibly combined with a byte range. |

According to 5.5.9 there exist three different ways to describe and generate a Segment List. This description focusses on the first two where either a SegmentList element or a SegmentTemplate element is present. The case with a single Media Segment using BaseURL element and SegmentBase element is considered straightforward.

- If the Representation contains or inherits a SegmentTemplate element, then the procedures in A.3.2 are used to generate a list of Media Segments.

- If the Representation contains or inherits one or more SegmentList elements, (providing a set of explicit URL(s) for Media Segments, then the procedures in A.3.3 are used to generate a list of Media Segments.

- If the MPD@type attribute is 'dynamic', then the restrictions on Media Segment Lists as provided in A.3.4 need to be taken into account.

- The client should only request Segments that are included in the Segment list when generated at the actual wall-clock time NOW.

    - Template-based generation of Segment list

If the Representation contains or inherits a SegmentTemplate element, then the procedures in this subclause are used to generate a list of Segment parameters, i.e. Segment URLs and Media Segment start times, and no byte ranges are associated with the URLs.

Assume that the Period end time documented in the current MPD with fetch time FetchTime is defined as PeriodEndTime. For any Period in the MPD except for the last one, the PeriodEndTime is obtained as the value of the PeriodStart time of the next Period. For the last Period in the MPD

- if the MPD@mediaPresentationDuration attribute is present, then PeriodEndTime is defined as the end time of the Media Presentation.

- if the MPD@mediaPresentationDuration attribute is not present, then PeriodEndTime is defined as FetchTime + MPD@minimumUpdatePeriod.

For the SegmentTemplate element, the relevant identifiers are replaced in the SegmentTemplate@media.

Assume now that Media Segments within a Representation have been assigned consecutive indices i=@startNumber, @startNumber + 1 i.e. the first Media Segment has been assigned the index i=1, the second Media Segment has been assigned the index i=2, and so on. If Index Segments are provided, each Index Segment has an identical index i assigned to it.

A valid list of Media Segments with Segment indices i, MediaSegment.StartTime[i] and MediaSegment.URL[i], and if present, a corresponding list of Index Segments with IndexSegment.URL[i], i=@startNumber, @startNumber + 1, …, is obtained as follows using the @duration attribute for this Representation:

  - Set i=@startNumber.

  - The MPD start time of the first Media Segment is 0, i.e. MediaSegment.StartTime[1] = 0.

  - The URL of the Media Segment i, MediaSegment.URL[i], is obtained by replacing the $Number$ identifier by i in the template.

  - If Index Segments are present, the URL of the Index Segment i, IndexSegment.URL[i], is obtained by replacing the $Number$ identifier by i in the template. Furthermore, any relative URLs are resolved by reference resolution.

  - If ((PeriodStart + MediaSegment.StartTime[i] + @duration) <= PeriodEndTime) then increment i, set MediaSegment.StartTime[i] = MediaSegment.StartTime[i-1] + @duration, and proceed with step 3. Otherwise, continue with step 5.

  - A new Media Segment is added to the list, i.e. i = i + 1, MediaSegment.StartTime[i] = MediaSegment.StartTime[i-1] + @duration and the guaranteed duration is set to MediaSegment.duration[i] = PeriodEndTime - MediaSegment.StartTime[i] The restrictions as specified in A.3.4 are applied for the creation of the accessible list of Media Segments and this concludes Segment List generation.

- If instead of the the @duration attribute a SegmentTimeline element is given, then the variable durations of the segments are used to compute the start times and durations. Also, depending on the identifier, $Number$ or $Time$, the appropriate replacements are done as introduced in 5.5.9.6.

    - Playlist-based generation of Segment list

If the Representation contains or inherits one or more SegmentList elements, each containing SegmentURL elements, then the procedures specified in this subclause apply to generate a valid list of accessible Segment URLs and Media Segment start times.

Assume that Media Segments within a Representation have been assigned consecutive indices i=@startNumber, @startNumber+1, …., i.e. the first Media Segment has been assigned i=@startNumber, the second Media Segment has been assigned i=@startNumber+1, and so on. If an Index Segment is provided for each Media Segment, each Index Segment has an identical index i assigned to it.

If a SegmentTimeline element has been given, a list of Segment start times and durations is first generated by expanding the SegmentTimeline from its run-length compressed form into a SegmentTimelineList of StartTime values for each Segment. This new list is indexed starting at @startNumber.

A valid list of Media Segments with Segment indices i=@startNumber, @startNumber+1, …, MediaSegment.StartTime[i] and MediaSegment.URL[i], and if present, a corresponding list of Index Segments with IndexSegment.URL[i] is obtained as follows:

  - Set i=@startNumber.

  - The URL of the Media Segment i, MediaSegment.URL[i], is obtained as the sourceURL attribute of the (i-@startNumber +1)th Url element in the SegmentList element taking into account URI reference resolution, restricted to the byte range specified in the range attribute of the same SegmentURL element, if present.

  - The URL of the Index Segment i, IndexSegment.URL[i], is obtained also from the SegmentURL element or inherited from above

  - If the duration attribute is provided, then the MediaSegment.StartTime[i] of Media Segment i is obtained as (i-@startNumber -1)*@duration. If the @duration attribute is not provided and a SegmentTimeline element is in effect then the variable durations are taken into account for the computation of the start times. Otherwise, the MediaSegment.StartTime[1] of the only provided Segment is set to 0.

  - If this is not the last SegmentURL element, a new Media Segment is added to the list, i.e. i = i + 1, and proceed with step 2; Otherwise continue with step 5.

  - The restrictions as specified in A.3.4 are applied for the creation of the accessible list of Media Segments. This concludes Segment list generation.

    - Media Segment list restrictions

The Media Segment List is restricted to a list of accessible Media Segments, which may be a subset of the Media Segments of the complete Media Presentation. The construction is governed by the current value of the clock at the client NOW.

Generally, Segments are only available for any time NOW between MPD@availabilityStartTime and MPD@availabilityEndTime. For times NOW outside this window, no Segments are available.

In addition, for services with MPD@type='dynamic', assume the variable CheckTime associated to an MPD with FetchTime is defined as:

  - If the MPD@minimumUpdatePeriod attribute in the client is provided, then the check time is defined as the sum of the fetch time of this operating MPD and the value of this attribute, i.e. CheckTime = FetchTime + MPD@minimumUpdatePeriod.

  - If the MPD@minimumUpdatePeriod attribute in the client is not provided, external means are used to determine CheckTime, such as a priori knowledge, or HTTP cache headers, etc.

The CheckTime is defined on the MPD-documented media time axis; when the client’s playback time reaches CheckTime it should fetch a new MPD.

Then, the Media Segment list is further restricted by the CheckTime together with the MPD attribute MPD@timeShiftBufferDepth such that only Media Segments for which the sum of the start time of the Media Segment and the Period start time falls in the interval [NOW- MPD@timeShiftBufferDepth - @duration, min(CheckTime, NOW)] are included.

  - Seeking

Assume that a client attempts to seek to a specific Media Presentation time TM in a Representation relative to the PeriodStart time. According to 7.2.1, the presentation times within each Period are relative to the PeriodStart time of the Period minus the value of the @presentationTimeOffset, TO, of the containing Representation.

Based on a fully assembled MPD, the client has access to the MPD start time and Media Segment URL of each Segment in the Representation, along with Index Segment URL, if present. The Segment number of the Segment most likely to contain media samples for Media Presentation time TM is obtained as the maximum Segment index i*, for which the start time MediaSegment[i].StartTime is smaller or equal to the TM. The Segment URL is obtained as MediaSegment[i*].URL.

Note that timing information in the MPD may be approximate due to issues related to placement of Representation Access Points, alignment of media tracks and media timing drift. As a result, the Segment identified by the procedure above may begin at a time slightly after TM and the media data for presentation time may be in the previous Media Segment. In case of seeking, either the seek time may be updated to equal the first sample time of the retrieved Media Segment, or the preceding Media Segment may be retrieved instead. However, note that during continuous playout, including cases where there is a switch between alternative versions, the media data for the time between TM and the start of the retrieved Segment is always available.

For accurate seeking to a presentation time TM, the DASH Client needs to access Stream Access Points (SAP). To determine the SAP in a Media Segment in case of DASH, the client may, for example, use the information in the Segment Index if present to locate the random access points and the corresponding presentation time in the Media Presentation.

In the case that the Media Presentation is based on the ISO base media file format and a Segment is a movie fragment, it is also possible for the client to use information within the ‘moof’ and ‘mdat’ boxes, for example, to locate Random Access Points in the media and obtain the necessary presentation time from the information in the movie fragment and the Segment start time derived from the MPD. If no SAP with presentation time before the requested presentation time TM is available, the client may either access the previous Segment or may just use the first SAP as the seek result. When Media Segments start with a SAP, these procedures are simplified.

In the case that the Media Presentation is based on MPEG-2 TS, the presentation units corresponding to the desired presentation time TM can be identified by using the indexing information, if present, in conjunction with the differential value of the presentation time stamps (PTS) within the Media Segment. For example, if TM,S denotes the presentation time corresponding to the last SAP leading the desired seek time tp, with a corresponding PTS denoted as PTSs, then the desired seek position within the media has a PTS expressed as: ((TM - TM,S)*timescale+PTSS%233.

Also note that not necessarily all information of the Media Segment needs to be downloaded to access the presentation time TM. The client may for example initially request the Segment Index from the beginning of the Media Segment using partial HTTP GET. By use of the Segment Index, Segment timing can be mapped to byte ranges of the Segment. By continuously using partial HTTP GET requests, only the relevant parts of the Media Segment may be accessed for improved user experience and low start-up delays.

  - Support for trick modes

The client may pause or stop a Media Presentation. In this case client simply stops requesting Media Segments or parts thereof. To resume, the client sends requests to Media Segments, starting with the next Subsegment after the last requested Subsegment.

If a specific Representation or SubRepresentation element includes the @maxPlayoutRate attribute, then the corresponding Representation or Sub-Representation may be used for the fast-forward trick mode. If a specific Representation or SubRepresentation element includes the @codingDependency attribute, then the corresponding Representation or Sub-Representation may be used for both fast-forward and fast-rewind trick modes. The client may play the Representation or Sub-Representation with any speed up to the regular speed times the specified @maxPlayoutRate attribute with the same decoder profile and level requirements as the normal playout rate.

Sub-Representations in combination with Index Segments and Subsegment Index boxes may be used for efficient trick mode implementation. Given a Sub-Representation with the desired @maxPlayoutRate, ranges corresponding to SubRepresentation@level all level values from SubRepresentation@dependencyLevel may be extracted via byte ranges constructed from the information in Subsegment Index Box. These ranges can be used to construct more compact HTTP GET request.

  - Switching Representations

Based on updated information during an ongoing Media Presentation, a client may decide to switch Representations. Switching to a “new” Representation is equivalent to tuning in or seeking to the new Representation from the time point where the "old" Representation has been presented. Once switching is desired, the client should seek to a SAP for each media stream in the “new” Representation at a desired presentation time tp later than and close to the current presentation time. Presenting the “old” Representation up to the RAP in the “new” Representation enables seamless switching.

If @segmentAligment is set true and the @startWithSAP is set to 1, 2 or 3 (and in the latter case the Representation@mediaStreamStructureId is identical for the two Representations), then the client may switch at any Segment boundary by just concatenating Segments with consecutive indices from different Representations. No overlap downloading and decoding is required.

The same can be achieved on Subsegment level with @subsegmentAlignment set true and @subsegmentStartWithSAP the same values and conditions as above.

  - Reaction to error codes

The DASH access client provides a streaming service to the user by issuing HTTP requests for Segments at appropriate times. The DASH access client may also update the MPD by using HTTP requests. In regular operation mode, the server typically responds to such requests with status code 200 OK (for regular GET) or status code 206 Partial Content (for partial GET) and the entity corresponding to the requested resource. Other Successful 2xx or Redirection 3xx status codes may be returned.

HTTP requests may result in a Client Error 4xx or Server Error 5xx status code. Some guidelines are provided in this subclause as to how an HTTP client may react to such error codes.

If the DASH access client receives an HTTP client or server error (i.e. messages with 4xx or 5xx error code), the client should respond appropriately (e.g. as indicated in RFC 2616) to the error code. In particular, clients should handle redirections (such as 301 and 307) as these may be used as part of normal operation.

If the DASH access client receives a repeated HTTP error for the request of an MPD, the appropriate response may involve terminating the streaming service.

If the DASH access client receives an HTTP client error (i.e. messages with 4xx error code) for the request of an Initialisation Segment, the Period containing the Initialisation Segment may not be available anymore or may not be available yet.

Similarly, if the DASH access client receives an HTTP client error (i.e. messages with 4xx error code) for the request of a Media Segment, the requested Media Segment may not be available anymore or may not be available yet. In both these case the client should check if the precision of the time synchronization to a globally accurate time standard is sufficiently accurate. If the clock is believed accurate, or the error re-occurs after any correction, the client should check for an update of the MPD.

Upon receiving server errors (i.e. messages with 5xx error code), the client should check for an update of the MPD. If multiple BaseURL elements are available, the client may also check for alternative instances of the same content that are hosted on a different server.

  - Encoder clock drift control

Non-alignment between the end of a Representation in one Period and the start time of the next Period may be caused by encoder clock inaccuracy. The client should align the Media Presentation time at each Period start. In addition, significant deviations of the start time of Segments to the media time should be detected and drift-compensating measures may be applied even before the start of the next Period is reached.

During long live streaming sessions, a difference in clock accuracy of the encoder and decoder may cause the playback to lag behind real-time or to interrupt temporarily due to the client trying to access data faster than real-time.

For ISO base media file format based, clients may avoid these anomalies by using the Producer Reference Time boxes as follows. The pace r1 of the encoder clock in relation to the UTC is recovered from Producer Reference Time boxes. If the relative pace r1 is less than 1, equal to 1, or greater than 1, the encoder clock runs more slowly than the UTC, at an identical pace compared to the UTC, or faster than the UTC, respectively. The pace r2 of the receiver playout clock in relation to UTC is created by accessing a UTC source. A timescale multiplication factor c is equal to r1/r2. A presentation time on a timeline of the receiver playout clock is derived for each sample or access unit by multiplying the composition time of the sample (as indicated by the file format structures) or the presentation time of the access unit (as indicated by the respective Program Elementary Stream header) by the timescale multiplication factor c.

In case of MPEG-2 TS segments, PCR-based drift control may be used.

- (normative)

MPD schema

<?xml version="1.0" encoding="UTF-8"?> <xs:schema targetNamespace="urn:mpeg:DASH:schema:MPD:2011" attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns="urn:mpeg:DASH:schema:MPD:2011"> <xs:import namespace="http://www.w3.org/1999/xlink" schemaLocation="http://www.w3.org/1999/xlink.xsd"/> <xs:annotation> <xs:appinfo>Media Presentation Description</xs:appinfo> <xs:documentation xml:lang="en"> This Schema defines the Media Presentation Description for MPEG-DASH. </xs:documentation> </xs:annotation> <!-- MPD: main element --> <xs:element name="MPD" type="MPDtype"/> <!-- MPD Type --> <xs:complexType name="MPDtype"> <xs:sequence> <xs:element name="ProgramInformation" type="ProgramInformationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Location" type="xs:anyURI" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Period" type="PeriodType" maxOccurs="unbounded"/> <xs:element name="Metrics" type="MetricsType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:string"/> <xs:attribute name="profiles" type="xs:string"/> <xs:attribute name="type" type="PresentationType" default="static"/> <xs:attribute name="availabilityStartTime" type="xs:dateTime"/> <xs:attribute name="availabilityEndTime" type="xs:dateTime"/> <xs:attribute name="mediaPresentationDuration" type="xs:duration"/> <xs:attribute name="minimumUpdatePeriod" type="xs:duration"/> <xs:attribute name="minBufferTime" type="xs:duration" use="required"/> <xs:attribute name="timeShiftBufferDepth" type="xs:duration"/> <xs:attribute name="suggestedPresentationDelay" type="xs:duration"/> <xs:attribute name="maxSegmentDuration" type="xs:duration"/> <xs:attribute name="maxSubsegmentDuration" type="xs:duration"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Presentation Type enumeration --> <xs:simpleType name="PresentationType"> <xs:restriction base="xs:string"> <xs:enumeration value="static"/> <xs:enumeration value="dynamic"/> </xs:restriction> </xs:simpleType> <!-- Period --> <xs:complexType name="PeriodType"> <xs:sequence> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="AdaptationSet" type="AdaptationSetType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Subset" type="SubsetType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="id" type="xs:string" /> <xs:attribute name="start" type="xs:duration"/> <xs:attribute name="duration" type="xs:duration"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean" default="false"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Adaptation Set --> <xs:complexType name="AdaptationSetType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentComponent" type="ContentComponentType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> <xs:element name="Representation" type="RepresentationType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="group" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="xs:string"/> <xs:attribute name="par" type="xs:string"/> <xs:attribute name="minBandwidth" type="xs:unsignedInt"/> <xs:attribute name="maxBandwidth" type="xs:unsignedInt"/> <xs:attribute name="minWidth" type="xs:unsignedInt"/> <xs:attribute name="maxWidth" type="xs:unsignedInt"/> <xs:attribute name="minHeight" type="xs:unsignedInt"/> <xs:attribute name="maxHeight" type="xs:unsignedInt"/> <xs:attribute name="minFrameRate" type="xs:string"/> <xs:attribute name="maxFrameRate" type="xs:string"/> <xs:attribute name="segmentAlignment" type="ConditionalUintType" default="false"/> <xs:attribute name="subsegmentAlignment" type="ConditionalUintType" default="false"/> <xs:attribute name="subsegmentStartsWithSAP" type="SAPType" default="0"/> <xs:attribute name="bitstreamSwitching" type="xs:boolean"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Conditional Unsigned Integer (unsignedInt or boolean) --> <xs:simpleType name="ConditionalUintType"> <xs:union memberTypes="xs:unsignedInt xs:boolean"/> </xs:simpleType> <!-- Content Component --> <xs:complexType name="ContentComponentType"> <xs:sequence> <xs:element name="Accessibility" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Role" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Rating" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="Viewpoint" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="id" type="xs:unsignedInt"/> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="contentType" type="xs:string"/> <xs:attribute name="par" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Representation --> <xs:complexType name="RepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:sequence> <xs:element name="BaseURL" type="BaseURLType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SubRepresentation" type="SubRepresentationType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="SegmentBase" type="SegmentBaseType" minOccurs="0"/> <xs:element name="SegmentList" type="SegmentListType" minOccurs="0"/> <xs:element name="SegmentTemplate" type="SegmentTemplateType" minOccurs="0"/> </xs:sequence> <xs:attribute name="id" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt"/> <xs:attribute name="dependencyId" type="StringVectorType"/> <xs:attribute name="mediaStreamStructureId" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- SubRepresentation --> <xs:complexType name="SubRepresentationType"> <xs:complexContent> <xs:extension base="RepresentationBaseType"> <xs:attribute name="level" type="xs:unsignedInt"/> <xs:attribute name="dependencyLevel" type="UIntVectorType"/> <xs:attribute name="bandwidth" type="xs:unsignedInt"/> <xs:attribute name="contentComponent" type="StringVectorType"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Representation base (common attributes and elements) --> <xs:complexType name="RepresentationBaseType"> <xs:sequence> <xs:element name="FramePacking" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="AudioChannelConfiguration" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="ContentProtection" type="DescriptorType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="profiles" type="xs:string"/> <xs:attribute name="width" type="xs:unsignedInt"/> <xs:attribute name="height" type="xs:unsignedInt"/> <xs:attribute name="sar" type="xs:string"/> <xs:attribute name="frameRate" type="xs:string"/> <xs:attribute name="audioSamplingRate" type="xs:string"/> <xs:attribute name="mimeType" type="xs:string"/> <xs:attribute name="segmentProfiles" type="xs:string"/> <xs:attribute name="codecs" type="xs:string"/> <xs:attribute name="maximumSAPPeriod" type="xs:double"/> <xs:attribute name="startWithSAP" type="SAPType"/> <xs:attribute name="maxPlayoutRate" type="xs:double"/> <xs:attribute name="codingDependency" type="xs:boolean"/> <xs:attribute name="scanType" type="VideoScanType"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Stream Access Point type enumeration --> <xs:simpleType name="SAPType"> <xs:restriction base="xs:unsignedInt"> <xs:minInclusive value="0"/> <xs:maxInclusive value="6"/> </xs:restriction> </xs:simpleType> <!-- Video Scan type enumeration --> <xs:simpleType name="VideoScanType"> <xs:restriction base="xs:string"> <xs:enumeration value="progressive"/> <xs:enumeration value="interlaced"/> <xs:enumeration value="unknown"/> </xs:restriction> </xs:simpleType> <!-- Subset --> <xs:complexType name="SubsetType"> <xs:attribute name="contains" type="UIntVectorType" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Segment information base --> <xs:complexType name="SegmentBaseType"> <xs:sequence> <xs:element name="Initialisation" type="URLType" minOccurs="0"/> <xs:element name="RepresentationIndex" type="URLType" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="timescale" type="xs:unsignedInt"/> <xs:attribute name="presentationTimeOffset" type="xs:unsignedInt"/> <xs:attribute name="indexRange" type="xs:string"/> <xs:attribute name="indexRangeExact" type="xs:boolean"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Multiple Segment information base --> <xs:complexType name="MultipleSegmentBaseType"> <xs:complexContent> <xs:extension base="SegmentBaseType"> <xs:sequence> <xs:element name="SegmentTimeline" type="SegmentTimelineType" minOccurs="0"/> <xs:element name="BitstreamSwitching" type="URLType" minOccurs="0"/> </xs:sequence> <xs:attribute name="duration" type="xs:unsignedInt"/> <xs:attribute name="startNumber" type="xs:unsignedInt"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Segment Info item URL/range --> <xs:complexType name="URLType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="sourceURL" type="xs:anyURI"/> <xs:attribute name="range" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Segment List --> <xs:complexType name="SegmentListType"> <xs:complexContent> <xs:extension base="MultipleSegmentBaseType"> <xs:sequence> <xs:element name="SegmentURL" type="SegmentURLType" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute ref="xlink:href"/> <xs:attribute ref="xlink:actuate" default="onRequest"/> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Segment URL --> <xs:complexType name="SegmentURLType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="media" type="xs:anyURI"/> <xs:attribute name="mediaRange" type="xs:string"/> <xs:attribute name="index" type="xs:anyURI"/> <xs:attribute name="indexRange" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Segment Template --> <xs:complexType name="SegmentTemplateType"> <xs:complexContent> <xs:extension base="MultipleSegmentBaseType"> <xs:attribute name="media" type="xs:string"/> <xs:attribute name="index" type="xs:string"/> <xs:attribute name="initialisation" type="xs:string" /> <xs:attribute name="bitstreamSwitching" type="xs:string" /> </xs:extension> </xs:complexContent> </xs:complexType> <!-- Segment Timeline --> <xs:complexType name="SegmentTimelineType"> <xs:sequence> <xs:element name="S" minOccurs="1" maxOccurs="unbounded" > <xs:complexType> <xs:attribute name="t" type="xs:unsignedInt"/> <xs:attribute name="d" type="xs:unsignedInt" use="required"/> <xs:attribute name="r" type="xs:unsignedInt" use="optional" default="0"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> </xs:element> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Whitespace-separated list of strings --> <xs:simpleType name="StringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <!-- Whitespace-separated list of unsigned integers --> <xs:simpleType name="UIntVectorType"> <xs:list itemType="xs:unsignedInt"/> </xs:simpleType> <!-- Base URL --> <xs:complexType name="BaseURLType"> <xs:simpleContent> <xs:extension base="xs:anyURI"> <xs:attribute name="serviceLocation" type="xs:string"/> <xs:attribute name="byteRange" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:extension> </xs:simpleContent> </xs:complexType> <!-- Program Information --> <xs:complexType name="ProgramInformationType"> <xs:sequence> <xs:element name="Title" type="xs:string" minOccurs="0"/> <xs:element name="Source" type="xs:string" minOccurs="0"/> <xs:element name="Copyright" type="xs:string" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="lang" type="xs:language"/> <xs:attribute name="moreInformationURL" type="xs:anyURI"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Descriptor --> <xs:complexType name="DescriptorType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="schemeIdUri" type="xs:anyURI" use="required"/> <xs:attribute name="value" type="xs:string"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Metrics --> <xs:complexType name="MetricsType"> <xs:sequence> <xs:element name="Reporting" type="DescriptorType" maxOccurs="unbounded"/> <xs:element name="Range" type="RangeType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="metrics" type="xs:string" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <!-- Metrics Range --> <xs:complexType name="RangeType"> <xs:attribute name="starttime" type="xs:duration"/> <xs:attribute name="duration" type="xs:duration"/> </xs:complexType> </xs:schema>

- (normative)

MIME type registration for MPD

  - Introduction

This Annex provides the formal MIME type registration for the MPD. It is referenced from the registry at http://www.iana.org/.

  - MIME type and subtype

The MIME Type and Subtype are defined as follows:

- MIME media type name:	application

- MIME subtype name:	dash+xml

- Required parameters:	none

- Optional parameters:	The ‘profiles’ parameter as documented in Annex C.3.1 in this part of ISO/IEC 23009

- Encoding considerations:	the same media type encoding considerations specified in section 3.2 of RFC 3023

- Security considerations:	The MPD is a Media Presentation Description and contains references to other resources. It is coded in XML, and there are risks that deliberately malformed XML could cause security issues. In addition, an MPD could be authored that causes receiving clients to access other resources; if widely distributed, this could be used to cause a denial-of-service attack.

- Interoperability considerations: 
The specification defines a platform-independent expression of a presentation, and it is intended that wide interoperability can be achieved.

- Published specification:	ISO/IEC 23009-1: Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats

- Applications which use this media type: 	various

- Additional information:

  - File extension(s):					mpd

  - Intended usage:					common

- Other Information/General Comment:	none

- Person to contact for further information:

  - Name:	Thomas Stockhammer

  - Email:	stockhammer@nomor.de

- Author/Change controller:	ISO/IEC JTC1/SC29 (MPEG)

  - Parameters

    - The profiles parameter

Parameter name:	profiles

Parameter value:	The 'profiles' parameter is an optional parameter that indicates one or more profiles to which the file claims conformance. The contents of this attribute shall conform to either the pro-simp or pro-fancy productions of RFC6381, Section 4.5. The profile identifiers reported in the MIME type parameter should match identically the profiles reported in the profiles attribute in the MPD itself (see Clause 8).

example:

application/dash+xml;profiles="urn:mpeg:dash:profiles:mythical,urn:3gpp:dash:profiles:fictional"

- (normative)
DASH Metrics

  - Introduction

This Annex defines the ISO/IEC 23009-1 DASH Metrics. The normative aspects of the Annex are defined in D.4, namely the semantics of the metrics and the associated keys to be used for requesting the collection of the metrics. The client reference model in D.2 and the observation points in D.3 serve as informative background information.

  - DASH-Metrics client reference model

The DASH-Metrics client reference model is depicted in highlighting so-called observation points (OPs) as defined in D.3.

Figure D.1 — DASH-Metrics client reference model

The DASH access client as defined in 4.2 in this part of ISO/IEC 23001, issues HTTP requests (for DASH data structures), and receives HTTP request responses (containing DASH data structures). Data structures may typically be MPDs, Segments or partial Segments. This input/output interface from the network towards the DASH client is referred to as observation point 1 (OP1).

Furthermore, the DASH client delivers encoded media samples to the DASH-enabled application for further processing and may receive also commands from it. This input/output interface of the DASH client towards the DASH-enabled application is referred to as observation point 2 (OP2).

NOTE	Further processing may include de-multiplexing (of audio/video) and/or decoding potentially involving several buffers.

Finally, the DASH-enabled application delivers decoded media samples to the media output, which displays the media to the user. This output interface towards the user is referred to as observation point 3 (OP3).

  - Definition of observation points

    - Introduction

This Clause defines the observation points as depicted in Figure D.1.

    - Observation point 1

The observation point 1 (OP1) is defined as:

- a set of TCP connections each defined by its destination IP address, initiation, connect and close times;

- a sequence of transmitted HTTP requests, each defined by its transmission time, contents, and the TCP connection on which it is sent; and

- for each HTTP response, the reception time and contents of the response header and the reception time of each byte of the response body.

- NOTE	The contents of the response body is fully defined by the contents of the request and response headers.

    - Observation point 2

The observation point 2 (OP2) consists of encoded media samples. Each encoded media sample is defined as:

- media type;

- decoding time;

- presentation time;

- the @id of the Representation from which the sample is taken; and

- the delivery time.

    - Observation point 3

The observation point 3 (OP3) consists of decoded media samples. Each decoded media sample is defined as:

- the media type;

- the presentation timestamp of the sample (media time);

- the actual presentation time of the sample (real time); and

- the @id of the Representation from which the sample is taken (the highest dependency level if the sample was constructed from multiple Representations).

  - Semantics of the DASH metrics

    - Introduction

This Subclause provides the general QoE metric definitions and measurement framework.

The semantics are defined using an abstract syntax. Items in this abstract syntax have one of the following primitive types (Integer, Real, Boolean, Enum, String) or one of the following compound types:

- Objects: an unordered sequence of (key, value) pairs, where the key always has string type and is unique within the sequence.

- List: an ordered list of items.

- Set: an unordered set of items.

Additionally, there are two kinds of timestamp defined, i.e., real time (wall-clock time) with type Real-Time and media time with type Media-Time.

Where lists are defined the name ‘entry’ is used to define the format of each entry, but since lists contain unnamed entries this name would not appear in any concrete syntax.

Each metric is defined as a named list of entries that logically contains the metric information for the entire session. Reporting of these lists, whether done at the end of the session or incrementally during the session is out of scope of this specification.

    - TCP connections

Table D.1 contains the metric defining the list of TCP connections. The key in Table D.1 shall be used to refer to the metric as defined in Table D.1.

Table D.1 — List of TCP connections

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| TcpList |  |  | List | List of HTTP request/response transactions |
|  | - Entry |  | - Object | An entry for a single HTTP request/response |
|  |  | - tcpid | - Integer | Identifier of the TCP connection on which the HTTP request was sent. |
|  |  | - dest | - String | IP Address of the interface over which the client is receiving the TCP data. |
|  |  | - topen | - Real-Time | The time at which the connection was opened (sending time of the initial SYN or connect socket operation). |
|  |  | - tclose | - Real-Time | The time at which the connection was closed (sending or reception time of FIN or RST or close socket operation). |
|  |  | - tconnect | - Integer | Connect time in ms (time from sending the initial SYN to receiving the ACK or completion of the connect socket operation). |

    - HTTP request/response transactions

Table D.2 contains the metric defining the List of HTTP Request/Response Transactions. The key in Table D.2 shall be used to refer to the metric as defined in Table D.2.

Table D.2 — List of HTTP request/response transactions

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| HttpList |  |  |  |  | List | List of HTTP request/response transactions |
|  | Entry |  |  |  | Object | An entry for a single HTTP request/response |
|  |  | tcpid |  |  | Integer | Identifier of the TCP connection on which the HTTP request was sent. |
|  |  | type |  |  | Enum | This is an optional parameter and should not be included in HTTP request/response transactions for progressive download. The type of the request: - MPD - XLink expansion - Initialisation Segment - Index Segment - Media Segment - Bitstream Switching Segment - other |
|  |  | url |  |  | String | The original URL (before any redirects or failures) |
|  |  | actualurl |  |  | String | The actual URL requested, if different from above |
|  |  | range |  |  | String | The contents of the byte-range-spec part of the HTTP Range header. |
|  |  | trequest |  |  | Real-Time | The real time at which the request was sent. |
|  |  | tresponse |  |  | Real-Time | The real time at which the first byte of the response was received. |
|  |  | responsecode |  |  | Integer | The HTTP response code. |
|  |  | interval |  |  | Integer | The duration of the throughput trace intervals (ms), for successful requests only. |
|  |  | trace |  |  | List | Throughput trace, for successful requests only. |
|  |  |  | Entry |  | Object | A single throughput measurement entry. |
|  |  |  |  | s | Real-Time | Measurement period start. |
|  |  |  |  | d | Integer | Measurement period duration (ms). |
|  |  |  |  | b | List | List of integers counting the bytes received in each trace interval within the measurement period. |

NOTE

1) Information additional to that specified in the type may be returned, for example if a client makes a request for a initialisation information from a self-initialising Media Segment then Segment Index may also be received.

2) All entries for a given object will have the same URL and range and so can easily be correlated. If there were redirects or failures there will be one entry for each redirect/failure. The redirect-to URL or alternative url (where multiple have been provided in the MPD) will appear as the actualurl of the next entry with the same url value.

3) The periods in entry should be those periods where the client was actively reading from the TCP connections (i.e., they should not include periods where the TCP connection is idle due to zero receive window).

The end of the last measurement period in the trace shall be the time at which the last byte of the response was received.

The interval and trace shall be absent for redirect and failure records.

The key HttpList(n,type) where n is a positive integer is defined for an HttpList with an interval of n ms and type is one of MPD, XLinkExpansion, InitialisationSegment, MediaSegment, IndexSegment BitstreamSwitchingSegment or other. If type is not present, all HTTP transactions are requested to be collected. If type is present, it specifies that the HTTP transactions concerning a resource equal to type are requested to be collected. Multiple keys HttpList(n,type) with different values of n and type may be present for a single @metrics attribute value.

An HTTP transaction that is not finished within a QoE metric collection period shall not be included in the metrics.

    - Representation switch events

Table D.3 defines the metric for Representation switch events. The key in Table D.3 shall be used to refer to the metric as defined in Table D.3.

Table D.3 — List of Representation switch events

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RepSwitchList |  |  | List | List of Representation switch events (a switch event is the time at which the first HTTP request for a new Representation, that is later presented, is sent) |
|  | Entry |  | Object | A Representation switch event. |
|  |  | t | Real-Time | Time of the switch event. |
|  |  | mt | Media-Time | The media presentation time of the earliest access unit (out of all media content components) played out from the “to” Representation. |
|  |  | to | String | value of Representation@id identifying the switch-to Representation. |
|  |  | lto | Integer | value of SubRepresentation@level within Representation identifying the switch-to level of the Representation. |

    - Buffer level

Table D.4 defines the metric for buffer level status events. The key in Table D.4 shall be used to refer to the metric as defined in Table D.4.

Table D.4 — List of buffer level

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| BufferLevel |  |  | List | List of buffer occupancy level measurements during playout at normal speed. |
|  | Entry |  | Object | One buffer level measurement. |
|  |  | t | Real-Time | Time of the measurement of the buffer level. |
|  |  | level | Integer | Level of the buffer in milliseconds. Indicates the playout duration for which media data of all active media components is available starting from the current playout time. |

The key is BufferLevel(n), where n is a positive integer is defined to refer to the metric in which the buffer level is recorded every n ms.

    - Play list

Decoded samples are generally rendered in presentation time sequence, each at or close to its specified presentation time. A compact Representation of the information flow can thus be constructed from a list of time periods during which samples of a single Representation were continuously rendered, such that each was presented at its specified presentation time to some specific level of accuracy (e.g., +/-10ms).

Such a sequence of periods of continuous delivery is started by a user action that requests playout to begin at a specified media time (this could be a “play”, “seek” or “resume” action) and continues until playout stops either due to a user action, the end of the content, or a permanent failure.

Table D.5 defines the play list event metric. The key in Table D.5 shall be used to refer to the metric as defined in Table D.5.

Table D.5 — Play list

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PlayList |  |  |  |  | List | A list of playback periods. A playback period is the time interval between a user action and whichever occurs soonest of the next user action, the end of playback or a failure that stops playback. |
|  | Entry |  |  |  | Object | A record of a single playback period. |
|  |  | start |  |  | Real-Time | Timestamp of the user action that starts the playback period. |
|  |  | mstart |  |  | Media-Time | The presentation time at which playout was requested by the user action. |
|  |  | starttype |  |  | Enum | Type of user action which triggered playout - New playout request (e.g. initial playout or seeking) - Resume from pause - Other user request (e.g. user-requested quality change) - Start of a metrics collection period (hence earlier entries in the play list not collected) |
|  |  | trace |  |  | List | List of periods of continuous rendering of decoded samples. |
|  |  |  | Entry |  | Objects | Single entry in the list. |
|  |  |  |  | representationid | String | The value of the Representation@id of the Representation from which the samples were taken. |
|  |  |  |  | subreplevel | unsignedInt | If not present, this metrics concerns the Representation as a whole. If present, subreplevel indicates the greatest value of any Subrepresentation@level being rendered. |
|  |  |  |  | start | Real-Time | The time at which the first sample was rendered. |
|  |  |  |  | mstart | Media-Time | The presentation time of the first sample rendered. |
|  |  |  |  | duration | Integer | The duration of the continuously presented samples (which is the same in real time and media time). “Continuously presented” means that the media clock continued to advance at the playout speed throughout the interval. |
|  |  |  |  | playbackspeed | Real | The playback speed relative to normal playback speed (i.e.normal forward playback speed is 1.0). |
|  |  |  |  | stopreason | Enum | The reason why continuous presentation of this Representation was stopped. Either: - Representation switch (not relevant in case of progressive download) - rebuffering - user request - end of Period - end of content - end of a metrics collection period - failure |

NOTE	The trace may include entries for different representations that overlap in time, because multiple representations are being rendered simultaneously, for example one audio and one video Representation.

- (normative)

Byte range requests with regular HTTP GET methods

  - Background

As a fact there exist deployment environments where the DASH application cannot issue HTTP partial GET, or even if they request it, they do not obtain the requested resources, but possibly the full resource. It is expected that these problems gradually disappear, but until this will be the case, a method is provided to not exclude DASH clients operating in this environments and service providers wanting to support such clients, are excluded from using this DASH specifications to deploy media streaming services using the formats defined in this specification. Still it is expected that this Annex will be deprecated in some later versions of this standard.

To address these requirements the BaseURL@byteRange attribute may be present. If present, it provides information that resources offered in the MPD but are requested by a HTTP partial GET (e.g. Segments for which HTTP-URLs contain byte ranges or Subsegments) may also be requested using a regular HTTP GET by mapping the information that is otherwise added in the Range header into the request URI of a regular HTTP GET request. It is expected that DASH clients only use this method if HTTP partial GET requests fail. If DASH clients only have this alternative to request segments or Subsegments, then it is expected that they request single units of segments or Subsegments.

  - Construction rule

The BaseURL@byteRange attribute represents a template that may be used to construct a URL requesting a byte range (a “byte range URL”) from a resource, given the original URL of the resource and the required byte range. The result of issuing a GET request to this byte range URL without including the HTTP Range header should be identical to the result of requesting the original URL with the byte range specified in the HTTP Range header.

The BaseURL@byteRange contains a template string that contains one or more of the identifiers as listed in Table E.1. The string shall contain identifiers $first$ and $last$ as specified in Table E.1.

The byte range URL shall be constructed from the template string by substituting the identifiers specified in the first column Table E.1 with the values specified in the second column of Table E.1. If the $query$ identifier is not present in the template and the query portion of the original URL as defined in RFC3986 is not empty, then the string “?” query shall be appended to the constructed URL.

If the template string contains unrecognised identifiers then the result of the URL construction is unspecified. In this case it is expected that the DASH Client ignores the entire containing ByteRange element and the processing of the MPD continues as if this ByteRange element was not present.

Strings outside identifiers shall only contain characters that permit to form a valid HTTP-URL according to RFC 1738.

Table E.1 — Identifiers for Byte Range Templates

| $<Identifier>$ | Substitution parameter |
| --- | --- |
| $$ | Is an escape sequence, i.e. "$$" is replaced with a single "$" |
| $base$ | The identifier shall be substituted by the scheme “:” hier-part of the original URL as defined in RFC3986. |
| $query$ | The identifier shall be substituted by the query part of the original URL as defined in RFC3986. If the query part of the original URL is empty then inclusion of this identifier in the template shall cause removal of the separator character immediately preceding the $query$ identifier in the template string if that character is not the “?” character, or, otherwise, the separator character immediately following the $query$ identifier if present. |
| $first$ | The identifier shall be substituted by the byte offset of the first byte in a range and shall be identical to the value of 'first-byte-pos' of 'byte-range-spec' in RFC2616, section 14.35.1, if this request would be executed using a partial GET request. |
| $last$ | The identifier is substituted by the byte offset of the last byte in the range; that is, the byte positions specified are inclusive. It shall be identical to the value of 'last-byte-pos' of 'byte-range-spec' in RFC2616, section 14.35.1, if this request would be executed using a partial GET request. |

  - Examples

| Original URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4?token=8787r08f2gf087g28gf926 |
| --- | --- |
| Byte Range | 1876-23456 |
| BaseURL@byteRange | $base$/range/$first$-$last$ |
| Byte range URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4/range/1876-23456?token=8787r08f2gf087g28gf926 |

| Original URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4 |
| --- | --- |
| Byte Range | 1876-23456 |
| BaseURL@byteRange | $base$?$query$&range=$first$-$last$ |
| Byte range URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4?range=1876-23456 |

| Original URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4?token=8787r08f2gf087g28gf926 |
| --- | --- |
| Byte Range | 1876-23456 |
| BaseURL@byteRange | $base$?$query$&range=$first$-$last$ |
| Byte range URL | http://cdn.example.com/movies/134532/audio/en/aac64.mp4?token=8787r08f2gf087g28gf926&range=1876-23456 |

- (informative)

Guidelines for extending DASH with other delivery formats

  - Adding delivery formats to DASH

In order to support use with DASH, a delivery format should have the property that decoding and playback of any portion of the media can be achieved using a subset of the media that is only a constant amount larger than the portion of the media to be played.

For example, a delivery format where the media is stored as a header followed by a sequence of small blocks, with the property that any block can be decoded and played out given only that block and the header has this property. The definition of these blocks and the mapping to the Subsegments in this part of ISO/IEC 23009 is encouraged. A Subsegment may be defined as a contiguous time interval of a Segment and a contiguous byte range of a Segment, for which no overlap in both dimensions with any other Subsegment in the Segment exists.

Additionally, it is desirable that the delivery format supports some kind of “index” which enables the byte range within the Segment corresponding to any given time range to be efficiently discovered. A suitable unit is the indexing of Subsegments. It should be possible to discover the position in the Segment of the index without downloading the whole Segment. The position of the index may also be advertised in the MPD or the index may be provided as a separate Index Segment. The Segment Index (‘sidx’) or Subsegment Index (‘ssix’), both defined in ISO/IEC 14496-12 may serve as a starting point and/or may be directly applied to any other media format.

  - Media Presentation authoring rules

A specification for how to use a media container format with DASH should include:

- Definition of the MIME type for the Representation as a concatenation of Segments, together with MIME parameters to specify the codecs and codec parameters of the contained media (e.g. the codecs parameter of RFC 6381).

- Description of either a self-initialising Media Segment or the combination of an Initialisation Segment and a Media Segment format.

In addition, the specification may further define:

- Index Segments

- Interpretation of a media Stream Access Point (SAP), potentially different types as defined in 4.5.1 in the context of the media container format. (The SAP types are fully defined in Annex K of ISO/IEC 14496-12 and should not be re-defined, but that interpretation of those definitions in media-container-specific language is valuable).

- Container-format-specific semantics for the @bitstreamSwitching, @segmentAlignment and @subsegmentAlignment. These should align with the definitions in this part of ISO/IEC 23009.

Note that Representation attributes present in the MPD may also be repeated in the media itself, e.g. in an Initialisation Segment or a Media Segment. The media content should be provided such that no mismatch between these two values occurs. If it does, the value in the media itself is expected to take precedence over values expressed in the MPD, especially when used in the media decoding process.

- (informative)

MPD Examples and MPD Usage

  - Example MPD for ISO Base media file format On Demand profile

Simple example for a static presentation, with self-initializing Media Segments, multiple languages, subtitles, content protection and multiple base URLs. This MPD document describes content available from two sources (cdn1 and cdn2) that has audio available in English or French at rates of 64kbits and 32kbits and subtitles in German. Six versions of the video are provided at bitrates between 256kbit/s and 2Mbit/s in different spatial resolutions. Content protection is applied.

The Media Presentation complies with the ISO Base media file format On Demand profile as defined in 8.3.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <Period> <!-- English Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en" subsegmentAlignment="true"> <ContentProtection schemeIdUri="urn:uuid:706D6953-656C-5244-4D48-656164657221"/> <Representation id="1" bandwidth="64000"> <BaseURL>7657412348.mp4</BaseURL> </Representation> <Representation id="2" bandwidth="32000"> <BaseURL>3463646346.mp4</BaseURL> </Representation> </AdaptationSet> <!-- French Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40.2" lang="fr" subsegmentAlignment="true"> <ContentProtection schemeIdUri="urn:uuid:706D6953-656C-5244-4D48-656164657221"/> <Role schemeIdUri="urn:mpeg:dash:role" value="dub"/> <Representation id="3" bandwidth="64000"> <BaseURL>3463275477.mp4</BaseURL> </Representation> <Representation id="4" bandwidth="32000"> <BaseURL>5685763463.mp4</BaseURL> </Representation> </AdaptationSet> <!-- Timed text --> <AdaptationSet mimeType="application/ttml+xml" lang="de"> <Role schemeIdUri="urn:mpeg:dash:role" value="subtitle"/> <Representation id="5" bandwidth="256"> <BaseURL>796735657.mp4</BaseURL> </Representation> </AdaptationSet> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4d0228" subsegmentAlignment="true"> <ContentProtection schemeIdUri="urn:uuid:706D6953-656C-5244-4D48-656164657221"/> <Representation id="6" bandwidth="256000" width="320" height="240"> <BaseURL>8563456473.mp4</BaseURL> </Representation> <Representation id="7" bandwidth="512000" width="320" height="240"> <BaseURL>56363634.mp4</BaseURL> </Representation> <Representation id="8" bandwidth="1024000" width="640" height="480"> <BaseURL>562465736.mp4</BaseURL> </Representation> <Representation id="9" bandwidth="1384000" width="640" height="480"> <BaseURL>41325645.mp4</BaseURL> </Representation> <Representation id="A" bandwidth="1536000" width="1280" height="720"> <BaseURL>89045625.mp4</BaseURL> </Representation> <Representation id="B" bandwidth="2048000" width="1280" height="720"> <BaseURL>23536745734.mp4</BaseURL> </Representation> </AdaptationSet> </Period> </MPD>

  - Example for ISO Base media file format Live profile

Simple example for a dynamic presentation, with multiple languages, multiple base URLs, multiple video bitrates, and segments about two seconds in length for low latency from live programming. At the time this MPD was fetched, 432 Segments of the dynamic presentation was available so the wall clock time must have been approximately 2011-12-25T12:44:24 UTC. All the video Segments are aligned and start with a Stream Access Point. All the audio Segments are aligned so language switching can done with the non-language sound (e.g. music) being seamlessly switched.

In this MPD assuming that the first BaseURL element and the video Representation with id "v1" is selected, and template results in http://cdn1.example.com/video/50000/$Time$.mp4v, the segment list starting at number 0 results in

http://cdn1.example.com/video/500000/0.mp4v http://cdn1.example.com/video/500000/180180.mp4v 
http://cdn1.example.com/video/500000/360360.mp4v 
http://cdn1.example.com/video/500000/540540.mp4v 
http://cdn1.example.com/video/500000/720720.mp4v
...

The media presentation conforms to the ISO Base media file format Live profile in 8.4.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="dynamic" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT30M" availabilityStartTime="2011-12-25T12:30:00" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <Period> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <BaseURL>video/</BaseURL> <SegmentTemplate timescale="90000" media="$Bandwidth$/$Time$.mp4v"> <SegmentTimeline> <S t="0" d="180180" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="v0" width="320" height="240" bandwidth="250000"/> <Representation id="v1" width="640" height="480" bandwidth="500000"/> <Representation id="v2" width="960" height="720" bandwidth="1000000"/> </AdaptationSet> <!-- English Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en" segmentAlignment="0"> <SegmentTemplate timescale="48000" media="audio/en/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000" /> </AdaptationSet> <!-- French Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="fr" segmentAlignment="0"> <SegmentTemplate timescale="48000" media="audio/fr/$Time$.mp4a"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000" /> </AdaptationSet> </Period> </MPD>

  - Example for MPEG-2 TS file format Live profile

Simple example for a static presentation, with multiple languages, multiple base URLs, multiple video bitrates, and segments about four seconds in length.

In this MPD assuming that the first BaseURL element and the Representation with id "1400kbps" is selected, and template results in http://cdn1.example.com/SomeMovie_1400kbps_$Number%05$.ts, the segment list starting at number 0 results in

http://cdn1.example.com/SomeMovie_1400kbps_00001.ts
http://cdn1.example.com/SomeMovie_1400kbps_00002.ts
http://cdn1.example.com/SomeMovie_1400kbps_00003.ts
http://cdn1.example.com/SomeMovie_1400kbps_00004.ts
http://cdn1.example.com/SomeMovie_1400kbps_00005.ts
...

The Media Presentation conforms to the profile in 8.7.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT6158S" availabilityStartTime="2011-05-10T06:16:42" minBufferTime="PT1.4S" profiles="urn:mpeg:profile:dash:m2ts-simple" maxSegmentDuration="PT4S"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <Period id="42" duration="PT6158S"> <AdaptationSet mimeType="video/mp2t" codecs="avc1.4D401F,mp4a" frameRate="24000/1001" segmentAlignment="true" subsegmentAlignment="true" bitstreamSwitching="true" startWithSAP="2" subsegmentStartsWithSAP="2"> <ContentComponent contentType="video" id="481"/> <ContentComponent contentType="audio" id="482" lang="en"/> <ContentComponent contentType="audio" id="483" lang="es"/> <BaseURL>SomeMovie_</BaseURL> <SegmentTemplate media="$RepresentationID$_$Number%05$.ts" index="$RepresentationID$.sidx" initialisation="$RepresentationID$-init.ts" bitstreamSwitching="$RepresentationID$-bssw.ts" duration="4" startNumber="1"/> <Representation id="720kbps" bandwidth="792000" width="640" height="368"/> <Representation id="1130kbps" bandwidth="1243000" width="704" height="400"/> <Representation id="1400kbps" bandwidth="1540000" width="960" height="544"/> <Representation id="2100kbps" bandwidth="2310000" width="1120" height="640"/> <Representation id="2700kbps" bandwidth="2970000" width="1280" height="720"/> <Representation id="3400kbps" bandwidth="3740000" width="1280" height="720"/> </AdaptationSet> </Period> </MPD>

  - Example for multiple stereo views

Simple example for a stereo video presentation from three cameras in a line where one stereo view is from the left-hand two cameras and the second is from the right-hand two cameras.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT10.00S" profiles="urn:mpeg:dash:profiles:isoff-main:2011"> <BaseURL>http://www.example.com/</BaseURL> <!-- In this Period there are 3 views: coming from three lined up cameras: C1-C2-C3. C1+C2 and C2+C3 each form a stereo pair but C1+C3 does not. C2 is taken as the base view for MVC while C1 and C3 are enhancement views --> <Period start="PT0.00S" duration="PT2000.00S"> <SegmentList> <Initialisation sourceURL="seg-m-init.mp4"/> </SegmentList> <AdaptationSet mimeType="video/mp4" codecs="avc1.640828"> <Role schemeIdUri="urn:mpeg:dash:stereoid:2011" value="l1 r0"/> <Representation id="C2" bandwidth="128000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C2view-1.mp4"/> <SegmentURL media="seg-m1-C2view-2.mp4"/> <SegmentURL media="seg-m1-C2view-3.mp4"/> </SegmentList> </Representation> </AdaptationSet> <!-- The following Adaptation set contains a Representation functionally identical to the Representation in the previous Adaptation set. Therefore, these both have the same Representation@id. This is done for compatibility to 2D receivers that do not understand the schemeIdURI of the Role Descriptor and may ignore the Adaptation set --> <AdaptationSet mimeType="video/mp4" codecs="avc1.640828"> <Representation id="C2" bandwidth="128000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C2view-1.mp4"/> <SegmentURL media="seg-m1-C2view-2.mp4"/> <SegmentURL media="seg-m1-C2view-3.mp4"/> </SegmentList> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="mvc1.760028"> <Role schemeIdUri="urn:mpeg:dash:stereoid:2011" value="l0"/> <Representation id="C1" dependencyId="C2" bandwidth="192000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C1view-1.mp4"/> <SegmentURL media="seg-m1-C1view-2.mp4"/> <SegmentURL media="seg-m1-C1view-3.mp4"/> </SegmentList> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="mvc1.760028"> <Role schemeIdUri="urn:mpeg:dash:stereoid:2011" value="r1"/> <Representation id="C3" dependencyId="C2" bandwidth="192000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C3view-1.mp4"/> <SegmentURL media="seg-m1-C3view-2.mp4"/> <SegmentURL media="seg-m1-C3view-3.mp4"/> </SegmentList> </Representation> </AdaptationSet> </Period> <!-- In this Period there are only 2 views: C1+C2 form a stereo pair; C2 is the base view for MVC and C1 is the enhancement view --> <Period duration="PT1256.00S"> <SegmentList> <Initialisation sourceURL="seg-m-init-2.mp4"/> </SegmentList> <AdaptationSet mimeType="video/mp4" codecs="avc1.640828"> <Role schemeIdUri="urn:mpeg:dash:stereoid:2011" value="r0"/> <Representation id="C2" bandwidth="128000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C2view-201.mp4"/> <SegmentURL media="seg-m1-C2view-202.mp4"/> </SegmentList> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs=" mvc1.760028"> <Role schemeIdUri="urn:mpeg:dash:stereoid:2011" value="l0"/> <Representation id="C1" dependencyId="C2" bandwidth="192000"> <SegmentList duration="10"> <SegmentURL media="seg-m1-C1view-201.mp4"/> <SegmentURL media="seg-m1-C1view-202.mp4"/> </SegmentList> </Representation> </AdaptationSet> </Period> </MPD>

  - Example for SVC alternative streams

Simple example for a piece of SVC content split into three Representations with each additional bitrate depending on the previous ones.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <!-- In this Period the SVC stream is split into three representations --> <Period> <AdaptationSet subsegmentAlignment="true" minBandwidth="512000" maxBandwidth="1024000" width="640" height="480" frameRate="30" lang="en"> <!-- Independent Representation --> <Representation mimeType="video/mp4" codecs="avc1.4D401E,mp4a.0x40" id="tag5" bandwidth="512000"> <BaseURL>video-512k.mp4</BaseURL> <SegmentBase indexRange="0-4332"/> </Representation> <!-- Representation dependent on above --> <Representation mimeType="video/mp4" codecs="avc2.56401E" id="tag6" dependencyId="tag5" bandwidth="768000"> <BaseURL>video-768k.mp4</BaseURL> <SegmentBase indexRange="0-3752"/> </Representation> <!-- Representation dependent on both above --> <Representation mimeType="video/mp4" codecs="avc2.56401E " id="tag7" dependencyId="tag5 tag6" bandwidth="1024000"> <BaseURL>video-1024k.mp4</BaseURL> <SegmentBase indexRange="0-3752"/> </Representation> </AdaptationSet> </Period> </MPD>

  - Example for trick play support

The below provides a simple example for using Sub-Representations to support layered coding.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <!-- Period with a multiplexed stream with subrepresentations described for use with fast forward --> <Period> <AdaptationSet mimeType="video/mp4" codecs="avc2.4D401E,avc1.4D401E,mp4a.0x40" width="640" height="480" frameRate="30" lang="en"> <ContentComponent id="0" contentType="video"/> <ContentComponent id="1" contentType="audio"/> <Representation id="tag0" bandwidth="512000"> <BaseURL>video-512k.mp4</BaseURL> <SubRepresentation level="0" contentComponent="0" bandwidth="128000" codecs="avc1.4D401E" maxPlayoutRate="4"/> <SubRepresentation level="1" dependencyLevel="0" contentComponent="0" bandwidth="320000" codecs="avc2.4D401E"/> <SubRepresentation level="2" contentComponent="1" bandwidth="64000" codecs="mp4a.0x40"/> <SegmentBase indexRange="7632" /> </Representation> </AdaptationSet> </Period> </MPD>

  - Example for content protected by multiple schemes

In the example below, example.com is a provider of CDN services and also a hosting service for movie service providers MoviesSP. The English audio and the video tracks are encrypted and licensed by MoviesSP. However, the French audio track is encrypted and licensed by a different service provider.

A hypothetical DRM standardization organization has registered a Scheme Type ‘ZZZZ’ with MP4REG and documented how scheme specific licensing information is stored entirely within the content so there is no additional information provided in the ContentProtection element. Since the Scheme Type is registered and the rules for its use are documented, the “urn:mpeg:dash:mp4protection” @schemeIdUri and “ZZZZ” @value is used.

In addition, a second DRM scheme being used comes from a DRM vendor who has published documentation of their system that declares they use the DASH ContentProtection element with a @schemeIdUri attribute value “http://example.net/052011/drm”. (This DRM vendor owned the domain example.net as of May, 2011.) Documentation for this scheme states there must always be two URLs in the ContentProtection element that are placed in elements defined in the http://example.net/052011/drm namespace. The <License> element contains a license token and the <Content> element contains a content token. Regardless of which service provider uses the protection product from this DRM vendor, these rules must always be followed.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xmlns:drm="http://example.net/052011/drm" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT10.00S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn.example.com/movie23453235/</BaseURL> <Period> <!-- Audio protected with a specified license --> <AdaptationSet mimeType="audio/mp4; codec=mp4a.0x40" lang="en"> <ContentProtection schemeIdUri="http://example.net/052011/drm"> <drm:License>http://MoviesSP.example.com/protect?license=kljklsdfiowek</drm:License> <drm:Content>http://MoviesSP.example.com/protect?content=oyfYvpo8yFyvyo8f</drm:Content> </ContentProtection> <Representation id="1" bandwidth="64000"> <BaseURL>audio/en/64.mp4</BaseURL> </Representation> </AdaptationSet> <!-- Audio protected with embedded information defined by 'ZZZZ' --> <AdaptationSet mimeType="audio/mp4; codec=mp4a.0x40" lang="fr"> <ContentProtection schemeIdUri=" urn:mpeg:dash:mp4protection:2011" value="ZZZZ"/> <Representation id="3" bandwidth="64000"> <BaseURL>audio/fr/64.mp4</BaseURL> </Representation> </AdaptationSet> <!-- Timed text in the clear --> <AdaptationSet mimeType="application/ttml+xml" lang="de"> <Representation id="5" bandwidth="256"> <BaseURL>subtitles/de.xml</BaseURL> </Representation> </AdaptationSet> <!-- Video protected with a specified license --> <AdaptationSet mimeType="video/mp4; codec=avc1" subsegmentAlignment="true"> <ContentProtection schemeIdUri="http://example.net/052011/drm"> <drm:License>http://MoviesSP.example.com/protect?license=jfjhwlsdkfiowkl</drm:License> <drm:Content>http://MoviesSP.example.com/protect?content=mslkfjsfiowelkfl</drm:Content> </ContentProtection> <BaseURL>video/</BaseURL> <Representation id="6" bandwidth="256000" width="320" height="240"> <BaseURL>video256.mp4</BaseURL> </Representation> <Representation id="7" bandwidth="512000" width="320" height="240"> <BaseURL>video512.mp4</BaseURL> </Representation> <Representation id="8" bandwidth="1024000" width="640" height="480"> <BaseURL>video1024.mp4</BaseURL> </Representation> </AdaptationSet> </Period> </MPD>

  - Example for usage of Role descriptor

In the following MPD example, “supplementary” audio Representation of which id is “31” or “32” can be presented together with “main” video Representation of which id is “11” or “12” since Viewpoint descriptors are equivalent, i.e. the @schemeIdUri and the @value are equivalent. Similarly, “supplementary” audio Representation of which id is “41” or “42” can be presented together with “alternate” video Representation of which id is “21” or “22”.

NOTE	The MPD is not complete and only provides a description of the concept.

<MPD>

<Period>
        <AdaptationSet mimeType=”video/mp4” group=”1”>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”main”/>
            <Viewpoint schemeIdUri=”urn:mpeg:DASH:viewpoint:2011” value=”vp1”/>
            <Representation id=”11” bandwidth=”1024000”>…</Representation>
            <Representation id=”12” bandwidth=”512000”>…</Representation>
                    …
        </AdaptationSet>
        
        <AdaptationSet mimeType=”video/mp4” group=”1”>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”alternate”/>
            <Viewpoint schemeIdUri=”urn:mpeg:DASH:viewpoint:2011” value=”vp2”/>
            <Representation id=”21” bandwidth=”1024000”>…</Representation>
            <Representation id=”22” bandwidth=”512000”>…</Representation>
                    …
        </AdaptationSet>
        
        <AdaptationSet mimeType=”audio/mp4” group=”2”>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”main”/>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”supplementary”/>
            <Viewpoint schemeIdUri=”urn:mpeg:DASH:viewpoint:2011” value=”vp1”/>
            <Representation id=”31” bandwidth=”128000”>…</Representation>
            <Representation id=”32” bandwidth=”64000”>…</Representation>
                        …
        </AdaptationSet>
        
        <AdaptationSet mimeType=”audio/mp4” group=”2”>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”alternate”/>
            <Role schemeIdUri=”urn:mpeg:DASH:role:2011” value=”supplementary”/>
            <Viewpoint schemeIdUri=”urn:mpeg:DASH:viewpoint:2011” value=”vp2”/>
            <Representation id=”41” bandwidth=”128000”>…</Representation>
            <Representation id=”42” bandwidth=”64000”>…</Representation>
                …
        </AdaptationSet>
    </Period>
    …
</MPD>

  - Example for MPD security

    - XML encryption of MPD confidential elements or attributes

An original MPD with clear-text BaseURL element is provided below.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://www.example.com/</BaseURL> <Period> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en"> <Representation id="1" bandwidth="64000"> <BaseURL>7657412348.mp4</BaseURL> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="avc1.4d0228"> <Representation id="2" bandwidth="512000" width="320" height="240"> <BaseURL>56363634.mp4</BaseURL> </Representation> </AdaptationSet> </Period> </MPD>

A corresponding MPD with an encrypted BaseURL element is provided below.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <!-- Encrypted BaseURL element --> <EncryptedData Type='http://www.w3.org/2001/04/xmlenc#Element' xmlns='http://www.w3.org/2001/04/xmlenc#'> <EncryptionMethod Algorithm='http://www.w3.org/2001/04/xmlenc#aes128-ctr'/> <KeyInfo xmlns='http://www.w3.org/2000/09/xmldsig#'> <KeyName>baseurl</KeyName> </KeyInfo> <CipherData> <CipherValue>jubafOqNEpcwR3RdFsT7bCqnXPBe5ELh5u4VEy19MzxkXRgr=</CipherValue> </CipherData> </EncryptedData> <Period> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en"> <Representation id="1" bandwidth="64000"> <BaseURL>7657412348.mp4</BaseURL> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="avc1.4d0228"> <Representation id="2" bandwidth="512000" width="320" height="240"> <BaseURL>56363634.mp4</BaseURL> </Representation> </AdaptationSet> </Period> </MPD>

    - XML signature on MPD

An MPD without any signature element is provided below.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <Period> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en"> <Representation id="1" bandwidth="64000"> <BaseURL>7657412348.mp4</BaseURL> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="avc1.4d0228"> <Representation id="2" bandwidth="512000" width="320" height="240"> <BaseURL>56363634.mp4</BaseURL> </Representation> </AdaptationSet> </Period> </MPD>

An corresponding MPD with a Signature element is provided below.

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:DASH:schema:MPD:2011" xsi:schemaLocation="urn:mpeg:DASH:schema:MPD:2011" type="static" mediaPresentationDuration="PT3256S" minBufferTime="PT1.2S" profiles="urn:mpeg:dash:profile:isoff-on-demand:2011"> <BaseURL>http://cdn1.example.com/</BaseURL> <BaseURL>http://cdn2.example.com/</BaseURL> <Period> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.0x40" lang="en"> <Representation id="1" bandwidth="64000"> <BaseURL>7657412348.mp4</BaseURL> </Representation> </AdaptationSet> <AdaptationSet mimeType="video/mp4" codecs="avc1.4d0228"> <Representation id="2" bandwidth="512000" width="320" height="240"> <BaseURL>56363634.mp4</BaseURL> </Representation> </AdaptationSet> </Period> <Signature xmlns="http://www.w3.org/2000/09/xmldsig#"> <SignedInfo> <CanonicalizationMethod Algorithm="http://www.w3.org/TR/2001/REC-XML-c14n-20010315"/> <SignatureMethod Algorithm="http://www.w3.org/2000/09/xmldsig#rsa-sha1"/> <Reference URI=" "> <Transforms> <Transform Algorithm="http://www.w3.org/2000/09/xmldsig#enveloped-signature"/> </Transforms> <DigestMethod Algorithm="http://www.w3.org/2000/09/xmldsig#sha1"/> <DigestValue>DiHakF8GUVLTCii</DigestValue> </Reference> </SignedInfo> <SignatureValue>i6watmQQQ1y3GB+VsWq5fJKzQcBB4jRfH1bfJFj0JtFVtLotttzYyA==</SignatureValue> <KeyInfo> <KeyValue> <RSAKeyValue> <Modulus>xA7SEU+e0yQH5rm9kbCDN9o3aPIo7HbP7tX6WOocLZAtNfyxSZDU16ksL6W=</Modulus> <Exponent>EQ==</Exponent> </RSAKeyValue> </KeyValue> </KeyInfo> </Signature> </MPD>

Bibliography

- 3GPP TS 26.234, Transparent end-to-end packet switched streaming service (PSS); Protocols and codecs, http://www.3gpp.org/ftp/Specs/html-info/26234.htm

- 3GPP TS 26.244, Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP), http://www.3gpp.org/ftp/Specs/html-info/26244.htm

- IETF RFC 1952, GZIP file format specification version 4.3, May 1996

- IETF RFC 2231, MIME Parameter Value and Encoded Word Extensions: Character Sets, Languages, and Continuations, November 1997

- IETF RFC 2405, Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies, November 1996

- IETF RFC 2818, HTTP Over TLS, May 2000

- IETF RFC 5905, Network Time Protocol Version 4: Protocol and Algorithms Specification, June 2010

- “XML Encryption Syntax and Processing”, W3C Recommendation 10 December 2002, http://www.w3.org/TR/xmlenc-core/

- “XML Signature Syntax and Processing (Second Edition)”, W3C Recommendation 10 June 2008, http://www.w3.org/TR/xmldsig-core/

- IEEE 1003.1-2008, IEEE Standard for Information Technology - Portable Operating System Interface (POSIX), Base Specifications, Issue 7.