SM- Bus protocol

Bus capable, certain and fast transmission protocol for Microcontroller systems

Protocol description

Stand: 12/96

Contents:

1	SM- bus protocol	3

2	Messages	4

2.1	Data transmission Master to Slave (SELECT):	4

2.2	Data Request Master to Slave (ENQUIRY):	4

2.3	Authentication Master to Slave	4

2.4	Acknowledgement of message reception:	5

2.5	Slaves data request Reply	5

2.6	Slaves Error Reply:	5

2.7	Slaves Reply:  Empty message	5

3	Elements in the SM bus protocol	6

3.1	Control characters:	7

3.2	Addresses	7

3.3	Checksum BCA and BCC:	7

3.4	Code	8

Codeoffset 40h:	8

Codeoffset 60h:	8

3.5	Data area	9

4	Bus participants	10

4.1	Master	10

4.2	Slave	10

5	Transmission securities	11

6	Examples:	12

6.1	Data communication with assumption into the non volatile module memory	12

6.2	Data communication without assumption into the non volatile module memory	13

6.3	Example service request	14

# SM- bus protocol

The engineers of the GHL considered demands following for the serial data transmission in the creation of the bus protocol.

• Transmission security

• use of standardized tax signs

• Bus capacity with a RS485 interface

• Data exchange of every to every device

• Slight data circumference in the relation to the utility data

# Messages

The Master can send the following types of commands to the slaves

- Messages to the device

- Data to the device

- Authentication commands

The Slave will send (after authentication) the following commands to the Master

- Acknowledgement of message reception

- The requested data (from a command)

- An error message

Note:

Slave = Controller Device

Master = Application software.

Messages Master to Slave

## Data transmission Master to Slave (SELECT):

Purpose:

Set a parameter (code) a value (data).

Format:

| SOH | ADRE | ADRS | BCA | STX | Code | Daten | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Data Request Master to Slave (ENQUIRY):

Purpose:

Retrieve the value (data) of a parameter (code).

Format:

| SOH | ADRE | ADRS | BCA | STX | Code | ENQ | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Authentication Master to Slave

Purpose:

Enable the Slave an answer on the data transmission or data request.

Format:

| SOH | ADRE | ADRS | BCA | EOT |
| --- | --- | --- | --- | --- |

Messages Save to Master

## Acknowledgement of message reception:

Purpose:

Confirmation of the data request and reception.

Format:

| SOH | ADRE | ADRS | BCA | STX | ACK | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Reference:

The Slave will answer only after Authentication.

## Slaves data request Reply

Purpose:

Transmit the requested values (data) of the parameter (code) the mast

Format:

| SOH | ADRE | ADRS | BCA | STX | Code | Data | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Slaves Error Reply:

Purpose:

Transmit the mast a defective reception or a negative analysis.

Format:

| SOH | ADRE | ADRS | BCA | STX | NAK | Nr | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Slaves Reply:  Empty message

Purpose:

Answer for the case that the mast gives an Authentication without having sent previously a data transmission or data request.

Format:

| SOH | ADRE | ADRS | BCA | EOT |
| --- | --- | --- | --- | --- |

# Elements in the SM bus protocol

Im SM-Busprotokoll werden folgende Elemente benutzt:

SOH : 	Start of Heading (0x01)

ADRE : 	Address of the Receiver +0x50 (eg. Device 1 = 0x51)

ADRS : 	Address of the Sender +0x50 (eg. Device 1 = 0x51)

BCA : 	Checksum über SOH, ADRE und ADRS (ggf +0x20)

STX : 	Start of Text (0x02)

Code : 	Defines the meaning of the data. 
Format: Half byte way transmission (lower first) +0x40 or +0x60

Note: Code offset 0x40:

Module transfers the parameter value to the main memory (RAM) and stores it non volatile (EEPROM). Code offset 0x60: Module transfers the parameter value only to the main memory (RAM), without transferring the value to the non volatile memory!

Data : 	Data byte:

Format: Half byte way transmission (lower first) +0x30 number: arbitrary, and/or of the receiver limits

ENQ :	Enquiry (0x05)

ETX : 	End of Text (0x03)

BCC : 	Checksum over the entire transmission to incl. ETX (eg. +0x20)

EOT : 	End of Transmission (0x04)

ACK : 	Acknowledge (0x06)

NAK : 	Not Acknowledge (0x15)

Nr : 	Error Code

|  | REFERENCE: The numerical values for code numbers (parameter numbers), used in the examples, and data values (parameter values) are fictitious and on the basis of didactical criteria selected. During communication with our micro controller modules the system software appropriate parameter list is to be consulted. Out these can do the valid parameter numbers (code) and the ranges of values of the parameter values (data) to be taken. |
| --- | --- |

## Control characters:

All control characters are defined and lie in the ASCII code beneath 0x20.  In order to guarantee the clarity, all must receive therefore other transmission signs a corresponding Offset.

## Addresses

The minutes elements ADRE, ADRS and BCA fulfill the basic conditions for a busable transmission protocol. It is ensured that in each case equipment sends on the bus, then a data communication bus can be developed with this minutes. A bus participant takes over then the master function and all other devices the Slave function.

Universal-Address ( 0 )

The universal address (= address 0) is a special address. Modules, which are preparatory for a Universaldresse in their system software, react both to messages with the correct module address ADRE , and on messages with the universal address in the same way. Under use of the universal address so the module address can ADRE  are adjusted.

With the answer messages the module uses the adjusted module address as transmitter address nevertheless.

Broadcast address ( 32 )

Apart from the own module address and the universal address a module can react also to messages with the Broadcast address (= address 32).

In contrast to the universal address the modules do not send an answer message back with Broadcast address used. The Broadcast address makes it possible thus to send message at the same time to all modules at the bus.

## Checksum BCA and BCC:

The checksum is formed from the addition of all previous DATA, whereby the transfer over 8-bits is ignored. If the block check into the range of the control characters falls (BCC and/or BCA < 20h) it is increased by 20h:

BCA = ( S ( SOH..ADRS)) mod 256; 	BCA < 20h ? ® BCA = BCA + 20h

BCC = ( S ( SOH..ETX )) mod 256; 	BCC < 20h ? ® BCC = BCC + 20h

## Code

So that the Slave knows, what the received DATA mean, (or several) code bytes transferred.

So for example a code byte (parameter number) can mean 80h that it concerns with the following data bytes the kp value of an automatic controller. The Slave knows the appropriate function on the basis the code byte would drive out.

That or the code bytes are isolated into half bytes and transferred of the low order half byte beginning in ever a transmission byte with the offset 0x40 or with the offset 0x60.

Thus the data area lies during the transmission between 40h and 4Fh or between 60h and 6Fh.

### Codeoffset 40h:

Wird bei einem SELECT-Befehl der Code mit dem Offset 40h übertragen,

	übernimmt das Modul die Data (den Parameterwert) in den Arbeitsspeicher (RAM)

	und speichert die Data (den Parameterwert) zusätzlich in den nichtflüchtigen Speicher des Moduls (z.B. EEPROM) ab.

Die Data bleiben also auch nach einem Modulreset oder Spannungsabfall erhalten!

Beispiel: 	Code 			Übertragung					 
80h 			0x40, 0x48 
0xFE002710 		0x40, 0x41, 0x47, 0x42, 0x40, 0x40, 0x4E, 0x4F

### Codeoffset 60h:

Wird bei einem SELECT-Befehl der Code mit dem Offset 60h übertragen,

	übernimmt das Modul die Data (den Parameterwert) in den Arbeitsspeicher (RAM)

	die Data (der Parameterwert) werden jedoch nicht in den nichtflüchtigen Speicher des Moduls (z.B. EEPROM) abgespeichert.

Die Data gehen nach einem Modulreset oder Spannungsabfall verloren. Das Modul benutzt die zuvor abgespeicherten Werte!

Beispiel: 	Code 			Übertragung					 
80h 			60h, 68h 
FE002710h 		60h, 61h, 67h, 62h, 60h, 60h, 6Eh, 6Fh

|  | REFERENCE: Codeoffset 60h ist vorgesehen für übergeordnete Leitsysteme, die zyklisch SELECT-Befehle an das Modul senden möchten. Dadurch kann das EEPROM des Moduls, dessen Schreibzyklen begrenzt sind, geschont werden. |
| --- | --- |
|  | REFERENCE: Bei ENQUIRY-Befehlen und AUTHENTICATION kann der Codeoffset sowohl 40h, als auch 60h betragen. |
|  | REFERENCE: In den Antwortnachrichten unserer Module wird ausschließlich der Codeoffset 40h verwendet. |

## Data area

There it in that to transmitting data around ASCII, but rather around binary data does not act itself, are restricted this also not on a value area.  In order to guarantee the clarity of the tax bytes, a transmission byte cannot be covered therefore by a data byte entire.  The data are undoed therefore in half bytes and are transmitted by the niederwertigen half byte beginning in a transmission apiece byte with the Offset 30h.  Therewith the data area is about the transmission between 30h and 3Fh.

Example: 	Data 			Transmission					 
0x1234 		0x34, 0x33, 0x32, 0x31 
0xFE002710 		0x30, 0x31, 0x37, 0x32, 0x30, 0x30, 0x3E, 0x3F

|  | REFERENCE: The length of the data area is limited not by minutes, but only by the devices. |
| --- | --- |

# Bus participants

## Master

The master has the task the bus to administer and may give as the only bus participant an Authentication at a Slave.  It can however also how all send and receive other bus participant, information.  If the Slaves also data to other Slaves and to the master should send, the master must give the single Slaves one after another an Authentication.  The master supervises the data bus and recognizes the end of the Slave transmissions.

The master must also through a 'Timeout' - recognize function the loss of a Slaves.  That is, a Slave sends no answer upon receipt of the Authentication, must wait the master a certain time before it gives the Authentication the next Slave to block around the total bus not.  If the answer of a Slaves will stay away on an Authentication repeatedly, the master can 'cancel' the Slave and give a corresponding error message at the operator.

## Slave

A Slave only may send after it received an Authentication.  If a Slave receives a message certain for it, it must confirm this with a positive or negative receipt or with a data message.  It may send this confirmation however not immediately on the bus, but rather it must discard in an internal envoy buffer.  It discards also data, that a Slave wants to send at another bus participant (master or Slave), first in the internal envoy buffer.  If the Slave receives now the Authentication, it tests the contents of its envoy buffer.  Data in the envoy buffer lie, these are sent.  If the envoy buffer is empty, the Slave that message' sends 'empty.

# Transmission securities

The transmission security is guaranteed through the protocol elements BCA and BCC, as well as the receipts handshakes.  The Checksum BCC alone would not offer the necessary transmission security because between a mistake in the addresses and the remaining data area distinguished become not could.  If a device receives a defective address that agrees with the own address, it would assess to be sure the incorrect transmission on the basis of the Checksums, would send then however a negative receipt to the sender.  The actually addressed device receives also the data and sends simultaneously a receipt back, what leads to a collision on the bus.

Therefore the additional Checksum was introduced over the addresses.  Does not agree in a reception of the Checksum BCA, are not supposed returned the data and also no receipt.  The Checksum agrees over the addresses (BCA), but not over the total data (BCC), sends the addressed device a negative receipt to the sender.

# Examples:

|  | REFERENCE: The numerical values for code numbers (parameter numbers), used in the examples, and data values (parameter values) are fictitious and on the basis of didactical criteria selected. During communication with our micro controller modules the system software appropriate parameter list is to be consulted. Out these can do the valid parameter numbers (code) and the ranges of values of the parameter values (DATA) to be taken. |
| --- | --- |

The master possesses the address 1,
the Slave possesses the address 4.

## Data communication with assumption into the non volatile module memory

The master (control computer) wants to set the kp value with the Slave (here an automatic controller) on 418h.
As code for the function " Kp-Wert" 80h is agreed upon.

1. The master sends to the Slave:

| SOH | ADRE | ADRS | BCA | STX | Code |  | Data |  |  | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x54 | 0x51 | 0xA6 | 0x02 | 0x40 | 0x48 | 0x38 | 0x31 | 0x34 | 0x03 | 0x76 | 0x04 |

2. The master the Slave the Authentication:

| SOH | ADRE | ADRS | BCA | EOT |
| --- | --- | --- | --- | --- |
| 0x01 | 0x54 | 0x51 | 0xA6 | 0x04 |

The Slaves received the message (and the DATA for valid recognized, into which main memories taken over and in the non volatile memory put down).

3. The Slave sends (on permission):

| SOH | ADRE | ADRS | BCA | STX | ACK | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x51 | 0x54 | 0xA6 | 0x02 | 0x06 | 0x03 | 0x57 | 0x04 |

## Data communication without assumption into the non volatile module memory

|  | REFERENCE: This example differs from the previous example „data communication with assumption of the parameter value into the non volatile module memory “only in the use of the alternative code off set 60h. |
| --- | --- |

The master (control computer) wants to set the kp value with the Slave (here an automatic controller) on 418h.
As code for the function " Kp-Wert" 80h is agreed upon.

1. The master sends to the Slave:

| SOH | ADRE | ADRS | BCA | STX | Code |  | Data |  |  | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x54 | 0x 51 | 0x A6 | 0x 02 | 0x 60 | 0x68 | 0x38 | 0x31 | 0x34 | 0x03 | 0xB6 | 0x04 |

2. The master the Slave the Authentication:

| SOH | ADRE | ADRS | BCA | EOT |
| --- | --- | --- | --- | --- |
| 0x01 | 0x54 | 0x51 | 0xA6 | 0x04 |

The Slaves received the message (and the DATA for valid recognized and transferred to the main memory).

3. The Slave sends (on permission):

| SOH | ADRE | ADRS | BCA | STX | ACK | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x01 | 0x51 | 0x54 | 0xA6 | 0x02 | 0x06 | 0x03 | 0x57 | 0x04 |

## Example service request

The master (control computer) wants to query the kp value with the Slave (here an automatic controller).

As code for the function " Kp-Wert" 80h is agreed upon.

The queried kp value is 418h. With the inquiry the same code (here 80h) becomes, as set with the kp value used. Instead of the DATA the master sends however the control byte ENQ.

1. The master sends the service request to the Slave:

| SOH | ADRE | ADRS | BCA | STX | Code |  | ENQ | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 54 | 51 | A6 | 02 | 40 | 48 | 05 | 03 | DE | 04 |

2. The master gives the Authentication to the Slave :

| SOH | ADRE | ADRS | BCA | EOT |
| --- | --- | --- | --- | --- |
| 01 | 54 | 51 | A6 | 04 |

The Slaves received and sends the message correctly the requested kp value to the master. So that the master recognizes that it concerns with the transferred DATA (inquired) the kp value, the Slave sends the code byte of the function as code byte " Kp-Wert" (80h).

3. The Slave answers:

| SOH | ADRE | ADRS | BCA | STX | Code |  | Data |  |  | ETX | BCC | EOT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 51 | 54 | A6 | 02 | 40 | 48 | 38 | 31 | 34 | 03 | 76 | 04 |

Schwarzmüller GmbH
Sauerwiesen 6
D 67661 Kaiserslautern
Tel: 	06301 / 7123-0
Fax:	06301 / 7123-99