Little Endian vs. Big Endian

Item definition:

Item name

Exact item content (bytes separated by ¦)

Short name

Long name

Size (with size unit)

description

Special:

«refers to another definition»

«refers to value stored in another block»

X× - count (can be ?, number or range (÷))

0AB0C0D

Big endian: 	0D|0C|0B|0A	(Motorola)

Little endian: 	0A|0B|0C|0D	(Intel)

# JPEG

| JPEG file (JFIF) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FF¦D8 | «segments» ?× Last segment before image stream is always SOS |  |  |  | FF¦D9 |
| SOI |  |  |  |  | EOI |
| Start of image |  |  | Image stream |  | End of image |
| 2B |  |  | Rest of file excl. last 2B |  | 2B |
|  | ?× |  |  |  |  |
| JPEG segment (chunk, marker) |  |  |  |  |  |
| Segment marker |  | Segment size |  | Segment data |  |
| 2B |  | 2B |  | «segment size»-2B |  |
|  |  | Size of segment in bytes excluding marker, including size definition |  |  |  |

Common JPEG segments markers:

FF|E0	APP0 Application 0 marker present in every JPEG file (theoretically)

FF|E1 ÷ FF|EF	APP1 ÷ APP15 Application markers used for various purposes

FF|DB	DQT Quantization table

FF|C0	SOF0 Start of frame

FF|C4	DHT Define Huffman Table

FF|DA	SOS Start of scan

Source: http://www.codeproject.com/bitmap/iptc.asp?print=true

Usage of APP1 ÷ APP15 markers

APP1	Exif

APP2	Exif FlashPix extension

APP13	PhotoShop segment (contains IPTC/NAA)

| Content of Exif block |  |  |
| --- | --- | --- |
| E45¦x78¦i69¦f66¦00¦00 |  |  |
| Exif header | TIFF header | Data |
| 6B | 8B | («segment size»-2-6-8)B |
|  | 1× |  |
| TIFF header |  |  |
| I49¦I49 or M4D¦M4D | 00¦2A or 2A¦00 |  |
| BOM |  |  |
| Byte order marker | BOM test | IFD0 offset |
| 2B | 2B | 4B |
| Defines byte align for number in whole Exif segment MM stands for Motorola (BE) and II for Intel (LE) Intel is more common | Number 0x002A in byte order specified in BOM | Offset of first IFD All offsets in Exif are measured from start of TIFF header, so if IFD0 starts immediately (as usual) its offset is 0x00000008. Keep BOM in mind! |

?×

| Image File Directory (IFD) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of directory entries |  | Directory entries |  | Offset to next IFD |  |
| 2B |  | 12B*«Number of directory entries» |  | 4B |  |
| Number of entries in this IFD |  |  |  | 0x00000000 means that this was last IFD |  |
|  |  | «Number of directory entries»× |  |  |  |
| Directory entry |  |  |  |  |  |
| Tag |  |  |  |  |  |
| Tag number | Kind of data |  | Number of components |  | Data or offset ? |
| 2B | 2B |  | 4B |  | 4B |
|  |  |  |  |  | Data are stored here in case «bytes/component»*«number of components» fits into 4B, otherwise pointer to data is stored here |

Data formats

| Value | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format | UByte | ASCII | UShort | ULong | URatio | SByte | N/A | SShort | SLong | SRatio | Single | Double |
| Bytes/component | 1 | 1 | 2 | 4 | 8 | 1 | 1 | 2 | 4 | 8 | 4 | 8 |

Rational = 2 signed/unsigned 4Bytes integers where 1st is numerator and 2nd is denominator

IFDs

IFD0	IFD of main image

IFD1	IFD of thumbnail image

Exif IFD	Pointed from IFD0 by tag Exif Offset (0x8769), contains IFD data

GPS IFD	Pointed from IFD0 by tag GPS Info IFD Pointer (0x8825), contains IFD data

Interoperability IFD	Pointed from Exif IFD by tag Interoperability IFD Pointer (0xA005), contains IFD data

Thumbnails

JPEG Thumbnail	in case Compression (0x0103) is 6. Thumbnail SOI lies at offset JPEGInterchangeFormat (0x0201). Image data are JPEGInterchangeFormatLength (0x0202) bytes long.

TIFF Thumbnail	in case Compression (0x0103) is 1 (uncompressed). Data are lied at offsets StripOffsets (0x0111) and has lengths StripByteCounts (0x0117). It can be either RGB or YCbCr depending on PhotometricInterpretation (0x0106) - 2 for RGB, 6 for YCbCr.

EXIF Tags

See http://www.media.mit.edu/pia/Research/deepview/exif.html, www.exif.org

| Content of PhotoShop block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P50¦h68¦06F¦t74¦06F¦s73¦h68¦o6F¦p70¦¦333¦﹒2E¦030¦00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Photoshop identification string |  |  |  |  |  |  |  | 8BIM segments |  |  |  |  |  |  |
| 14B |  |  |  |  |  |  |  | («Segment size»-2-14)B |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ?× |  |  |  |  |  |  |
| 8BIM segment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 838¦B42¦I49¦M4D | 0404 (for IPTC) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Segment marker | Segment type |  | Name |  |  | Segment size |  |  | Data (IPTC records) |  |  |  |  |  |
| 4B | 2B |  | «Size»B padded to even length |  |  | 4B |  |  | «Segment size»B, padded to event length |  |  |  |  |  |
|  | Defines type of segment data |  | Pascal string. Followed with 00 if needed to pad to even length (measured with size specifier) |  |  | Excluding marker, type, padding and size |  |  | Followed with 00 if needed to pad to even length. |  |  |  |  |  |
|  |  |  | 1× |  |  |  |  |  | ?× |  |  |  |  |  |
| Pascal string |  |  |  |  |  | IPTC record |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1C |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Tag |  |  |  |  |
| Size |  | String |  |  |  | Tag marker | Record number |  |  | Data set number | Size | Data |  |  |
| 1B |  | («Size»-1)B |  |  |  | 1B | 1B |  |  | 1B |  | «Size»B |  |  |
| Length of string including this byte |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1× |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Size specifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Size specifier |  |  |  |  | Size specifier 2 |  |  |  |  |  |  |  |  |  |
| 2B |  |  |  |  | «Size specifier»B |  |  |  |  |  |  |  |  |  |
| if data are larger then 32768B specifies the length of Size specifier (MSB=1) 2 otherwise length of data (MSB=0, usual) |  |  |  |  | Present only when data are greater than 32768B |  |  |  |  |  |  |  |  |  |

www.iptc.org, http://www.codeproject.com/bitmap/iptc.asp, http://www.ap.org/apserver/userguide/codes.htm, http://www.sno.phy.queensu.ca/~phil/exiftool/TagNames/IPTC.html